# Connecting Brailliant BI X and Mantis Q40 to Mac via USB and Bluetooth

Brailliant BI X and Mantis Q40 braille displays can connect to multiple host devices including a MAC. Below are the methods of connecting these displays to the MAC through the accessibility of the onboard Voiceover.

## Connect Via USB

The below procedure is dependent on your settings set for your USB on the Brailliant. The following procedure assumes you are using the default values set within both the Brailliant BIX and Mantis Q40. For more information on the behavior of USB connection, please refer to the user guide.

Please Note: Voiceover must be running for the braille display to be automatically detected and connect.

To connect your Brailliant BI 20/40 X, Mantis, and Chameleon to your Mac via USB, do the following:

- From the display’s Main Menu, choose Terminal>connected devices>USB connection

From your Mac:

- Please use a USB-C to USB-A cable, (if your Mac has a USB-A port). You may also use a USB-C to USB-C cable if your Mac only has a USB-C port. Simply plug one end into the braille display’s USB-C port, and the other into the corresponding port on your Mac to connect.

## Connect Via Bluetooth

From the braille display’s Main Menu:

- Navigate to the terminal item.

- Select add bluetooth device, (or hold down enter on Mantis, dot 8 on the other braille devices for 5 seconds to put the display in Bluetooth discovery mode)

From your Mac:

- In Finder, navigate to Main Menu>Go>Utilities or press Shift+Command+U  and Choose VoiceOver Utility.

- In VoiceOver Utility, choose the Braille category or press Command+9. choose the Displays tab, Choose Add to add a Bluetooth Braille display, a table of visible Bluetooth devices will appear. Choose the Brailliant BI/Mantis display and you will be connected.

Once connected, press the Home key, (circular button on the front of your display), and choose terminal>connected devices and find the name of your Mac.

Press enter to choose this device, and then press a key on the braille display, and braille from the chosen Mac will be shown.