# VICTOR READER STREAM

USER GUIDE

by HumanWare

V1.6

January 30, 2026

Rev2 FINAL

- About VICTOR READER STREAM

HumanWare is proud to introduce the third version of VICTOR READER STREAM, the powerful digital talking book player.

VICTOR READER STREAM is designed as an advanced high-performance DAISY, MP3 and NISO player for students, professionals and on-the-go persons who have extensive reading needs. It allows you to transfer content from your PC to its internal memory or to an external SD card and to receive content through Wi-Fi. It provides advanced navigation of multiple formats including DAISY, MP3, NISO and electronic text. The VICTOR READER STREAM will be referred to as the Stream throughout this user guide.

Copyright 2026. All rights reserved, Technologies HumanWare inc.

This User Guide is protected by copyright belonging to HumanWare, with all rights reserved. The User Guide may not be copied in whole or in part, without written consent from HumanWare.

# Table of Contents

About VICTOR READER STREAM	3

Table of Contents	4

1	Overview of VICTOR READER STREAM	8

1.1	Unpacking the Player	8

1.2	Physical Description of VICTOR READER STREAM	8

1.2.1	Front Face of the Player	8

1.2.2	Left Side of the Player	8

1.2.3	Right Side of the Player	8

1.2.4	Top Edge of the Player	8

1.2.5	Bottom Edge of the Player	9

1.2.6	Back side of the Player	9

1.2.7	Recharging the Battery	9

1.3	Power Button	10

1.3.1	Turning the Player On and Off	10

1.3.2	Suspend mode	10

1.3.3	Player Reset Using Power Button	10

1.4	Inserting or Removing an SD Card	10

1.4.1	SD Card Voice Label	11

1.5	SD Card Detection	11

1.6	Bookshelf Structure	11

1.7	Other Reserved File Names	13

1.8	Transferring Files Between Your Computer and Stream	13

1.9	HumanWare Companion	13

2	Basic Functions	14

2.1	Changing Volume, Speed and Tone/Pitch	14

2.1.1	Different Speed settings for TTS and Audio playback	14

2.2	Changing Bass and Treble (Music bookshelf)	14

2.3	Play/Stop	14

2.4	Rewind and Fast Forward	15

2.5	Sleep Mode and Time announcement	15

2.6	Setting the Date and Time	15

2.7	Key Describer Mode	16

3	Numeric Key Functions	17

3.1	Numeric Keypad List	17

3.2	Navigation Keys	17

3.2.1	Undo navigation	17

3.3	Time Jump Navigation Mode	17

3.4	Text-to-Speech Navigation Elements	18

3.5	Spell Mode for Text Content	18

3.6	Bookshelf Navigation — Key 1	18

3.6.1	Online Bookshelf Navigation	18

3.6.2	Multi-Level Bookshelf Navigation	18

3.7	User Guide — Key 1 (Press and hold)	19

3.8	Manage Books — Key 3	19

3.9	Download manager (key 3 press and hold)	19

3.10	Where am I? — Key 5	20

3.10.1	Where am I tag information	20

3.10.2	Where am I for online books	20

3.11	Multi-Voice Text to Speech (TTS)	21

3.12	Audio, Text, and Random Music Playback Modes — Key 9	21

3.13	Confirm, Lock and Cancel — Pound and Star keys	21

3.14	Info — Key 0	21

3.14.1	Available Information Items	22

4	Direct Navigation (Go To Key)	23

4.1	Go to Page	23

4.2	Go to Heading	23

4.3	Go to Time	23

4.4	Go To Percent	23

4.5	Go to Beginning and End of Book	23

4.6	Go to Book	23

4.7	Skip back or forward 10 items	24

4.8	Functions for Online Services	24

5	Advanced Functions	25

5.1	Text Search	25

5.1.1	Search Next or Previous	26

5.1.2	Other types of text search	27

5.2	Recording Audio Notes	27

5.3	Bookmarks	28

5.3.1	Go to Bookmark	28

5.3.2	Navigating by bookmarks	28

5.3.3	Insert Bookmark	28

5.3.4	Insert audio Bookmark	29

5.3.5	Highlight Bookmark	29

5.3.6	Remove Bookmark	30

5.3.7	Bookmark alert	30

6	Configuration Menu – Key 7	31

6.1	General Settings	31

6.1.1	Language	31

6.1.2	System	32

6.1.3	Power off options	33

6.1.4	Idle time	33

6.1.5	App toggle menu	33

6.1.6	Bookshelf navigation mode	33

6.2	Navigation and Playback	34

6.2.1	Time Jump	34

6.2.2	Save Last Used Navigation Level for Each Book	34

6.2.3	Bookmark alert	34

6.2.4	Audio adjustment mode	34

6.2.5	Loop	34

6.2.6	Music	34

6.2.7	End of book	34

6.2.8	Book time details	34

6.3	Wireless	34

6.3.1	Airplane Mode	35

6.3.2	Wi-Fi	35

6.3.3	Bluetooth	36

6.4	Recording	37

6.4.1	Recording Volume Adjustment	38

6.4.2	Preferred Recording Source	38

6.4.3	Recording and audio bookmarks default save location	38

6.4.4	Built-in Microphone File Type	38

6.4.5	Headset Recording File Type	38

6.4.6	External Recording Source	38

6.4.7	External Recording Mode	38

6.4.8	External Recording File Type	39

6.5	About	39

6.6	Online settings	39

6.6.1	Software Update	39

6.6.2	Notification mode	40

6.6.3	Book Services	40

6.6.4	Other Services	42

7	Bookshelf Structure and Characteristics	44

7.1	Other Books	44

7.1.1	Other Books Structure	44

7.1.2	Other Books Characteristics	44

7.2	Audible books	45

7.2.1	Activating your Stream with Audible	45

7.2.2	Book transfer	45

7.2.3	Audible Structure	45

7.2.4	Audible Characteristics	45

7.3	Music Bookshelf	46

7.3.1	Music Structure	46

7.3.2	Music Characteristics	46

7.3.3	Music Search	47

7.3.4	Playlists	47

7.3.5	Custom folder Playlist	47

7.3.6	Folder and File Name Announcement	48

7.4	Saved Podcasts	48

7.4.1	Saved Podcast Structure	48

7.4.2	Saved Podcasts Characteristics	48

7.5	Text Bookshelf	49

7.5.1	Text File Structure	49

7.5.2	Text File Characteristics	49

7.5.3	Navigating HTML/XML/DOCX Headings	49

7.6	Notes	49

7.6.1	Note File Structure	50

7.6.2	Note File Characteristics	50

8	Daisy Online	52

8.1	Daisy Online configuration menu	52

8.2	Using Daisy Online Service	52

8.3	Download Methods	53

8.4	Navigating in your DAISY online bookshelves	53

9	Other Stream Wireless Features	55

9.1	Online Check for Updates	55

9.2	Online Services	55

9.2.1	NFB Newsline (Eligible United States citizens and residents only)	55

9.2.2	NLS BARD (Eligible United States citizens and residents only)	56

9.2.3	Bookshare (eligible users only)	57

9.2.4	Internet Radio	58

9.2.5	References (Wikipedia and Wiktionary)	61

9.2.6	Podcasts	61

9.2.7	TuneIn Radio	63

9.3	NLS Online Authorization	65

10	Updating Stream Software	67

11	Technical Specifications	68

12	Safety Precautions	69

12.1	Contraindications	69

12.2	Warnings	69

12.3	Care and maintenance	69

12.4	Storage and transportation	69

12.5	Additional information	69

12.6	Measured sound Power Level	69

12.7	Service information	69

12.8	Susceptibility to Interference	70

12.9	Audio Warning	70

12.10	Battery Safety Precautions	71

12.11	Disposal Instructions	71

13	Technologies HumanWare Contact Information	72

14	End User License Agreement	73

Appendix 1 - Manufacturer Warranty	74

# Overview of VICTOR READER STREAM

## Unpacking the Player

This package contains the following items:

- VICTOR READER STREAM

- Power supply with four international adapters

- USB-C to USB cable to connect to the PC or power adapter

- Getting started guide (hardcopy)

## Physical Description of VICTOR READER STREAM

### Front Face of the Player

The face of the player can be divided into upper and lower sections:

- In the upper section, there are 5 rows, each row contains 3 keys. The top left, square-shaped key is the Go To Page key that allows you to directly access a desired page or heading number. Above the Go To Page key is a very small hole, this is the built-in mono microphone. To the right of the Go To Page key is the circular shaped Online button for turning Airplane Mode (Wi-Fi) on or off and switching between the player’s offline and online bookcases. To the right of the Online button is the diamond-shaped Bookmark key for selecting and returning to marked passages. The Bookmark key also allows you to toggle text input mode when typing. Rows 2 through 5 comprise a 12 key telephone-style numeric keypad with two raised dots on the number 5 key. There is a raised edge on the 2, 4, 6 and 8 keys. The numeric keypad is used to move through the structure of a book as well as entering bookmark, page or heading numbers. It is also used to enter text in text fields.

- Below the number pad are 4 keys. The Play/Stop key is located at the bottom of the player between the Rewind and Fast Forward keys. Both the Rewind and Fast-Forward keys can be identified via their raised edge. Above the Play/Stop key is the Sleep key. Press the Sleep key multiple times to activate different Sleep timers after which the player will automatically shut off. Press and hold the Sleep key to announce the time and date.

### Left Side of the Player

On the left side of the player, near the top corner, is the Power/Toggle button. Press and hold this button to power the player on and off. Below the Power button is an LED indicator. This LED glows steady when the Stream is powered on and blinks when the Stream is off and recharging the battery. When the player is on, press the same Power button to toggle the volume, speed and tone/pitch settings. Below the LED is the Up/Down button used to increase or decrease volume, speed or tone/pitch selected with the toggle button. Volume settings are saved between sessions.

At the bottom of the left side of the player is the stereo microphone jack, which can be used to connect an external microphone or line-in.

### Right Side of the Player

On the top, right side of the player is the Record button identified by a raised red circle.

### Top Edge of the Player

The top surface consists of the SD card slot, which is protected by a small silicone cover.

### Bottom Edge of the Player

The USB-C port is located at the bottom, center of the player. Use this port to recharge the device, connect to a computer and access the internal memory and/or the SD card using Media Transfer Protocol. The battery will also recharge while connected to the computer. Depending on the amount of power supplied by the computer, charging from the computer USB port may be slower or the player may not charge at all. The most effective way to recharge is to use the USB cable connected to the (included) power adapter to recharge the player from the power mains. Note: If you wish, you may safely use any other commercial USB charger without damaging your Stream. However, other chargers may take longer to charge the Stream. HumanWare recommends a 1.5-amp USB charger.

To the left of the USB-C port is the headphone jack, which can also be used to connect to external speakers.

To the right of the USB-C port is the speaker (four small holes). The speaker is deactivated when headphones are plugged in.

### Back side of the Player

The player's model and serial number can be found on the back of the device. This same information can also be obtained by pressing INFO (key 0) or by pressing and holding key 5. Around the label is a rubber outline.

### Recharging the Battery

The battery will automatically recharge whenever the player is plugged to AC power or a computer. When the player is turned Off and plugged in, the power LED indicator will blink if the battery is recharging. The player can also be used while recharging but in this case the LED will glow steadily instead of blinking. To obtain battery and recharge status, press and hold the Confirm key. You can also obtain the battery and recharge status when you press the INFO key (key 0). A full recharge takes up to 3 hours on AC power. Computers supply different levels of power or perhaps no power at all on their USB ports depending on the computer model and its configuration. Therefore, when recharging from a computer USB port, the Stream will likely take longer to recharge, or possibly not recharge at all, depending on the USB power supplied by the computer.

You may safely use a USB-C AC adapter other than the supplied power adapter. For optimal recharge, HumanWare recommends a 1.5-amp USB power adapter and a quality USB Cable. Adapters with less power will likely require more than 3 hours to fully recharge the battery.

When the player is operating on battery and left in pause mode for more than 30 minutes, it will shut Off automatically to conserve battery life.

Important: It is normal for the battery to become warm during recharging. Therefore, it is recommended not to charge the battery with the player near a heat source or within any kind of enclosure.

The fully charged battery will provide up to 15 hours of playtime on headphones. However, battery playtime may be reduced in the following cases:

- If the battery is charged for less than 3 hours.

- When the player is used in online mode.

- Extensive use of book navigation commands.

- High volume level or High-Speed playback.

- After about 400 recharge cycles, the ability for the battery to hold a charge will start to diminish.

## Power Button

### Turning the Player On and Off

To turn the player On, press and hold the Power key for a few seconds, which is located on the top, left side of the player. You will hear a beep and a welcome message.

To turn the player Off, press and hold the Power key again. The device will announce “Shutting down, please wait”, followed by two beeps, indicating the player has completed powering Off.

Note: when the device is configured to power off while the sleep mode is configured (see section 2.5 "Sleep mode and time announcement), if the battery is too low, the device will shutdown before attaining the configured period of time.

### Suspend mode

In the Configuration menu, under the General settings submenu, you will find a submenu called “Power off options”, containing a toggle between “Power off” and “Suspend”. By default, the option “Power off” is selected, and your device is powering off when pressing and holding the Power button, as described in the subsection above. If you toggle this option to “Suspend”, when you will press and hold the Power button, it will put the device in Suspend mode, that is working the same way as a Sleep mode. To toggle between the two options, use the pound key. The Suspend mode can be useful if you want to save time when using regularly your device throughout the day. To put your device in Suspend, press and hold the Power button. The two beeps sound will be emitted. To awake your device, press and hold again the Power button, the device will emit a beep sound and the device will return to the last position you were before putting the device in Suspend. To shutdown completely the device when in Suspend mode, go to the Configuration menu, then select the option “Shutdown now?”. Press on the Pound key on this option, then press again on the Pound key on the confirmation prompt to shutdown your device.

Note: when in Suspend mode, you should shutdown your device when you know that you will not use the device for a long period of time, as the Suspend mode will consume more energy than if the device had been powered off. If your device is in Suspend, the device will shutdown when the battery will be too low. Finally, please note that in suspend mode, when using one of the sleep time configurations, the device will be put in suspend when the sleep timer attains 0.

### Player Reset Using Power Button

If the Stream does not respond to any key presses including power off, hold down the Power button for 7 seconds to force a player reset.

## Inserting or Removing an SD Card

Follow these steps to insert and remove an SD card:

- Hold the player facing you. The SD card slot is on the top of the player.

- Pull back the rubber cover that covers the slot.

- One corner of the SD card is cut at a 45-degree angle. This cut corner should be at the bottom, left when you insert the card into the player slot. If the card is not oriented properly, it will resist insertion. Note that some SD cards have a small write-lock tab on the right edge.

- Gently push the card down into the slot until it clicks into place. Do not use excessive force. Only slight pressure is required to lock the SD card into place.

- Put the rubber cover back in place to cover the slot.

- To remove the card, push down on the top edge to release it. Then pull it out.

### SD Card Voice Label

Attaching print or Braille labels to SD cards is not possible as this can damage the Stream. To help identify SD cards, the Stream supports an optional voice label. The label is stored in a reserved text file named $VRLabel.txt saved on the root of an SD card. In the Tools menu of the free HumanWare Companion software, you can create this label file for your SD card. You can also use Windows Notepad to create the file. Whenever you insert the card or turn on the Stream, it will look for this label file. If found, Stream will announce its contents. The label cannot exceed 50 characters. Stream will ignore any characters after the first 50 in the label file. This label file is optional. If you do not require an auto-announcing label for your SD card there is no need to set it up. This label is not related to the label you see when viewing the SD card Properties in Windows.

## SD Card Detection

When playing the contents of an SD card and when it is removed from the player, the player will switch to internal memory if there are files present. If no files are present in the internal memory, the player will enter the key describer mode.

If the player is powered On with both an SD card and/or files in its internal memory, the system will scan the SD card first and will select the last book played. If the last book read is not present, it will select the first available book present on the SD card. If no books are found on the SD card, the system will scan for books on the internal memory.

## Bookshelf Structure

The Stream recognizes various book types which are saved in separate folder structures called Bookshelves. To select the Bookshelf that you want to read, press the Bookshelf (1) key multiple times. It is also possible to navigate through your bookshelves by list instead of rotation. In the Stream’s Configuration menu, under the General submenu, there is an option named “Bookshelf navigation mode” that can be toggled from “rotation” to “list” and vice-versa. If the option “list” is selected, while pressing the Bookshelf (1) key, your bookshelves (offline or online) will be listed. Navigate in this list with the 4 and 6 keys, then press the pound key on the bookshelf you want to select and you will be directed to that bookshelf. In a bookshelf, to move backward and forward along each Bookshelf, use the 4 and 6 keys. The Bookshelf list is circular. Except for the Notes bookshelf and bookshelves for online services, only the non-empty bookshelves are announced. On the SD or internal memory, each Bookshelf is stored within a reserved folder name beginning with “$VR”. Within each of these reserved folders (Bookshelves), there may be user-defined subfolders containing separate books or file categories. You may place other folders and files on the SD card, but it is only the contents of the $VR reserved folders that define the Stream bookshelves. When you insert a card that is not write-protected, Stream will automatically create $VR reserved folders.

Bookshelf structure and usage are defined as follows:

Bookshelf: DAISY Talking Books ($VRDTB folder).

Book types: DAISY, NISO and unprotected EPUB books (see Technical specifications sections for more details).

Usage: To navigate in this bookshelf, it is recommended that the files comprising each of your DTB books should be saved in separate subfolders under this folder. Single book files such as EPUB files may be saved in the root of this folder. Use the 2 or 8 keys to select folder or book navigation levels and keys 4 or 6 to navigate at the chosen level. Press PLAY to select the book. In this bookshelf, the current reading position and bookmarks are saved separately for each book.

Bookshelf: Other Books ($VROtherBooks folder).

Book types: Non-DAISY, non-NISO recorded books composed of any of the Stream's supported audio file types (see Technical specifications section for additional details).

Usage: To make navigation in this bookshelf easier, it is recommended that the files comprising each book should be saved in separate subfolders under this folder. Folders may be nested. Optionally, single-file books may be directly placed in the root of this folder. Use the 2 or 8 keys to select folder or book navigation level and keys 4 or 6 to navigate at the chosen level. Press PLAY to select the book. In this bookshelf, the current reading position and bookmarks are saved separately for each book. Playback stops at the end of the last file of each book.

Bookshelf: Music ($VRMusic folder).

Book types: Playlists and recorded files composed of any of the Stream's supported audio file types (see Technical specifications section for more details).

Usage: A music book is either a playlist or the entire music structure (referred to as All Music book). The current reading position and bookmarks are saved separately for each book. Navigate between music books (playlists) using the 4 and 6 keys on the bookshelf. The All Music book consists of nested folders containing audio files. For example, the structure can be genre, artist, album and tracks. Navigate the All Music book using 2 or 8 keys to choose folder or file navigation level and then move within that level by using 4 or 6 keys. Playback continues from the last file of one folder to the first file of the next folder. Random options apply only to the Music books.

Bookshelf: Saved Podcasts ($VRPodcasts)

Book types: Any of the Stream's supported audio file types (see Technical specifications section for more details).

Usage: Each saved podcast file is defined as a separate book meaning you must move from file to file using the 4 or 6 keys while in the Bookshelf. Files may be in subfolders in which case you use keys 2 and 8 to select folder or book navigation levels. Reading positions and bookmarks are saved separately for each book (file). Playback stops at the end of each book (file). In addition to podcasts, it can be used to place the audio track of a movie, a concert or any lengthy recording that the user wants to consider as one item for current position and bookmarks purpose.

Bookshelf: Text ($VRText folder).

Book types: all types of text files supported by the Stream (see Technical specifications section for more details).

Usage: Each text file is defined as a separate book meaning you must move from file to file using the 4 and 6 keys while in the Bookshelf. Files may be in subfolders in which case you use keys 2 and 8 to select folder or book navigation level. Reading position and bookmarks are saved separately for each book (file). Playback stops at the end of each book (file). Note: If your braille file is not playing correctly, check that you have selected the correct braille translation table in the configuration menu (Key 7). Some text files might have different encoding. If the file doesn’t play well, you can change the default text encoding in the configuration menu.

Bookshelf: Notes ($VRNotes folder)

Book types: Recorded audio notes

Usage: This is a single list of files recorded by the Stream where each file is given an incremental number (on which you can add a customised filename) for each separate recording. Each note is defined as a single book, meaning there is one current reading position and set of bookmarks for each note.

Bookshelf: Radio Recordings ($VRInternetRadio)

Book types: Recorded Internet radio tracks

Usage: Each saved radio recording is defined as a separate book, meaning you must move from file to file using the 4 and 6 keys while in the bookshelf. Press Play/Stop to play the selected recording.

Note: Folder names are shown in mixed case for readability, but Stream does not require mixed case.

## Other Reserved File Names

The Stream may create other file names or folders beginning with “$VR” on an SD card. Removing or altering these files could result in unpredictable behavior.

## Transferring Files Between Your Computer and Stream

To connect Stream to your computer for transferring files, connect the small end of the USB cable to the USB-C port on the bottom edge of the Stream and the other end to your computer's USB port. Windows will recognize the Stream and you can then use Windows Explorer to transfer files back and forth. Stream will also recharge while connected although the recharge time could be longer than recharging from AC power because the power supplied by the computer USB port may be less than from AC power. If you have an SD card inserted into your Stream, both the Stream’s internal memory and the SD card memory will be available on your PC, under separate folders. Inserting or removing the SD will have that SD card folder added or removed. If you have an SD card reader on your computer, you may find it easier to transfer files using the computer's SD card reader than using the USB cable with the Stream.

If you are not familiar with file transfers using Windows Explorer, you may want to install the optional HumanWare Companion software that may be downloaded for free from the HumanWare.com web site.

## HumanWare Companion

The HumanWare Companion is a Windows program used in conjunction with the Victor Reader Stream, Stratus or Trek to manage your books, music, saved podcasts, voice notes, text notes, and text files. You can use the HumanWare Companion to copy your books, music, saved podcasts and text files to your player’s storage card, or remove these files from the storage card. You can also use the HumanWare Companion to update your Stream’s software. To do so, connect your Stream to the computer with the USB cable, or load an SD card into a computer card reader. To download and install the HumanWare Companion software, please visit: www.humanware.com/companion.

# Basic Functions

## Changing Volume, Speed and Tone/Pitch

When the Stream is on, press the Power button on the top left side multiple times to toggle volume, speed, or tone/pitch. Depending of the bookshelf, Tone/Pitch can be replaced by bass/treble settings. After 10 seconds of non-use, the toggle control will return to Volume. Use the Up / Down arrows on the left side just below the Power button to raise or lower the selected setting. There is a beep to mark the upper and lower range of each control. If the book is not playing, STREAM will announce the setting position. For tone/pitch, bass/treble and speed, there is also a beep to mark the normal or 0 position. This indicates a flat tone setting, a normal pitch or normal speed setting.

If preferred, you can configure the Tone control so that it varies the audio pitch instead for playing audio that is not on the music bookshelf. Some people benefit more from a change in the pitch of recorded audio than a change in the tone. To vary the pitch instead of the tone, press key 7 to open the Navigation and Playback Menu. Then use the right arrow to find Audio Adjustment mode and use the pound key to select Pitch.

To comply with governmental regulations from multiple countries, the Stream must enforce a high volume warning notification system. When using a wired headset or headphones, you will receive a warning when you try to increase the volume over the level of 9 out of 20. The warning message is mandatory and must be listened to in its entirety before confirming the volume increase using the pound key. The high-volume confirmation will be repeated after 20 hours of activity, suspend mode and shutdown will pause the timer. After 20 hours of activity, the volume will automatically return below 9 and you will have to listen to the high volume warning again.

### Different Speed settings for TTS and Audio playback

The Stream will remember separate Speed settings for each of text-to-speech and audio playback. You can change the speed of one without affecting the other. The same will apply to all books using Text-to-Speech. Music files are not affected by your speed setting because their speed is always set to normal by default. The TTS speed applies also to messages and menus spoken in TTS.

## Changing Bass and Treble (Music bookshelf)

For the Music bookshelf, the Tone/Pitch setting is replaced with Bass and Treble controls. Press the Power button multiple times to toggle between Volume, Speed, Bass and Treble. To alter the lower frequencies, select the Bass control and either add bass by setting the control to a positive value or remove bass by selecting a setting below zero. Similarly, you can add or remove treble. Adding or removing bass will not alter the treble and vice versa. To hear the music unaltered, set the bass and treble controls to their 0 setting.

## Play/Stop

To start playing a book, press the Play/Stop key.

To stop playing a book, press the Play/Stop key again.

NOTE: Instead of the Confirm key, you can also press Play/Stop to jump to an entered page, heading or bookmark number. If you end your numeric entry with Play/Stop then playback will start at the specified page, heading or bookmark number.

## Rewind and Fast Forward

These keys allow you to go backward or forward quickly.

Press and hold the Rewind or Fast Forward key until you reach the desired position, the movement of the Rewind or Fast Forward keys will accelerate the longer you hold the key down. For each three seconds you hold the keys, you will move forward or backward 30 seconds, 1 minute, 2 minutes, 5 minutes, 10 minutes, 15 minutes and so on. The largest jump is 5 minutes. Between each three second jump, Stream will announce the time jump and play a clip at normal.

If you simply press and release Fast Forward or Rewind instead of holding it down, playback will jump forward or backward by 5 seconds.

If you are reading a text file, pressing and holding the Rewind and Forward key permit to jump from percentages instead of time: 1%, 2%, 5% and so on. The largest jump is 5%. If you do a short press on the Rewind or Forward key, your text will jump from a line.

## Sleep Mode and Time announcement

The Sleep button allows you to obtain time announcements and set the Sleep Timer.

Press multiple times to set a Sleep timer of 15 minutes, 30 minutes, 45 minutes, 60 minutes and custom, after which the Stream will automatically shut off. The Custom option lets you enter the desired number of minutes using the keyboard. Your custom timer can last 99 minutes or less. End your entry with Confirm or Play. You may configure which Sleep timer values appear using the settings menu.

To deactivate the Sleep function, press Sleep repeatedly until you reach the OFF option.

One minute before the timer powers off the player, you will hear a message informing you that 1 minute of Sleep remains. When the timer expires, the player announces that it is powering off. If desired, you may turn off these warning messages in the Playback and Navigation section of the configuration Menu (key 7).

Please note that if you put your device in suspend mode, all the active custom sleep timers will be deactivated.

Press & hold the sleep key to hear the time and date. You can configure the way the time and date are announced in the settings menu.

## Setting the Date and Time

To set the date and time, press key 7 to open the Configuration menu. Use keys 4 and 6 to access the item General Settings. Use keys 4 and 6 to access the item System and press Confirm. In the System menu, use keys 4 and 6 to access the item Date and Time and press Confirm. Use the 4 and 6 keys to navigate the menu. Date and time menu items are as follows: Change Time, Change Date and Advanced Time Settings. When on the menu item of your choice, press Confirm to enter that given menu. This menu will have various submenu items. Enter the correct value for each submenu using the numeric keys. Press Confirm to move to the next submenu item or press Star to return to the menu. If an invalid entry is made, the operation failed. When you reach the last submenu, press Confirm to confirm your settings and return to the date and time configuration menu.

The daylight savings setting is a quick way to manually adjust your clock forward or back one hour. You can manually activate Daylight Saving Time in the Advanced Time Setting submenu.

Note: This setting does not make the player automatically change the time at the predetermined date twice a year.

Example:

If you wish to modify the current month:

- Access the Date and Time menu.

- Press 6 once to reach the Change Date menu and press Confirm.

- Press Confirm once to reach the Month submenu.

- Enter the month (i.e. 1 to 12) using numeric keys.

- Press Confirm to move to the next submenu (day).

- If you do not wish to modify the day, press Confirm again to reach the end of the Change Date menu, at which point any settings entered in the Change Date menu will be saved.

- Press the Back button to exit the Time and Date settings menu.

## Key Describer Mode

Press and hold the Info (0) key to access the Key Describer Mode. Press and hold the Info key again to exit the Key Describer Mode.

If there is no content available, the key describer mode will be activated automatically.

While in Key Describer mode, you may press any key to hear its functions.

# Numeric Key Functions

## Numeric Keypad List

- 1: if pressed, Bookshelf; if pressed and hold, entering in the user guide

- 2: Navigation element

- 3: Delete / Copy / Move

- 4: Back

- 5: if pressed, Where am I; if pressed and hold, Short info (user keys, serial and version number)

- 6: Forward

- 7: if pressed, Menu; if pressed and hold, Text-to-Speech voice toggle

- 8: Navigation Element

- 9: Text-to-Speech / Recorded audio modes toggle / Random and Loop modes toggle (Music)

- Star: if pressed, Cancel; if pressed and hold, keypad lock

- 0: if pressed, Info; if pressed and hold, Key Describer mode

- Pound: if pressed, Confirm; if pressed and hold, battery level and download information

## Navigation Keys

Stream allows you to navigate by chapter, section, page, time jump, paragraph, or any other indexed element defined by the producer of the book.

Use the 2 and 8 keys to choose a navigation level. These differ from book to book but typically level 1 means chapter, level 2 means section, and level 3 means sub-section and so on. For some books the actual name of the level will be said (Chapter, section, etc.) The page element may not be present on all books. The phrase level is usually the smallest navigation element but this is defined by the book producer. Stream limits phrase jumps to a maximum length of 1 minute. First, choose a navigation level (chapter, page, etc.), using the 2 and 8 keys. Then use the 4 and 6 keys to move backward and forward between elements at the selected level. The 2 and 8 keys will announce only the levels available for the book. The phrase level is always provided for DAISY books but its extent depends on the book producer. If bookmarks are present in the book, you can choose to navigate by bookmarks. You can choose to save the last used navigation level for each book from the Navigation and Playback configuration menu.

### Undo navigation

Pressing the Star key within 10 seconds of any single move backwards or forwards using keys 4 or 6 or single go to page move will undo the operation and return you to your previous position.  The Undo function does not apply to the Rewind or Fast Forward buttons.

## Time Jump Navigation Mode

Time Jump navigation can be selected using the 2 and 8 keys.

Press the key 4 to move backward or the key 6 to move forward by the jump interval.

You can choose to turn on or off any or all of the time intervals (30 seconds, 1, 5, 10 or 30 minutes) in the Navigation and Playback section of the configuration Menu (key 7). Only the time jump intervals you turned on will appear in the up and down navigation menu.

## Text-to-Speech Navigation Elements

When Stream is in Text-to-speech mode, there will be additional text navigation levels added to the 2 and 8 (up and down) key rotation. These include the text elements: screen, paragraph, line, sentence, bookmark, word, spell and character. If you do a search in a text, then a Search level is added to allow search of the next or previous search term. Screen and line elements are defined arbitrarily as 25 lines per screen and a maximum of 80 characters per line. As with other navigation levels, you can move between elements of the selected level using the 4 and 6 (left and right) keys. The Screen element will only be available when no pages are defined in the book.

## Spell Mode for Text Content

Spell mode can be selected using the 2 and 8 keys. The Spell mode can be found between the Word and character navigation levels. Use keys 4 and 6 to move word by word. Stream will announce the word and spell it. Capital letters are indicated and that word will be spelled out.

## Bookshelf Navigation — Key 1

Press the Bookshelf (1) key multiple times to rotate through the circular list of Bookshelves. With the exception of Notes and configured online services bookshelves, empty bookshelves will not be announced. Bookshelves contain different types of books and files as defined in the section Bookshelf Structure. It is also possible to navigate through your bookshelves by list instead of rotation. In the Stream’s Configuration menu, under the General submenu, there is an option named “Main menu mode” that can be toggled from “rotation” to “list” and vice-versa. If the option “list” is selected, while pressing the Bookshelf (1) key, your bookshelves (offline or online) will be listed. Navigate in this list with the 4 and 6 keys, then press the pound key on the bookshelf you want to select and you will be directed to that bookshelf. In a bookshelf, move backward and forward along a Bookshelf using the 4 and 6 keys. Press the Go To key (above key 1) to enter a relative book number. End the book number entry with the Confirm key to move to the entered book number and remain on the bookshelf. When you reach the desired book, press Play to have the Stream start to play from where you left off in the selected book.

Note: if you have selected the “list” option to navigate through your bookshelves and if you are reading a book, you will have to press one time the (1) key to return to the books list for this current bookshelf, then press a second time the (1) key to be directed to your bookshelves list.

### Online Bookshelf Navigation

The Stream contains two bookcases, offline and online, each containing multiple bookshelves. Toggle between the offline and online bookcases using the Online button above key 2. The online bookshelves have the same structure as the offline bookshelves; use the Bookshelf (1) key multiple times to rotate through them, and the 4 and 6 keys to move backward and forward along them. If you prefer to navigate through bookshelves by list, press the 4 and 6 keys to navigate through the list of offline or online bookshelves, then press the pound key on the desired one.

### Multi-Level Bookshelf Navigation

Optionally, you can create a multi-level folder structure for the Talking books, Other Books, Saved Podcasts and Text File bookshelves. Instead of only cycling through books one at a time with keys 4 and 6, you can choose the level of navigation with the UP and DOWN keys 2 and 8. Then, when pressing keys 4 or 6, you will move from folder to folder at the chosen level in a circular fashion. When you arrive at the folder of interest, press keys 2 or 8 again to select the book level within that folder and proceed to select the desired book. Navigating at the book level will also move from folder to folder. The Stream will navigate up to 8 levels of folders under the $VR folder. If you have more than 8 levels, the additional levels will be recognized by the Stream as level 8. If you also put files in the root of the $VR folder, those files will only be listed at the Book level of the 2 and 8 rotation.

Note: Multi-level bookshelves are not mandatory. If you choose not to organize books at multiple subfolder levels, you can just browse the bookshelf with keys 4 and 6 to navigate at the Book level.

## User Guide — Key 1 (Press and hold)

You can access the built-in user guide at any time by pressing and holding key 1. Press and hold key 1 again to exit. The built-in user guide is in DAISY format, so it can easily be navigated by heading and section levels.

## Manage Books — Key 3

When browsing bookshelves or from a book, you can manage your books by rotating between the following actions with key 3: Delete, Copy, Copy All, or Move. Additionally, other operations are available when pressing the key 3, depending on each bookshelf. To know more about the options available with the key 3 in a specific bookshelf, press multiple times the key 3 to rotate between options. When you heard the desired option, press the Confirm key to select it. You can press the Star key at any time to cancel and return to your book. The actions that are available for each book depend on the type and location of the book. There are some exceptions, but the basic rules are:

- Books located on an SD card or the internal memory can be deleted.

- Books located in the internal memory can be moved.

- Books located on the Online Bookshelf can be moved or deleted.

While browsing or playing the files in the All Music book, you can press key 3 to delete a single file. Press the Confirm key to confirm deletion or any other key to cancel. To delete a folder, navigate back to the folder’s level. Press key 3 to delete the folder and the Confirm key to confirm deletion or any other key to cancel. You may not however delete a file from a playlist.

While browsing the Notes book, you can press key 3 to delete a single Note file. Press the Confirm key to delete the selected item and the Stream will ask you to confirm. Press Confirm again to delete or any other key to cancel deletion. You can consolidate your notes by pressing key 3 twice. Consolidating your notes will number them from 1 to the number of notes you have. Press the Confirm key to consolidate your notes and press the Confirm key again to confirm or any other key to cancel.

Note that books can be stored on both the SD card and the internal memory of the player. For book management, the behavior for SD cards and internal memory is the same.

## Download manager (key 3 press and hold)

While booting your device or going out of suspend mode, or after specific podcast intervals (depending of your selection in the Podcasts menu), your device will download online content. It happens often that downloads are grouped; in that situation, several elements, provided by multiple apps (podcasts, some DAISY Online accounts, etc.), are included in the download queue and are downloaded one after the other. It is now possible to prioritise some downloads among others to better fit your moments needs. For example, maybe you want to read a specific book soon or you want to cancel a download to do it later.

To manage your downloads, press and hold the 3 key. All your downloads (items in the download queue and the item that is now downloading if any) will be listed here. It will first say the number of the item representing its position in the download queue starting by 1. The number 1 is the first item in the download queue that will be downloaded after the current one. the download now in progress is not numbered. Then, it says the item name, then the app name, then its status (downloading followed by the download progress in percentage, or in the download queue, or “download completed” if this download was completed while you are navigating in the download manager). If no download is present in the download queue, the message “No downloads” will be said.

Navigate through your downloads with the 4 and 6 keys. You can navigate through the available options by pressing several times the 3 key. The available options are: “Download next”, that will put the selected item in the position numbered 1 in the download queue and will download the selected item just after the item that is now downloading if any, “Cancel download”, that will remove this item from the download queue, and “Cancel all downloads” that will empty the download queue.

To select an option, press multiple times the 3 key until attaining the desired option, then press the Pound key to select. To cancel a specific download or to cancel all downloads, you will have to confirm the operation by pressing a second time the Pound key. Finally, to leave the Download manager, press the Star key. Please note that if no downloads are present when opening the Download manager, you will be returned automatically to your previous position.

Note: the software update file is visible in the download manager. When you receive a prompt to update your system and you accept this update, the software update file will automatically be downloaded, prioritising itself among any other file, including the current download. To download the software update file later, it will be necessary to cancel the update.

## Where am I? — Key 5

The Where am I key will announce your reading position without interrupting the reading process. Depending on the book type, Stream will announce one or more of the pages and heading numbers, section title, or file name. For Music, it will announce current folder and file names and the file times. For Talking books, Other Books, Saved Podcasts and Notes, it will announce the percentage of the book or note played as well as individual file times. For notes, it will also announce the category where the note can be found. For text files, it will announce the percentage of the file played. Reading will resume once this information has been announced. For talking books, the time remaining in the section will also be announced. For most books, this translates as the time remaining in the current chapter. Note that time information is not always available.

When you press and hold the key 5, it will give you a shortened version of the Info menu containing the version number, number of user keys installed and serial number.

Note: You can customise the amount of information given when pressing the Where Am I or the Info key while reading a book. To do so, navigate to the Configuration menu (key 7) until attaining “Navigation and playback”, then, in this submenu, navigate with the 4 and 6 keys until attaining the “Book time details” option. You can hear the time remaining, the time elapsed or both.

### Where am I tag information

For mp3 and mp4 files, if you press Where am I (key 5) twice, the Stream will announce ID3 tags data. While listening to an Internet Radio station, pressing Where am I (key 5) once will announce the song or Stream title if available. Pressing Where am I (key 5) twice will announce additional information about the station if available, such as its Name, Genre, URL, Publicity, Content type, and Bitrate. If you don't want to hear all the tags, press Play to interrupt the tag information and resume playback.

### Where am I for online books

While reviewing the list of results after doing an online book search, press Where am I (key 5) to hear additional information about the book such as the book synopsis.

## Multi-Voice Text to Speech (TTS)

The Stream allows the usage of one or two voices of your choice: one menu voice and one additional voice.

You can toggle which voice is used to read the content by pressing and holding key 7. The menu is always read with the menus voice as the name implies.

## Audio, Text, and Random Music Playback Modes — Key 9

There are two types of Playback modes available: Recorded audio and Text-To-Speech (TTS). For DAISY or NISO books with full text/full audio, use the 9 key to switch back and forth between the recorded audio and the same position in the equivalent electronic text. The synchronization depends on the book producer but typically text and audio are synchronized to the paragraph or sentence element.

The last used playback mode is saved between reading sessions. Only the modes available for the selected book will be available for selection with the 9 key. If the book is only recorded or only text then the appropriate mode is automatically selected and the 9 key has no choices available.

When playing Music, you can press key 9 to toggle between the following playback modes: Random Off, Random On, Loop Folder, and Loop Single. Random On will randomize your playlist elements, while Loop folder will automatically start playback from the beginning of the current folder when the end of the folder is reached. Loop Single will automatically start playback from the beginning of the current file when the end of the file is reached. Note that the Loop Folder and Loop Single modes have priority over the Loop settings in the configuration menu.

## Confirm, Lock and Cancel — Pound and Star keys

The Pound (Confirm) key allows you to confirm an operation or end a numeric entry such as a page number or a text entry such as a text search item. Press and hold Confirm to announce battery and recharge status as well as download information (current download progress and number of downloads in the downloads queue).

The Star key allows you to cancel an operation. If you press and hold the Star key, it will also lock the keypad. To unlock, press keys 1, 2, and 3 in sequence. There is one exception to the keyboard locking and it is the power off. So, a short press on the Power key will not shutdown the device and a message “Keyboard lock” will be heard, but holding down the Power key for 2 seconds will power down the player.

## Info — Key 0

The Info (0) key announces book, player, and battery information. There are two ways to access the information.

Press the Info (0) key. All information items will be announced. You can also press the Info (0) key and press key 4 or 6 to move backward or forward through the items. Pressing the Info (0) key repeatedly will also permit to move through each item.

To cancel the Info messages, press the Star or Play keys.

You may also press and hold the Info key to enter and exit Key Describer Mode.

While the information items are announced, the Key Describer Mode cannot be activated.

### Available Information Items

Book title

Number of Notes (Notes bookshelf only)

Number of files and folders (Music Bookshelf only)

Number of pages where available and number of headings

Recording time remaining (Notes bookshelf only)

Total time in the book

Playtime elapsed and playtime remaining

Number of Bookmarks set in the book

Number of Books

Note: You can customise the amount of information given when pressing the Where Am I or the Info key while reading a book. To do so, navigate to the Configuration menu (key 7) until attaining “Navigation and playback”, then, in this submenu, navigate with the 4 and 6 keys until attaining the “Book time details” option. You can hear the time remaining, the time elapsed or both.

Note: the information about the internal space is announced at first, then the information about the SD card.

Internal memory space remaining

SD space remaining

Battery status or, if the player is operating on AC power, the status of the battery recharge

downloads Info

Wireless status

VICTOR READER Stream model number, software version number, number of user authorization keys (if any), and player serial number.

# Direct Navigation (Go To Key)

## Go to Page

The Go to Page function allows you to go directly to a desired page.

Press the Go to key, above key 1. If you press this key once, you will hear “Go to page”. Enter a page number. Press Confirm to go to the selected Page. Stream will confirm the entered Page number. You can also press the Play/Stop key to have Stream start playback from the chosen page.

## Go to Heading

If you press the Go to key twice, you will hear “Go to heading”. It allows you to enter a heading number relative to the start of a DAISY or NISO book. This is useful for books that do not have page markers. For example, you could jump directly to a chapter by going to its heading number rather than advancing chapter by chapter.

End your heading number entry with Confirm to move to that heading or end with Play to move and start playback.

## Go to Time

The Stream has a Go To Time feature for DAISY or NISO recorded books, Other Books, Saved Podcasts, and Audio Notes bookshelves. While in these bookshelves, press the Go To key until you hear, “Go To Time”. Then, enter a time in hours and minutes relative to the start of the book or audio note. The last 2 digits of the entry represent minutes and the other digits represent hours. The minutes can range from 00 to 99. For example:

- Enter 1 to position 1 minute from the start of the book or note

- Enter 12 to position 12 minutes from the start

- Enter 123 or 83 for 1 hour 23 minutes

- Enter 1200 for 12 hours and 0 minutes

## Go To Percent

When reading files in the Text Bookshelf or audio files in the Other Books or Saved Podcast bookshelves, the Go to Page key will become a Go To Percent key for navigating to relative positions x% from the start of the file. Press Go To Percent and then enter a percentage from 0 to 100. Press Confirm to position to that relative position in the file or press Play to position and start playback at that relative position. For example, entering 0 will position to the start of the file, entering 50 will position to the middle of the file, and entering any number greater than 99 will position to the end of the file. For text files, Stream moves to the beginning of the paragraph that contains the position of the specified percentage.

## Go to Beginning and End of Book

To quickly reach the beginning or end of a book, you can press the Go to key followed by Rewind or Play to go to the Beginning of the book or press the Go To key followed by Fast Forward to jump to the end of the book.

## Go to Book

If you are browsing the Bookshelf, the Go to Page key becomes a Go to Book key allowing you to enter a relative book number on a given Bookshelf which is useful for Bookshelves that contain many books. Press Go To key and enter a number for the book you want to go to. You can then press Confirm to navigate to the book or Play to start playing the book.

## Skip back or forward 10 items

While holding the 4 and 6 keys when navigating in all bookshelves, you can skip back or forward 10 items. For example, when on the results list after a search, or in the books list on a bookshelf, while holding the 4 and 6 keys, you will skip back or forward 10 items.

## Functions for Online Services

Special functions are available on the Go To key when browsing the online bookshelves while connected to a wireless network. If the Go To key is pressed twice when browsing the NFB Newsline bookshelf, it allows you to manually synchronize downloaded content with the NFB Newsline online service. When browsing the Bookshare or NLS BARD bookshelf, pressing the Go To key twice will allow you to search for books using the online services. When browsing the Internet Radio bookshelf, pressing the Go To key will allow you to navigate to a specific radio station or a specific playlist containing radio stations. Then, press the Play key to start listening the radio station selected. From the References bookshelf, the Go To key also allows you to search for Wikipedia and Wiktionary references from a book or go directly to a specific file. From the Podcasts bookshelf, you can use the Go To key to add podcast feeds. In the TuneIn Radio bookshelf, while listening to a podcast episode, the Goto key allow to go to a specific position in the podcast episode you are listening. TuneIn Radio options can be accessed by pressing the Go To key multiple times until reaching the desired options, which are: Go to, Search TuneIn Stations, Search TuneIn Podcasts and Browse.

For all online searches, the Go To key can be used to go directly to a specific search result by entering the search result number followed by Confirm.

# Advanced Functions

## Text Search

The Stream has a text search function for text books. Using the multi-tap text entry method on the number keys as you would to enter text on a telephone, you can type in your search item. If you use the Stream with a different text-to-speech language, the typing keypad will offer the characters of the currently selected language. Press and hold key 7 to switch from one text-to-speech language to the other.

The following describes how to perform a text search from a book in the Text Files bookshelf. This method can also be used to search DAISY or NISO books (e.g. Bookshare) found in the Talking Books or online bookshelves providing the book has text and the player is in text-to-speech mode (key 9). DAISY or NISO books that are recorded only cannot be searched as they have no text.

Prior to searching in text for the first time, you should select your preferred method of typing on the Stream. There are two Multi-tap text entry methods: “Announce final character only” and “Announce character on each key press, then enter character after pause”. Select your desired method from the Multi-tap text entry method section of the configuration menu (key 7).

This can also be toggled by holding the Bookmark key for a few seconds when in a text entry field.

Each button is listed with its letter or symbols in order of their appearance.

When entering your password, you may toggle between lowercase, uppercase, and numeric only with the Bookmark key. Please note that the possibility to switch from lowercase to uppercase is only present in text fields that are text sensitives, for example when you enter your password. In other text fields, you only have the choice to switch between text and numeric. When using a non-English voice, the list will vary based on the language.

1 Key:  1, period, comma, question mark, dash, forward slash, colon, semi colon, single quote, quotes, back slash, less than sign, greater than sign, opening bracket, closing bracket

2 Key:  a, b, c, 2

3 Key: d, e, f, 3

4 Key: g, h, i, 4

5 Key: j, k, l, 5

6 Key: m, n, o, 6

7 Key: p, q, r, s, 7

8 Key: t, u, v, 8

9 Key: w, x, y, z, 9

0 Key: space, 0, exclamation mark, at sign, pound sign, dollar sign, percent sign, caret, ampersand, asterisk, opening parenthesis, closing parenthesis, underscore, plus sign, equal sign, UK Pound sign, euro, Yen sign

Follow these steps to use the search function:

- Open a text file from the Text Files bookshelf.

- Press the Go To key multiple times until you hear, “Search in text”.

- The Stream will announce the current word in the text. Press Confirm or Play to search for other occurrences of this word in the text.

- You can also add text to this word, delete single characters starting from the end of the word, or delete the whole word in one key press. Here’s how:

  - To enter your text with the “Announce final character only” method, press the number keys 0 to 9 to type the text. For example, key 2 is used for letters a, b, c, and 2. Key 3 has d, e, f, and 3. Key 6 has m, n, o, 6 and so on. The space character is on key 0. Punctuation and special characters are on keys 0 and 1. To enter a letter, press the number key associated to that letter the appropriate number of times. When entering successive letters on the same key you need to pause until Stream announces the letter. For example, to type the word, “cab”, you press key 2 three times, then pause until you hear, “c”, then press it once, pause until you hear, “a”, then press it twice to type the last letter, “b”. You do not need to pause to type successive letters that are on different keys. For example, to enter the word, “mama”, you can press 6 2 6 2 as quickly as you like.

  - In the “Announce character on each key press, then enter character after pause” method, the Stream will announce the character as soon as the key is pressed, leaving enough time to press again and select another character on the same key. After a short time, if no other key is pressed, the Stream will make a clicking noise, indicating that the character has been entered. Like in the previous method, you do not need to pause to type successive letters that are on different keys. Pressing another key enters the previous character automatically, and no clicking noise is made. This method allows you to find the letters on the numeric keypad more easily.

- Both text entry methods allow you to use the Bookmark key to toggle between lower case, upper case when applicable, and Numeric input types while entering your text to search.

- You may press and hold the Info key to enter the Key Describer mode. After that, press other keys to announce their function including announcing the characters for each number key. Press and hold the Info key again to exit the Key Describer and return to Text Entry mode.

- To announce what you have typed, press the Fast Forward key.

- To erase the last character typed, press the Rewind key.

- To erase all the characters at once, press and hold the Rewind key until you hear a beep.

- When you finish typing, you can exit the text entry mode in 3 ways: Press Cancel to exit without searching, or press Confirm or Play to start the search. Cancel will not affect any previous search information.

- If you pressed the Confirm key, the Stream will position itself at the location of the found search item and announce the line. If you pressed the Play key, it will move to the found search item and start playing. The search is case insensitive. The search will find partial words in which case it will position at the start of the word containing the search text. Accented characters are not matched to non-accented characters or vice versa. A search always starts from the current position in a book.

If searching a large file, you may hear “Please Wait” while the search is in progress. If you press the Cancel key while searching, the search function is stopped, the current position remains at the original starting position.

### Search Next or Previous

- Upon finding the search item, Stream will add a Search item to the navigation levels of keys 2 and 8 and select that level automatically. So, upon finding the first occurrence of your text, you can simply press keys 4 or 6 to find the previous or next occurrence. The previous or next Search function will remain on the 2 and 8 key rotation until the book is closed. You can press key 4 or 6 to search for the previous or next item while in play or stop mode.

- If a search forward is performed and the end of the book is reached, the message "End of book" will be announced, and the search then continues from the beginning of the book down to the original starting position of the search.

- If a search backward is performed and the beginning of the book is reached, the message "Beginning of book" will be announced and the search then continues from the end of the book up to the original starting position of the search.

- If no match is found, Stream will announce the search item was not found and return you to the original starting position.

- When you open a new book, the previous search item is cleared.

### Other types of text search

Here’s a list of all search functions that use the multi-tap text entry method as described above.

- Text search in a text book.

- Search on Wikipedia and Wiktionary from either a text book or directly from the References bookshelf.

- Search for music files in the All Music and Music Playlists books.

- Search online services such as Internet Radio, Podcasts, NLS BARD and Bookshare.

## Recording Audio Notes

You can use Stream for voice recordings which are called audio notes. These notes will be saved on the SD card or on the internal memory (depending of the option chosen in the “Recording and audio bookmarks save location” submenu), in the $VRNotes folder. By default, each recording is assigned an incremental numeric filename. You can also add customised filenames. You can record using either the built-in microphone, the microphone of a headset connected in the audio jack or an external microphone plugged into the microphone jack located at the bottom left of the player. The built-in microphone is omni directional which is useful for recording meetings or lectures while an external directional microphone may be better for recording interviews. The maximum length of a single Audio note is 24 hours or a file size of 2 GB, whichever comes first. The number of Audio notes you can record is only limited by the space left on your SD card or on the internal memory of the device.

There are two ways of recording notes. You can either use the quick-record feature or use the standard procedure.

To quick-record a note at any point, press and hold the Record button located on the right side of the player. A message and beep will be heard. Talk into the player's integrated microphone (located just above the Go To key) or into an external microphone. To end quick recording, release the Record button.

To record a long message, press the Record button located on the right side of the player to start the recording. To pause and resume recording, press the Play/Stop button. Press the Bookmark key during the recording to insert a bookmark. To end the recording, press the Record button again. While in recording mode, you may press the Star key to cancel the recording, and then press the Star key again to confirm that you really want to cancel the recording.

Please note that while recording with a plugged-in microphone, no system message will be heard, including if the battery level gets low, so make sure you have plenty of power or are plugged in to a power source.

To listen to your audio notes, use the Bookshelf (key 1) to find the Notes (SD card and/or internal memory) bookshelf. Press Confirm to enter the Notes bookshelf. Then, use the 4 and 6 keys to move from folder to folder (Generic or another folder, see section 7.6 “Notes” to know more). Then, press the Pound key to enter to the desired folder, then navigate with the 4 and 6 keys to move from note to note. You may choose to record in MP3, WAV or FLAC file types. Press menu key 7 to access the Configuration menu. Use the 4 and 6 keys to access the item Recording and press Confirm. Use the 4 and 6 keys to find the recording file type. Then use keys 4 and 6 to choose your desired file type.

Choosing the right recording file type, external source, and recording mode depends on the type of recording you want to do. For example, MP3 files allow you to record more audio than WAV files because of their smaller size, but WAV files offer a higher recording quality. The FLAC format is also available with the same quality of WAV, but smaller in file size. For MP3 files, selecting a smaller bitrate will result in smaller but lower quality files.

You can adjust the recording volume by navigating to the submenu “Recording volume adjustment”. You have the choice between “Fixed” and “Manual”. The fixed adjustment is the option by default, that allows to record with a normal volume value determined by the player. If you want to fine-tune the volume of your recordings, the Manual option will be better to suit all your needs. After selecting this option, you will be able to change the recording volume using the key 2 to lower the recording volume and the key 8 to raise it.

Microphone recording is generally used for voice notes and memos, while line-in recording is best if you want to record music or the radio from external sources. If you can barely hear your recording when using an external microphone, make sure the recording file type is not set to line-in. Stereo recording has a better audio quality and takes twice as much space as mono recording, so use the mono recording mode to reduce the file size or when recording from a mono source, which is the case for headsets and external microphones that are not specifically sold as being stereo.

## Bookmarks

Bookmarks allow you to return to a place quickly and easily. You can place a virtually unlimited number of bookmarks in the same book. The Bookmark key lets you insert and return to bookmarks, hear them listed or remove them.

### Go to Bookmark

Press the Bookmark key (above key 3) until you hear ‘go to bookmark’. Enter the number of the bookmark you wish to go to. Press Confirm. Stream will go to the bookmark and announce its number. Alternatively, press the Play/Stop key. Stream will go to the requested bookmark and begin playback.

### Navigating by bookmarks

Using the 2 and 8 keys, navigate through the navigation options until you hear “Bookmark”. Then, using the 4 and 6 keys, navigate in the bookmarks present in your currently opened file.

### Insert Bookmark

Press the Bookmark key twice or until you hear ‘insert bookmark’. You can also access this function by holding the Bookmark key down instead of pressing it twice. Enter the number from 1 to 99998 that you wish to assign to the Bookmark. Press Confirm to confirm, or the Play/Stop key to confirm and start playback. The bookmark will be stored in memory.

NOTE: If you confirm without entering a number, Stream will assign the first available bookmark number.

NOTE: If you enter 99999 as the bookmark number, an invalid entry message will be announced and the action will be cancelled, since this number is reserved to remove all bookmarks. See Remove Bookmark.

You may even insert a simple bookmark while recording a long audio note. This would be useful to mark an important comment made during a lecture or meeting that you are recording. To set a bookmark while recording, simply press the Bookmark key once. You cannot insert a bookmark during a quick recording (while holding down the Record key).

### Insert audio Bookmark

To record an audio bookmark, do the following:

- Press the Bookmark key twice or until you hear ‘insert bookmark’.

- Enter the number you wish to assign to the Bookmark. You may skip this step and a number will be assigned automatically.

- Invoke the quick recording mode by pressing and holding the Record key and recite a short message. Then release the Record key. Instead of the quick recording method, you may also use the standard recording method by pressing and releasing the Record key, say your message, then press the Record key again to end the recording. Whichever method you use; audio bookmark recordings are limited to 1 minute in length. If the one-minute limit is reached, the recording is stopped, and the bookmark is saved. To play back your audio bookmark, simply go to or browse to that bookmark. You will hear your recording and then playback will continue at the bookmarked position. Unlike audio notes, you cannot fast forward or rewind within your audio bookmark recording.

- To cancel, press the Star key. Cancel is not available while using Quick recording mode.

Audio bookmarks may be recorded during the playback of books or audio notes. The bookmark will be stored in memory and the associated recording will be stored on the SD card or on the internal memory (depending on the option chosen in the “Recording and audio bookmarks default save location” submenu), in a reserved folder named $VRAudioBkm. You should not modify any files within this folder. The recorded files are linked to the simple bookmarks in internal memory. The recorded file name format is x_y_z.MP3 or x_y_z.wav where x is the first 13 characters of the book title text, y is an 8-character computer generated unique book identifier, and z is a 6-digit bookmark number. For bookmarks placed in MP3 files or audio notes, there is no book title. In these cases, x will be “Other______”, “music___”, or “AudioNote____”, respectively. While playing an audio bookmark, you can press the Rewind key once to replay the recorded voice note or press Fast Forward to skip it and start playing the book at the bookmarked position. You cannot Fast Forward or Rewind within the audio bookmark.

### Highlight Bookmark

Highlight Bookmarks are used to define the Start and the End positions of a passage for later playback. Setting highlight bookmarks is a great way to study important passages in text books.

#### Start Highlight Bookmark

Press the Bookmark key 3 times or until you hear “Start highlight bookmark”.

Enter a bookmark number followed by Confirm.

NOTE: You can omit entering the bookmark number in which case Stream will assign the first available bookmark number.

#### End Highlight Bookmark

After setting the starting position, navigate to the point where you wish to mark the end of the highlight.

Press the Bookmark once. You will hear “End highlight bookmark”. Press Confirm or the Play/Stop key to confirm. The current position is set as the ending position. If the end position is placed before the start position, they will be switched. The message "Highlight bookmark "X" inserted" is then played. The user can also cancel the highlight bookmark by pressing the Cancel key. In both cases, the state reverts to Start.

NOTE: When a highlight bookmark starting position is set but not the ending position, its highlight bookmark number becomes unavailable. Stream will announce “Bookmark "X" already exists” if the user enters a regular bookmark with this number.

NOTE: If you remove the book or power off the Stream before closing the highlight, highlight will be auto-completed. If this occurs, the current location will be used as the end highlight position.

#### Go to Highlight Bookmark

When going to a highlight bookmark, if you end the bookmark number entry with the Play/Stop key, playback will start from this current position and stop at the highlight bookmark ending position. If the Confirm key was used, Stream will position at the bookmark but not play. While playing the highlighted passage, you can jump to the beginning by using the Rewind key. At the end of the highlight, you can play it again by using the Rewind key.

#### Remove Highlight Bookmark

When deleting a highlight bookmark, the "Highlight bookmark "X" removed" message will be announced. It is possible to delete a highlight bookmark that was started but not closed. This action reverts the highlight bookmark state to Start.

### Remove Bookmark

Press the Bookmark key four times, or until you hear ‘remove bookmark’. Enter the number of the Bookmark you wish to remove. Press Confirm.

To remove all the Bookmarks in a book, first press the Bookmark key four times or until you hear ‘remove bookmark’, then press the key 9 five times (enter number 99999). Then press Confirm.

### Bookmark alert

While playing a book, you can have the Stream alert you when you pass over a previously set bookmark. If you created an audio bookmark, it will also automatically play the recorded message attached to the bookmark. Bookmark alerts are turned off by default.

Bookmark alerts can be turned on or off. In the Configuration menu, navigate until reaching the “Navigation and playback” option, then navigate until reaching the “Bookmark alert” option, that is Off by default. Press the Pound key to turn it on.

# Configuration Menu – Key 7

The Stream can be customized using the Configuration menu. When using an application, press the key 7 to open the local menu. Press key 7 again to access the global menu. Please note that if there are no local settings in an application, the message “No local settings” will be heard when pressing the first time the Key 7 and you will be directed automatically to the global menu. When in a menu, use the 4 and 6 keys to navigate through the list or items and the Pound (#) key to access an item. Press the Star (*) key to go back one level. In the next sections, we will go through all the device’s menus. You will learn how to interact with the functions of your device.

## General Settings

### Language

#### Download language and voices

##### Change menu voice

Use this item to change the voice that will read the menus. Use keys 4 and 6 to cycle through the available variants of your system language. Press Confirm to accept the variant of your choice. When cycling through the available voices using the 4 and 6 keys, the text-to-speech will read the voice name, then a sample of this voice will be played, helping you to determine if this voice suits your needs. Once you have selected your desired voice, press Confirm to install it. The download and installation of the selected voice will begin automatically and the new voice will apply upon restarting the device.

##### Change Additional Voice

Use this item to add a second text-to-speech voice (note that the additional voice reads content, not menus). Use keys 4 and 6 to cycle through the available languages supported by the device, then press Confirm on the desired language. Then, using the 4 and 6 keys, navigate in the list of variants of your selected language if any, and press Confirm to select the variant of your choice. When cycling through the available voices using the 4 and 6 keys, the text-to-speech will read the voice name, then a sample of this voice will be played, helping you to determine if this voice suits your needs. Cycle through the available voices, then press Confirm on the desired voice. The download and installation of the new voice will begin automatically and the new voice will apply upon restarting the device. Note that you can toggle from one voice to another by pressing and holding key 7.

##### Swap voices

The Swap voices feature allows you to quickly swap the Content voice with the Menu voice. Use keys 4 and 6 to select “Swap voices” then press Confirm to confirm voice swap (note that the unit will automatically shut down after confirmation). Please also note that this option only works if the two voices are in the same language and in your system language.

##### Change system language

The Change system language feature allows you to change the system language. Use keys 4 and 6 to cycle through the available languages supported by the device. Once selected, this will trigger a software update. See section 6.6.1.3 "Download and install a software update" for details.

#### Select content voice

Use the Select content voice to select the voice that is used to only describe content. Press the Pound key to toggle between one voice and the other. The change will take effect immediately.

#### Braille translation table

Different braille translation tables are made available to you depending on the Text-to-Speech language selected. In the Language Menu, use keys 4 or 6 to access the braille translation table item, then the Confirm key to enter in this submenu. Use the keys 4 or 6 to select the braille translation table value of your choice. Press Confirm to accept choice. The first table in the list is the current default. Whichever table you select will become the new default table for the corresponding text-to-speech voice.

#### Select encoding

When a text or braille file is opened, the device will try to use the appropriate encoding associated with the content. Unfortunately, it happens that the document cannot be read properly and in these circumstances, you would have to use the option “Select encoding” to force the device to read your document in another encoding and so, to be able to read your document properly. To change the encoding, go to the item “Select encoding”, then press the Confirm key. You will have access to the list. Select your preferred option, then press the Confirm key to validate. The selected option will become the option by default and will be the first on this list the next time you will return in this submenu.

### System

#### Keyboard

##### Key Beeps

You can turn on or off the beeps heard when pressing keys. Turning off this option will also suppress the second beep heard when starting playback as well as the "locked" message indicating that the keypad has been locked. You can also set the “key beeps” option to “play only”. In this case, all the beeps are deactivated except the beep that is heard when resuming the playback after a pause in a book. The Locked message cannot be turned off for the Power key. The up / down keys on the left side of the player will only beep at their minimum, normal, and maximum positions regardless of this menu setting.

##### Multi-Tap Text Entry Method

There are two multi-tap text entry methods: “Announce character on each key press, then enter character after pause” and “Announce final character only” (default method). You can toggle between methods using Confirm. See section 5.1 "Text Search" to learn how to use each method.

This can also be toggled by pressing and holding the Bookmark key when in a text entry field.

#### Date and Time

##### Change Time

Select this item to change the time on the device. You can set the hours, minutes, and switch from AM to PM (if in 12h hour format).

##### Change Date

Select this item to change the date on the device. You can set the year, month, and day.

##### Advanced Time Settings

Select this item to change the format of the time and date.

You can change the Time Format: (12h/24h); the Date Format (Month/Day/Year, Year/Month/Day, Day/Month/Year); the Daylight-Saving Time/Standard Time; the Time and date announcements (Announce Time Only, Announce Both Time and Date).

#### Sleep

##### Sleep Messages

The Sleep timer's 1 minute warning and Sleep off messages can be turned on or off.

##### Sleep Time Configuration

Select this item if you wish to change the Sleep timer values that are available on the Sleep key: 15 minutes, 30 minutes, 45 minutes, 60 minutes and custom timer values can be individually activated and deactivated.

### Power off options

This is a toggle between “Power off” and “Suspend”. By default, the option is set at “Power off” and if you press and hold the Power button, the device will shutdown as described in section 1.3.1 "Turning the player on and off". If you toggle to “Suspend”, when pressing and holding the Power button, it will activate the suspend mode as described in section 1.3.2 "Suspend mode".

Note: when the toggle is set to “Suspend”, an option is added to the Configuration menu called “Shutdown now?”. Press on the Pound key on this option to shutdown now your device, then press again on the Pound key on the prompt asking you if you want to shutdown now.

### Idle time

You can choose how much time your device should stay awaken. The available choices are: 15 minutes, 30 minutes, 1 hour, 2 hours or never. When the idle time configured will be reached, the device will shutdown or go to suspend, depending on the option configured in the “Power off options” submenu.

Please note that this option applies only when the device is on battery. When plugged, the device will always stay awake until you decide to shutdown or put your device in suspend mode.

### App toggle menu

This option allows to hide some online apps and delete their associated content. Those apps can be toggled ON (visible in the bookshelves list) or OFF (hidden): Podcast, Internet radio, TuneIn Radio, References. When an app is turned OFF, this app will no longer be visible in the Bookshelves.

### Bookshelf navigation mode

This option allows to change how you will navigate through bookshelves. By default, the toggle is  set to “rotation”, meaning you will navigate through your bookshelves by cycling with the 1 key pressed multiple times. By selecting the “list” toggle, pressing the 1 key one time will display a list of your online or offline bookshelves. Press the 4 and 6 keys to navigate through the list, then press the Pound key when positioned on the desired bookshelf. Pressing the Star key will return to the first item of the list.

Note: if you have selected the “list” option to navigate through your bookshelves and if you are reading a book, you will have to press one time the (1) key to return to the books list for this current bookshelf, then press a second time the (1) key to be directed to your bookshelves list.

## Navigation and Playback

### Time Jump

You can choose to turn on or off any or all of the following time intervals: 30 seconds, 1 minute, 5 minutes, 10 minutes, or 30 minutes. Only the time jump intervals you turn on will appear in the up and down (keys 2 or 8) navigation menu when playing books.

### Save Last Used Navigation Level for Each Book

Select this item if you wish to save your navigation level for a book you are reading. With this feature, when reopening the book, the current navigation level remains the same as the one configured before closing it.

### Bookmark alert

The “Bookmark alert” option is Off by default, so when you will reach a location in your book where a bookmark is saved, you will not be warned. Press the Pound key to turn it on.

### Audio adjustment mode

When you listen to audio content except music, this option allows to toggle between Tone and Pitch. By default, the option “Tone” is selected, indicating that if you do short press few times on the power button, you will be able to adjust the tone, along with the speed and volume. If you toggle the “Audio adjustment” to Pitch and then do some short press on the Power button, you will be able to adjust the pitch along with the speed and volume control.

### Loop

Playback will play repeatedly when Loop mode is selected. It will automatically restart again at the beginning when reaching the end of a book.

### Music

#### Random and Loop

Select this menu to configure your music on the device. You have the choice to play music randomly or not, to loop a folder, or to loop a single file.

### End of book

This option allows to the user to decide how they want to be warned when the end of the book is reached. You can toggle between “Message”, in which the message “End of book” will be said, and “Beep”, in which a beep will be emitted when the end of the book is reached.

### Book time details

This option allows to adjust the amount of information given about book details, when pressing the 0 (Info) or the 5 (Where Am I) keys while reading a book. You can hear the time elapsed, the time remaining or both.

## Wireless

Here are a few points to note before describing the wireless configuration menu:

- The Airplane Mode is used to turn off all wireless communication for situations where you are not allowed to use a wireless device. If the Airplane Mode is turned on, then the wireless communication is turned Off. You can also turn on the Airplane Mode in situations where you know you don’t need wireless communication to save battery power. To quickly turn on the Airplane mode, press and hold the Online bookshelf key and the system will announce “Airplane Mode: On”. To turn it Off, simply press and hold the Online bookshelf key again and you will hear “Airplane mode Off”. Finally, if the Airplane Mode is turned on, pressing the Online button will remind you by announcing the message “Airplane Mode: On”.

- The SSID is the name of a network router. It is announced when you perform a Scan for Available Connections. It is possible that a router has been configured to hide its SSID. In that case, it will not appear in the list of connections when performing a scan. To connect to such a network, you need to use the Create a New Connection option and manually enter the SSID.

- The Password is the authentication key used with your network router. It is case sensitive so when entered with the Stream’s multi-tap feature, you may toggle between lowercase, uppercase, and numeric only with the Bookmark key.

- The Nickname is a friendly name to be used by the Stream to identify a network. This optional name will not be spelled as is the case with the SSID so you should enter an easily pronounceable name for the built-in text-to-speech. The maximum length of the nickname is 50 characters long.

### Airplane Mode

The first item of the Wireless menu is “Airplane mode”. Use this item to turn on or off the airplane mode. By default, the airplane mode is on. When the airplane mode is on, the wireless and Bluetooth features are turned off. When airplane mode is turned off, the wireless features are turned on automatically.

### Wi-Fi

#### Wi-Fi

The first item of the Wi-Fi submenu allows to toggle Wi-Fi on or off.

#### Status

Use this item to hear information on the network status. You can use 4 and 6 keys to navigate between the various available network information like the SSID, the signal strength and more.

#### New Connection

Use this menu if you wish to establish a new Wi-Fi connection. You will be given the choice to scan for a connection, to establish a WPS connection, or to manually create a connection.

##### Scan for SSID

Use this item to scan for Wi-Fi routers within range of your Stream. After a few seconds, a list of available routers should be displayed. Use keys 4 and 6 to browse the available router SSIDs that were found. Press Confirm to confirm the router you wish to access. You will then be prompted to enter the password for the chosen router. Enter the password using the multi-tap text entry on the numeric keypad. Press and hold the Info key to access the key describer mode and have each letter and symbol announced when the key is pressed on the text entry keypad. End your entry with the Confirm key. If successful, the router SSID and password will be added to your list of configured connections (routers) and the new connection will be launched as your active router. You will then be asked to enter an optional nickname for the new connection.

##### WPS Connection

Select this item to connect to a protected Wi-Fi network. If a WPS network is detected by the Stream, press on the WPS button on your router to establish the connection.

##### Manually Connect

Use this menu item instead of scanning if you know the specific router SSID you wish to configure or if your router does not broadcast its SSID. You will be prompted to enter the SSID. Use the multi-tap text entry on the keypad to enter the SSID and press Confirm. You will then be asked to enter the router password. Enter it and end with the Confirm key. Use the Bookmark key to toggle between uppercase, lowercase, and numeric only. Press and hold the Info key to access the key describer mode and have each letter and symbol announced when the key is pressed on the text entry keypad. If successful, the router SSID and password will be added to your list of configured connections (routers) and the new connection will be launched as your active router. You will then be asked to enter an optional nickname for the new connection. Please note that both SSID and password are case sensitive.

#### Launch a Connection

By default, the Stream will automatically connect to the best available Wi-Fi router in range. Optionally, you may use the left and right arrows to select from a list of Wi-Fi routers that you have previously configured. Press Confirm to confirm your selection. The selected router will then be used for subsequent wireless data transfer. You only need to use this item when you have configured multiple routers and want to launch an alternate router as your active connection for data transfer.

#### Delete a Connection

Use keys 4 and 6 to select from a list of Wi-Fi routers that you have previously configured. Press Confirm to confirm your selection. The selected router will then be deleted from your list of configured Wi-Fi routers.

#### Import Wi-Fi Configuration

Use this item to process a network configuration file generated with the HumanWare Companion software. Press Confirm to process a network configuration from a file on the SD card. To generate a network configuration file, select the Wi-Fi Configuration item in the Tools menu of the HumanWare Companion. Enter your SSID and password. Optionally, you can enter a nickname for the network. The configuration file will be saved on the Stream’s SD card.

#### Validate Connection

Use this item to validate your current connection to the Internet. The Stream will connect to a webpage and if successful, the Stream will prompt a short message that informs you of a successful connection. You can replay the message by pressing the Play key.

Note: If you require the MAC address of your Stream in order to configure the MAC filtering of your router, you can find it by pressing the Info (key 0) on your Stream. The Stream will announce it in the Wireless section of the Info.

### Bluetooth

Use this menu to configure your Bluetooth settings. You have the option to connect to, disconnect from or forget Bluetooth devices.

#### Bluetooth

Use this item to toggle Bluetooth on or off. Turn it off to preserve your battery if you don’t intend to use Bluetooth.

#### Pair an audio device

First, make sure the Bluetooth device you want to pair with is turned on. Upon selecting this sub-menu, the Stream will start scanning for available Bluetooth devices nearby. Scroll to the list of available devices using the 4 and 6 keys then press Confirm to pair with it. You will receive confirmation once you are connected.

Once you are paired, your Stream will remember your Bluetooth device and pair with it whenever the device is activated and in close proximity.

Please note: you can access the list of Bluetooth audio devices you had previously paired to your Stream by pressing and holding the 9 key. Cycle through the list with the 4 and 6 keys, then press the Pound key on the device you want to connect. If this device is active and in close proximity, it will connect. If the selected device is already connected, pressing the Pound key on this device will disconnect it. Finally, you can access other options by pressing the 3 key while positioned on one of your previously Bluetooth audio devices configured, such as Connect, Disconnect or Delete.

#### Pair a keyboard

By selecting this option, your Stream will scan your environment and present a list of external Bluetooth keyboards detected. Select your keyboard in the list and Press the pound key. The connection will be established. This external keyboard will now be used to enter any information, when connected.

#### Delete paired device

By selecting this option, you will access a list of your previously paired Bluetooth devices on the Stream. Select the device you want to delete in the list and Press the pound key. The deletion will be processed.

#### Connect device

In this list, you will see all your previously paired Bluetooth devices to the Stream. Press the pound key on one of them, then the Stream will try to connect to this device. If the device is nearby and is working properly, it will connect. If not, an error message will be said.

#### Disconnect device

In this sub-menu, the Stream will list the name of the devices you are currently connected to. Scroll to the name of the device you wish to disconnect from using the 4 and 6 keys then press Confirm to disconnect from it. You will receive confirmation once you are disconnected.

#### Reconnect devices

If some of your Bluetooth devices are not connected properly to your Stream, you can use this option. Press the pound key on this option, then the Bluetooth capability on the Stream will be deactivated, then re-activated, allowing your devices to reconnect properly to the Stream.

## Recording

When considering your recording configurations, note that the built-in and headset microphones operate only in mono mode. Stereo recording is only possible with the external microphone or line-in. The choice of recording file type will affect both the quality of the recording and the size of the recorded file. If you increase the quality of the recording, it will produce a larger recorded file and stereo recorded files are bigger than mono recordings.

### Recording Volume Adjustment

Use this item to adjust the recording volume. You can toggle the recording volume from “Fixed” (the player adjusts the volume automatically) or “Manual” (volume adjusted by user, normally used with an external device).

### Preferred Recording Source

The Stream is able to record from a variety of sources. Use this item to select a Preferred recording source. Options include “Currently active” (whichever source is presently being used), “External microphone”, “Headset” and “Internal microphone”.

### Recording and audio bookmarks default save location

You can choose your preferred recording save location for your newly created notes and audio bookmarks. You can choose between SD card and internal memory. Please note that if you have chosen to save your notes on an SD card, if no SD card is present, your notes will be recorded in the internal memory.

### Built-in Microphone File Type

For the built-in microphone (which records only in mono mode), use keys 4 or 6 to choose to record in one of these formats:

- FLAC

- MP3

- Wav

Upon choosing a recording file type, the Stream will give you an estimated recording time remaining. This time is based on the free space on your SD card and the size of the files generated by the selected recording type.

### Headset Recording File Type

For a headset plugged into the device, use keys 4 or 6 to choose to record in one of these formats:

- FLAC

- MP3

- Wav

Upon choosing a recording file type, the Stream will give you an estimated recording time remaining. This time is based on the free space on your SD card and the size of the files generated by the selected recording type.

### External Recording Source

Use keys 4 and 6 to choose whether you want to plug a microphone or a line-in source to the external jack on the left side of the player. Use line-in when you want to record from an external device such as a CD or cassette player. When you plug an external device, you may monitor the recording by listening with headphones. Note that when using line-in to record from a device, you can adjust the volume of the recording by raising or lowering the volume on the device you are recording from and not on the Stream. Adjusting the recording volume is not possible directly on the Stream.

### External Recording Mode

Use keys 4 or 6 to choose whether you wish to record in stereo or mono. Most microphones, even mono microphones, will work properly with the Stream set to stereo mode. But if your external microphone or line-in recordings can only be heard on the left channel, it is recommended to set the recording mode to mono.

### External Recording File Type

For the external microphone or line-in source, use keys 4 or 6 to choose your recording file type. If your recording mode is stereo, then choose one of these formats:

- FLAC

- MP3

  - 128kilobits per second (kbps) medium quality

  - 192kbps good quality

  - 320kbps high quality

- Wav

If your recording mode is set to mono, then choose one of:

- FLAC

- MP3

  - 32kbps medium quality

  - 64kbps good quality

  - 96kbps high quality

- Wav

Upon choosing a recording file type, the Stream will give you an estimated recording time remaining. This time is based on the free space on your SD card and the size of the files generated by the selected recording type.

## About

In this menu, you will obtain more information about your Stream:

- Stream version

- Model: Stream V3

- Serial number

- Copyright

- Licenses

## Online settings

### Software Update

The Software Update menu contains two items: “Automatic check” and “Check for update”. By default, the Stream will automatically check for any available update if connected to a Wi-Fi network. If you select the item “Check for update”, the Stream will immediately check for any available updates. This item can be used multiple times even if the “Automatic check” item has already been used once. If the “Automatic check” is set to Off, it is necessary to use the “Check for update” item to check for updates as the Stream will not do so automatically.

#### Automatic check

When opening the Software Update menu, this item will be the first. It’s a toggle, that is put at on by default, meaning that the device will check for updates automatically after you boot your device and connect or reconnect your device to a Wi-Fi network.

#### Check for update

When navigating in the Software Update menu with the 6 key, the second item will be “Check for update”. Press the Pound key to check if an update is available. Note that if the option “Automatic check” is put at off, it will be necessary to check for available updates using the “Check for update” function. You can perform a check for update at anytime when you are connected to a Wi-Fi network.

#### Download and install a software update

If an update is available, a prompt will be said indicating that an update is available, the build version and will invite you to press the Pound key to download the update, or the star key to download the update later. Please note that you can use the device while downloading the update.

When the download will be completed, a prompt will be said inviting you to press the Pound key to install the update. Press the Pound key and the device will restart automatically to install the update. Percentages of progression will be announced during the installation process. When the installation will be completed, the device will shut down and you will have to reboot it to use it again.

Please note that if the device is on battery, after receiving the prompt to install the update and pressing the Pound key to confirm, the Stream will shutdown and you will have to restart it manually to install the update.

Please also note: the software update file is visible in the download manager. Please read the “Download manager” section to know more. When you receive a prompt to update your system and you accept this update, the software update file will automatically be downloaded, prioritising itself among any other file, including  the current download. To download the software update file later, it will be necessary to cancel the update.

You can also download software updates from the HumanWare website and copy the file directly to the root of your SD card. After that, insert this SD card to the device and a prompt asking if you want to install the update will be said. Press the Confirm key to install the update or the star key to install the update later. Please note that after the installation process, the file will remain on your SD card until you reboot your device for the first time after the update process. If you want to update more than one device using this method, simply eject the SD card from the device after the shutdown of the device and before rebooting it.

NOTE: It is also possible to copy the downloaded update file in the root of your internal memory. Once the file is copied, use “check for update” to start the installation.

NOTE: It is highly recommended to plug your device during a software update, to be sure to perform it properly. You can also perform a software update on battery, but battery must be at least 50% of charge when beginning the software update installation process. If the battery is too low to update, a message will be said indicating this and you will have to plug the device to perform the software update installation.

### Notification mode

In this submenu, you can select the notification mode used to transmit information about the online services. By default, the option “Beep, message and title” is selected, meaning that if a notification about an online service appears, for example the completion of a book downloading from an online service, a beep will be heard, a message and the book title will be said. You can change this setting to “Beep and message”, “Beep only” or “No notifications”.

### Book Services

Use this menu to access all of the book services available on the device.

#### Bookshare (applicable to some countries)

Use the Bookshare menu to manage the online service. The service can be turned on by going to the Online services menu, Book services, then selecting the Bookshare submenu and selecting the option “Add account”. Press the Confirm key to validate this option. You will be prompted to enter your registered email address and password using the multi-tap text entry method. Passwords are usually case sensitive. End your entry with the Confirm key. It is also possible to use the “Import configuration” item to import your Bookshare account information with a file created using the HumanWare Companion software. Once you have successfully entered your account information, a Bookshare bookshelf will be added to the online bookcase. To remove the Bookshare online service, select the option “Remove account”. Use the item “Modify login information” to modify your account information. Finally, it is possible to select the book format by toggling between DAISY and BRF.

Please note that you can manage your BookShare downloads with the Download manager (please read the “Download manager” section to know more).

#### Daisy Online (applicable to some countries)

Use this menu to manage the Daisy Online service. In the Online services menu, Book services and the DAISY online service submenu, select the item “Add account”, then choose a service to add. You will be prompted to enter your username and password. You can also use the item “Import configuration” to import a custom Daisy Online configuration to your Stream. This custom configuration can be created with the HumanWare Companion software (see section 1.9 about the HumanWare Companion to know more). The software will generate a file that you will be able to import to your Stream using the “Import configuration” option.

For each DAISY online service configured, you can use the item “Modify login information” to modify your account information. You can also select the item “Download Method” to choose the way in which the Stream will download books from this Daisy Online library. You can choose between Automatic download, Manual download (default), and Semi-automatic download. With the Automatic download method, your chosen books or those suggested by the library will automatically be downloaded on your Stream. With the Manual download method, you will be able to use the option “Download more books” in the appropriate Daisy Online bookshelf and manually select which books you want to download and return other books directly without downloading them. The Semi-automatic download method will allow you to download newspapers and magazines automatically, and manually select other content to download. In each DAISY online library added, you can use the item “Remove account” to remove this Daisy Online account and delete all associated books from the Stream.

Please note that you can manage your DAISY Online downloads with the Download manager (please read the “Download manager” section to know more).

#### Eole

Eole is a French library that contains accessible books for disabled people. You can access that resource by using the following URL: https://eole.avh.asso.fr/

More information about EOLE library can be found in the French user guide of this device. Alternatively, this user guide can be downloaded from our website.

#### NFB Newsline (United States citizens or residents)

Use the NFB Newsline menu to manage the online service. To add a NFB Newsline account, go to the Online services, Book services and then select the NFB Newsline submenu. The first item is “Add account”. Press the Confirm key to activate this option. You will then be prompted to enter your NFB Newsline member ID and PIN, each followed by the Confirm key. After you have successfully entered your account information, an NFB Newsline online bookshelf will be added to the online bookcase. If you already had configured a NFB Newsline service account, the option “Add account” is replaced by “Remove account”. If you select this item, the service and all the associated books are removed, and you will be prompted to confirm your choice. All synchronized content will be deleted and the NFB Newsline bookshelf will be removed from the online bookcase. Use the item “Update frequency” to configure how often the Stream will download new content by choosing between “Daily”, “Always” or “Manually”. Use the item “Modify login information” to modify your account information. Finally, the option “Keep update editions” ensure you to have the latest editions of your newspapers and magazines. If you put this option at “On”, most oldest editions will be deleted when necessary. If it is put at “Off” all editions, even the oldest one downloaded, will be stored.

Please note that you can manage your NFB NewsLine downloads with the Download manager (please read the “Download manager” section to know more).

#### NLS BARD (United States citizens or residents)

Use this menu to manage the NLS BARD online service. To turn on this service, go to the Online services menu, Book services and then select the NLS BARD submenu. Select the option “Add account” and press the Confirm key. You will be prompted to enter your registered username and password using the multi-tap text entry method. Passwords are usually case sensitive. End your entry with the Confirm key. You can also use the item “Import configuration” to import your account information with a file created using the HumanWare Companion software. Once you have successfully entered your account information, a NLS BARD bookshelf will be added to the online bookcase. To remove this service, select the option “Remove service and all associated books”. Use the option “Book types to search” to search for Audio books only (default), Audio and Braille, or Braille books only. Use the item “Modify login information” to modify your account information.

Please note that you can manage your NLS BARD downloads with the Download manager (please read the “Download manager” section to know more).

### Other Services

Use this menu to access all of the other services available on the device.

Note: you can manage downloads from all other services with the Download manager (please read the “Download manager” section to know more).

#### Internet Radio

Use this menu to manage the Internet Radio online service. To turn on this service, simply connect your Stream to a wireless network. An Internet Radio bookshelf will be added to the online bookcase. Use the “HumanWare playlist” item to change the region of the playlist that will be offered to you. The default playlist depends on your country or region of purchase (ex: North America). First, enter in the list of playlists and then select a playlist with key 4 or 6, and press Confirm. You can also use the “Import playlist from SD” function to add your own playlists. With the item “Export playlists to SD”, you can export all Internet Radio playlists to your SD card.

#### Podcasts

Use this menu to manage the Podcasts online service. To turn on this service, simply connect your Stream to a wireless network. Use the “Downloaded podcast episodes to keep” item to choose how many podcast episodes should be kept on your Stream (between 1 and 10, default value 3) or whether you want to keep the episodes manually only. Please note that you can customise the number of episodes to keep for each podcast feed you are subscribed. To do so, while positioned on a podcast feed, press the key 3 twice to hear “Downloaded episodes to keep”. Press the Pound key, then navigate with the 4 and 6 keys in the list of available values, from 1 to 10, including the “default”  and the “Manual only” values, then press the Pound key on the desired choice. This selection will apply only to this specific podcast feed. Automatically downloaded episodes will be deleted automatically from internal memory, whereas manually downloaded episodes will have to be deleted manually with key 3. You can also set the podcast update interval, for all your podcast feeds using the Podcasts menu. Your feeds can be updated every 30 minutes, 1 hour, 2 hours, 8 hours and 24 hours. When the podcast update interval is reached and if the device is active at that time, the Stream will connect to the server to check if new episodes are available and download them if it is the case. If the device is shutdown or in suspend mode, the device will check if new episodes are available when the podcast update interval will be reached and when the device will be active again. Press the pound key to select a value. After that, all your podcast feeds will be updated with the interval set. For example, if the podcast update interval is set to 30 minutes, your Stream will check, if the device is active, every 30 minutes if new episodes are available and download them if this is the case. Use the “Import podcast feeds from file” option to import a podcast feed to your Stream with a file created using the HumanWare Companion software. You can use the item “Export all podcast feeds to SD card” to export your subscribed podcast feeds to your SD card. Use the “HumanWare podcasts suggestions” item to change the region of the podcasts that will be offered to you. The default podcasts depend on your country or region of purchase (ex: North America).

#### TuneIn Radio

Use this menu to manage the TuneIn Radio online service. Go to the Online settings menu, then go to Other services and then select the TuneIn Radio submenu. If no account had been previously configured, the option “Add account” will be displayed. Press the Confirm key. You will be prompted to go to TuneIn.com/pair using a web browser (on a phone or computer). Press 6 to obtain the pairing code to enter in the web browser.

Please note that an account is not necessary in order to use TuneIn Radio. For additional details, please refer to section 9.2.7 "TuneIn Radio".

# Bookshelf Structure and Characteristics

For files on all bookshelves, the maximum file name length, including the file path, is 512 characters (includes media/book path). When scanning an SD card at insertion, any folder or file with a full path name greater than 512 will be ignored. Folders and files at the same level are always displayed alphabetically.

The following sections provide general rules and information for listening to the non-Talking Books bookshelves.

## Other Books

### Other Books Structure

The following list provides an example of how book files may be grouped in folders. Folders are numbered in the order in which they are found starting with the root, assuming it contains files.  There can be up to 8 levels of folders.

Folder: \$VROtherBooks.

Rules: Each file in the root is defined as a book with its own reading position and bookmarks. The single book files in the root can be deleted using key 3 while browsing the bookshelf.

Folder: \$VROtherBooks\Drama.

Rules: This is an optional folder to categorize the type of book.

Folder: \$VROtherBooks\Drama\BookA.

Rules: This is the folder containing the book files. The entire folder is defined as the book with one reading position and bookmark set. While browsing the bookshelf, you may use key 3 to delete the book which means the entire book folder will be deleted.

Folder: \$VROtherBooks\Drama\BookB.

Rules: Same as Book A.

Folder: \$VROtherBooks\Mystery.

Rules: This is an optional folder for categorizing type of book.

Folder: \$VROtherBooks\Mystery\BookC.

Rules: Same as Book A.

### Other Books Characteristics

Bookshelf Browse: Move between folder levels with keys 2 and 8. The lowest folder level is also the book level since a book is defined as a collection of files in the lowest level folder. Higher level folders are optional for categorizing types of books as in Drama folder above. There can be up to 8 levels of folders. Use Go To Book to jump to a specific book number. Use key 3 to delete a book. Press key 3 twice to delete a folder. To confirm deletion, press the Confirm key or any other key to cancel.

Reading: Playback loops between the files within a folder (book). Navigate between files in a folder using keys 4 or 6. skip back or forward 10 items while holding the 4 and 6 keys. File and Time Jump navigation levels are available. Go To File and Go To Time are available.

Info (key 0): Reports information for the current book (folder).

Where Am I (key 5): Reports the percentage within the entire book, the current file number, and name as well as the time (total, elapsed and remaining) for the current file.

## Audible books

Your Stream can play Audible books in the Enhanced Audio books format (.aax). To be able to play Audible books, you will need to activate your Stream with Audible using a software app called Audible Sync. You will need to download the Audible Sync from the Audible website. Navigate to the Audible website and type “Audible Sync” in the search field, then select the option for the Audible Sync app from the results and download and install the app.

### Activating your Stream with Audible

After installing the Audible Sync app, choose to sign in then connect your Stream to your computer using the USB cable. Once it is plugged in, wait a few moments for your computer to detect your device as a portable device. Head to the devices area in the Audible Sync app and choose the “Activate Device” option.

Sign in again, and then you can choose to head to your library to download a book. Note that you can download one book at a time. Once it is downloaded, you’ll choose the “Copy to Device” option. Once the copy is complete, you will find the book on the Audible bookshelf of the Stream.

When Audible Sync detects the Stream for the first time, it creates a new reserved folder named $VRAudible in the root of the Stream’s internal memory. When activating your device, it will place a hidden system (.SYS) activation file in that $VRAudible folder which must not be deleted. If you want to read Audible books on multiple devices, you will need to activate each device separately. All Audible books must also be saved in the $VRAudible folder that you will find in the internal memory. Please note that the Stream can also detect Audible books that are present in the $VrAudible books on the SD card.

### Book transfer

To transfer Audible books, you may either use the Audible Sync with the Stream connected, the HumanWare Companion or use Windows Explorer to transfer the .aax Audible book file from the Audible downloads folder on your computer directly to the $VRAudible folder on the internal memory or your SD card. If you use Windows Explorer for the file transfer, you can take advantage of the faster transfer speed of your computer card reader. If you choose to transfer books with Audible Sync, you must always connect the Stream to the computer. Please note that books in internal memory can be move to SD card with the key 3.

### Audible Structure

Simply place your Audible books in the $VRAudible folder. They must not be placed in subfolders.

### Audible Characteristics

Bookshelf Browse: Use keys 4 and 6 to move between each Audible book file or Go To Book to jump to a specified book number. skip back or forward 10 items while holding the 4 and 6 keys. Use key 3 to Delete the Book. To confirm deletion, press the Confirm key or any other key to cancel.

Reading: Use keys 2 and 8 to navigate by the heading marks provided in the Audible book or by selected Time Jumps. Go to Heading and Go To Time are available.

INFO Key 0: Reports info for the current Audible file.

Where Am I: Reports current reading position in the Audible file.

## Music Bookshelf

The music bookshelf is in the reserved folder $VRMusic. It contains a folder structure of audio files and this entire structure is defined as the All Music book. Unless you create playlists, this will be the only Music book. If you add playlist files to the root of $VRMusic then each playlist will be an additional music book. In the Music bookshelf, the Delete Book key 3 is not functional. However, you can delete individual files or entire folders from the All Music book once it has been opened.

### Music Structure

You can navigate up to 8 levels of folders in music. Use the 2 and 8 keys to select the folder or file navigation level. Then, you can move backward or forward at the chosen level with the 4 and 6 keys. skip back or forward 10 items while holding the 4 and 6 keys. If you have more than 8 folder levels, those additional levels will be added to the level 8 list of folders.

If you select the ‘File’ navigation level, you will hear the name of the current file. If you select any folder Level navigation, you will hear the name of the current folder at that level. At any point in your structure, you can always select the deepest navigation level. If the current folder is at a higher level than your deepest level, then the folder name is not announced. For example, let’s say you have a music structure that contains $VRMusic\Classic\Mozart and $VRMusic\Classic\Beethoven\Ninth_symphony. If you are currently located in Classic\Mozart while playback is inactive, changing the navigation from File to Level 1 will announce ‘Classic’. Changing from Level 1 to Level 2 will announce ‘Mozart’. Changing from Level 2 to Level 3 will announce nothing since there is no folder name for a level 3 folder in Classic\Mozart. If located in Classic\Beethoven while playback is inactive, changing from Level 2 to Level 3 will announce ‘Ninth_symphony’.

If you place any files in the root of $VRMusic then they will be listed only at the file navigation level.

### Music Characteristics

Bookshelf Browse: Use keys 4 and 6 to move between each Music book or Go To Book to jump to a specified book number. skip back or forward 10 items while holding the 4 and 6 keys. Only 1 music book (All Music) will be on the music bookshelf unless you create additional playlist books. You cannot use key 3 to Delete a Book.

Reading: If the player is set to Loop Folder, playback loops within the files of a music folder. Playback will automatically start from the beginning of the current folder when the end of the folder is reached. If the player is set to Loop Single, playback will automatically start from the beginning of the current file when the end of the file is reached. You can use keys 2 or 8 to select folder or file navigation. Go To Folder and Go To File functions are also available. If the player is set to Random On, then the files in the current music book will be shuffled. For example, if the All Music book is being played, all of your music files will be shuffled. If a Playlist book is being played, then only the files in that playlist will be shuffled. When the end of a shuffled music book is reached, playback will start at the beginning of the shuffled music book.

Deleting music files: Use key 3 to delete music files when navigating at the file level. You will be asked to confirm file deletion. To delete a folder, navigate back to the folder’s level. Press key 3 to delete the folder and the Confirm key to confirm deletion or any other key to cancel.

INFO (key 0): Reports information for the current music book such as total folders and files and total times for the book.

Where Am I (key 5): Reports current position and information for the current file. Pressing Key 5 twice within 10 seconds will announce tag information data. If you don't want to hear all the tags, press Play to interrupt and resume playback.

Speed: Stream will reset the variable speed control to normal speed and return to the previous speed setting when you exit the Music Bookshelf. Note: It is only possible to listen to stereo music at normal speed. If you speed up or slow down the music, it will play in mono.

Bass and Treble: The Tone control is replaced with separate bass and treble controls.

### Music Search

You can search your music by pressing the Go To key until you hear “Search”. Then, using the multi-tap keypad as described in the Text Search section, enter the word or words to search for and press the Confirm key. Use the Bookmark key to toggle between Text and Numeric input types while entering your text to search. Stream will find the first folder name or file name containing the search term. Playback will start from the beginning of the file or the first file in the folder. Then, you can press key 4 or 6 to find the previous or next occurrence of the search term.

### Playlists

The Music Bookshelf may contain playlists. A playlist is a text file containing a list of supported audio files to play. You can use the Music dialog of the free HumanWare Companion software to help you make playlists. A playlist cannot link to other playlists. Although, you may delete files in the temporary playlists, you cannot use key 3 to delete files in a permanent playlist.

### Custom folder Playlist

You can create a custom folder playlist for a subset of folders found in the $VRMusic folder. Custom folder playlists behave the same way as the All Music book, but only includes files located under the folder that was selected instead of including all files in $VRMusic. You cannot create a custom folder playlist from the files found at the root level of the $VRMusic folder.

- Press key 1 multiple times until you reach the Music bookshelf, or navigate to this bookshelf with the 4 and 6 keys in the list, depending of the setting configured in the bookshelf navigation mode.

- Use keys 4 or 6 to navigate to the “custom folder playlist” item of the Music bookshelf, then press the Confirm key. Now, use keys 4 or 6 to select either ‘SD’ or ‘internal memory’, then use Confirm key to select the media you want.

- Use keys 4 or 6 to navigate to the folder you want in your playlist. If a folder contains other folders, you can enter that folder using the Confirm key, or you can return to the previous folder by pressing the Cancel key. If a folder does not contain other folders, you will not be able to enter it.

- Once you hear the folder name you want for your custom folder playlist, press the Play key to create the playlist.

The folder you selected and all the subfolders below it will now form your custom folder playlist.

Once a custom folder playlist has been created, it will automatically be added to the Music bookshelf, so you will be able to quickly select it again from the bookshelf without having to navigate the folder structure.

When created, a custom folder playlist is named using the complete path for that folder name. You can rename or delete an existing custom folder playlist from the Music bookshelf by moving to the custom folder playlist with 4 or 6 keys, then pressing key 3 until you hear ‘rename’ or ‘delete’, then press Confirm key.

For example, let’s say you have organized your $VRMusic files into 3 levels of folders. You could have the music genre at level 1, artists at level 2 and albums at level 3. You can now easily play only music of a particular genre, or all the albums of a particular artist, or listen to a single album.  Suppose your level 1 genre folders included Classical, Country, Folk, and Rock. Now, suppose, within Classical, you have level 2 folders for the artists, Beethoven, Mozart, and Strauss. You decide you want to play all your Mozart music. Go to the Music bookshelf. Press the 4 or 6 keys until you hear, “Custom folder playlist”. Press key 6 until you hear, “Classical”. Now, press Confirm key to move down to your level 2 folders and press key 6 to reach the Mozart folder. Press the Play key. All your Mozart albums will now be used to create your custom folder playlist and it will begin playing.

### Folder and File Name Announcement

When browsing the Music bookshelf, you may or may not want to hear file and folder name announcements. The Stream will provide this information if the player is in Stop mode but will omit the announcements if the player is in Play mode. In either case, it will repeat the announcements if you press the key 5 (Where Am I).  Thus, if you wish to use key 4 or 6 to navigate back and forth between music files and do not want to hear the folder/file announcements, simply press the Play key first to put the player in play mode.

## Saved Podcasts

The Saved Podcasts bookshelf contains a folder structure of audio files where each folder represents a podcast feed and each file an episode of that feed. Each file (episode) is defined as a separate podcast book with its current position and set of bookmarks. This is where you will find the podcasts you have saved on an SD card from the Podcasts bookshelf in your online bookcase. It can also be used for lengthy recordings like concert or audio description of a movie.

### Saved Podcast Structure

The following list provides an example of how podcast files may be grouped in folders. There can be up to 8 levels of folders.

Folder: $VRPodcasts

May contain files: Yes

Rules: Any files found in the $VRPodcasts root will be announced as individual books.

Folder: \$VRPodcasts\podcasts\Feedname.

May contain audio files: Yes.

Rules: This is the folder for grouping the podcast episode files under their feed name.

### Saved Podcasts Characteristics

Bookshelf Browse: Move between folder levels of the Podcast structure using keys 2 and 8. The lowest level is the book (file) level. Files in the root of $VRPodcasts will only be announced at the book level. Use keys 4 and 6 to move between each Podcast book file (episode) or Go To Book to jump to a specified book number. skip back or forward 10 items while holding the 4 and 6 keys. Use key 3 to Delete a Book file. Press key 3 twice to delete a folder. To confirm deletion, press the Confirm key or any other key to cancel.

Reading: Playback stops at the end of each file. Only Time Jump navigation level is available on keys 2 or 8. Go To Time is also available.

INFO (key 0): Reports information for the current Podcast book (file).

Where Am I (key 5): also Reports current position and information for the current file but adds the percentage of your position in the file and don’t recap the folder information. Pressing Key 5 twice will announce tag information data. If you don't want to hear all the tags, press Play to interrupt and resume playback.

## Text Bookshelf

The Stream can also store text files grouped in folders under the $VRText reserved folder. They are played with the Stream text-to-speech. Each text file within the $VRText folder is considered a book. When present, image description tags will be announced in html and xml documents.

### Text File Structure

The text files can be either in the root of $VRText or organized into sub-folder categories. There can be up to 8 levels of folders.

### Text File Characteristics

Bookshelf Browse: Move between folder levels of the Text Files structure using keys 2 and 8. The lowest level is the book (file) level. Files in the root of $VRText will only be announced at the book level. Use keys 4 and 6 to move between each text file or Go To Book to jump to a specified file number. skip back or forward 10 items while holding the 4 and 6 keys. Use key 3 to Delete a text file. Press key 3 twice to delete a folder. To confirm deletion, press the Confirm key or any other key to cancel.

Reading: Playback stops at the end of each file. The navigation levels on keys 2 or 8 include: page (if page markers are in the file), screen (if no page markers), paragraph, line, sentence, word, spell, and character. If you do a search in text, then a Search level is added as the last navigation level to allow you to find the next or previous search item. Go To Page is available if the file has page markers. Go To Percent is also available.

INFO (key 0): Reports information for the current text file such as its size.

Where Am I (key 5): Reports current position as a percentage from the start of the file.

### Navigating HTML/XML/DOCX Headings

For HTML/XML files that have heading tags (<H1> to <H6>), or DOCX files with stylized headings, these will correspond to equivalent navigation levels when navigating with 2 or 8. Be aware that some HTML/XML files do not use these heading tags consistently. For example, the file may have an H1 and H3 heading with no H2 heading. In this case, the Stream will present only level 1 and level 3 when navigating with 2 or 8. Also, an html/xml file could have a single H1 heading for a title with no other H1 tags in the file. In this case, Stream would jump to the end of the file if you pressed right arrow (key 6) after choosing level 1 navigation.

## Notes

To record a new note, press the Record key, then record your note and press on the Record key again to finish your recording and create a note. You can also pause your recording while you are creating your note by pressing the Play/stop key during your recording, then pressing the Play/Stop key again to resume it. Finally, it is possible to record quick notes by pressing and holding the Record button. The note will be created when you will release the Record button.

The audio files (notes) are in the reserved folder $VRNotes. There could be one or two $VRNotes folder in your device, as one $VRNotes folder is created by default when plugging your SD card in your device if none had been created before, and another $VRNotes folder will be added in the internal memory if you wish to save audio notes in your device internally. As a result, two Notes bookshelves can be present, one in the SD card and another in the internal memory. These two Notes bookshelf are completely independent from each other. When your Stream is placed on the Notes bookshelf, a Generic folder will be announced, regrouping your notes saved at the root of the $VRNotes folder. This folder is seen only by the Stream. As a result, if you plug your Stream to a computer and explore the content in the device, all the notes in the Generic folder will be seen directly at the root of the $VRNotes folder. This folder can also contain subfolders, where you will find notes files. This entire structure is defined as the All Notes book. In the Notes bookshelf, while using the 3 key, you will be able to copy, move or delete the notes in the Generic folder, as well as renaming and deleting any subfolders (categories), but it is not possible to rename or delete the Generic folder, and the Notes bookshelf cannot be deleted even though it is empty. You can also set a custom name for a specific note.

Finally, you can export or import all the notes from a notes folder or all the content of your Notes bookshelf from the internal memory to your SD card and vice-versa. During the import/export progression, your device will inform you regularly. When the operation will be completed, you will be informed. Please note that in order to do it properly, folders and note names/numbers should be different to avoid any conflict.

### Note File Structure

By default, the note files are organized in numerical order in the root of $VRNotes, in the Generic folder. If a note has a custom name, the number of this note, followed by its custom name, will be read. You can create subfolders (categories) in which you will be able to create/move notes. If you create subfolders, when navigating, they will be displayed alphabetically. Subfolders can be created/renamed/deleted. The Generic folder is the only folder present by default, is at the root of the $VRNotes folder and cannot be deleted.

Notes will be recorded by default in the SD card unless you are positioned in the Notes bookshelf in the internal memory, if no SD card is inserted in the device or if you had changed your preferred recording and audio bookmarks default save location. If Notes folders were created in the SD card and in the internal memory, when rotating with the 1 (bookshelf) key in the offline bookshelves or when navigating through your bookshelves list with the 4 and 6 keys, depending of the setting configured in the bookshelf navigation mode, the Notes bookshelf will be identified: Notes (SD Card) and Notes (internal memory), with their specific folder, subfolders if any and files.

### Note File Characteristics

Bookshelf Browse: Move between the Generic folder and subfolders with 4 and 6, then press the Pound key on the generic folder or the desired subfolder. The generic folder and your subfolders are displayed at the same level. In the generic folder or in the subfolder of your choice, use keys 4 and 6 to navigate between note files. Skip back or forward 10 items while holding the 4 and 6 keys. By pressing key 3 rotatively, you can Delete a file, move note, copy note, set a custom note name and delete note. When on the Generic folder or any subfolder, pressing key 3 rotatively will give you the following options: delete category and notes, delete notes in category, delete category and move notes, copy note category to SD card (or to the internal memory if on the SD card), consolidate notes, rename category and copy all notes to the SD (or the internal memory if on the Notes folder on the SD card). Consolidating means we rename in an incremental order the files from 1 to the total number of notes. Consolidating notes impact all the notes present on the device for this specific media (SD card or internal memory). To confirm any action that you will find while rotating with the 3 key, press the Pound key. You can press any other key to cancel.

Reading: Playback stops at the end of each file. Bookmarks, Time Jump navigation levels and Notes are available on keys 2 or 8. Go To Time and Go to Note are also available.

INFO (key 0): Reports information for the current note and report the remaining recording time for this media (internal memory or SD card).

Where Am I (key 5): Reports current position and information for the current file. Pressing Key 5 twice will announce tag information data. If you don't want to hear all the tags, press Play to interrupt and resume playback.

Set a custom name for a specific note: while rotating with the 3 key in the options for a specific note, you can set a custom name for this note. Press the Pound key on this option and it will be possible, with the keyboard, to give a custom name for this note. The file will be name: notenumber-customname.extension. If you Press the Pound key without entering any character, the name for this note will not change and will remain numeric only, as it was before this action. Also, if you enter an invalid character, you will be notified after writing it and the character will not be entered. No special character is allowed. Finally, a custom name for a note cannot be more than 50 characters.

# Daisy Online

## Daisy Online configuration menu

To manage the Daisy Online service, go to the menu and select the option Online services, then the option Book services and the item DAISY online. Select the item “Add account” to choose a service to add. You will be prompted to a list of libraries that you can add, select one of them. You will have to enter your username and password. You can also use the item “Import configuration” to import a custom Daisy Online configuration to your Stream. By navigating with 4 or 6, you will find a list of libraries that are already added on your device. If you select one of these libraries, you will have the opportunity to change the download method, to modify login information and to remove account. The item “Download method” offers three ways to download your books from this library: manual download (by default), semi-automatic (if available) and automatic download. With the Automatic download method, your chosen books or those suggested by the library will automatically be downloaded on your Stream. With the Manual download method, you will be able to use the option “Download more books” in the Daisy Online bookshelf and manually select which books you want to download and return other books directly without downloading them. The Semi-automatic download method will allow you to download newspapers and magazines automatically, and manually select other content to download. The item “Modify login information” permit to change the username and password used to connect to this library. Finally, use the item “Remove account” to remove this specific library and delete all associated books from the Stream.

Please note that you can manage your DAISY Online downloads with the Download manager (please read the “Download manager” section to know more).

## Using Daisy Online Service

When the Stream is connected to a wireless network, you will be able to access Daisy Online, an online service used to access and download books from compatible Daisy Online libraries.

To activate the Daisy Online service:

- Press menu key 7 to access the Configuration menu.

- Use the 4 and 6 keys to access the item Online Services and press Confirm.

- Use the 4 and 6 keys to access the item Book Services and press Confirm.

- Use the 4 and 6 keys to access the Daisy Online menu and press Confirm.

- Select the item “Add account” using keys 4 and 6 followed by Confirm.

- Using keys 4 and 6, select a service to add followed by Confirm.

- Enter your Daisy Online username and password.

- Alternatively, the HumanWare Companion software can be used to create a file containing your Daisy Online account details, which can be imported from the “Import configuration” item on the DAISY Online service menu on the Stream.

- Once you have configured a Daisy Online account and added it from the online configuration menu, a Daisy Online bookshelf with the DAISY Online service name will be added to your online bookcase. Subsequently and for each configured account, a new Daisy Online bookshelf will be added to your online bookcase. If there is more than one configured account for the same DAISY Online server, the account username will be appended to the service name when announcing the bookshelf to help you differentiate your accounts.

- In your Daisy Online bookshelves, you will be able to find the books downloaded from your account. When a book’s expiration date is known, it will be announced after the book title. When a book is expired and needs to be returned, it will be announced before its title. Pressing the Where Am I key will give you a book description when available, whether a book is downloaded or not. From a book, you can also press the Where Am I key to know its expiration date.

## Download Methods

If you selected Manual download method in the Daisy Online menu, you will have an option on the bookshelf named “Download more books”. Use this option to browse all the books issued to your account that are available for download. You can choose to download these books with the Confirm key, or return them directly without downloading them using the “return” option on key 3. You can also return a book while offline; the return will be completed once you are connected to a wireless network.

If you selected Automatic download method in the Daisy Online menu, the Stream will periodically download new books issued to your account on your Daisy Online bookshelf. However, if you want the Stream to immediately download any available books, you can use the option on the bookshelf named “Check for New Content”. Once the player’s memory is full, new ones will be downloaded as books are returned.

If you selected Semi-automatic download method in the Daisy Online menu, you will be able to download Newspapers and Magazines automatically, and manually select other content to download, such as Books, and Other.

## Navigating in your DAISY online bookshelves

- Each Daisy Online service offers a menu that may differ from one service to another. For example, some services allow the ability to copy books using key 3, while others do not. Different download methods can be set as default, while different return policies may apply for expired books.

- Two search types are possible: Single input and Multiple inputs. Generally, DAISY Online Service supports either one of these search types. Most services use the Single input search type, while only a few use the Multiple inputs search type.

- With the Single input search type, you will be prompted to enter your text using the multi-tap text entry method. Use the Bookmark key to toggle between Text and Numeric input types while entering your text to search. You can edit a previous search by entering new search terms which will be added after your previous search criteria. Press Confirm to start the search and display search results list, or press the Star key to cancel the search. You will return to your DAISY Online bookshelf.

- With the Multiple inputs search type, you can enter multiple criteria to search, such as the title, author, etc. Each available criteria will have a distinct input field, and you can enter multiple criteria to search for at the same time. Press the 4 or 6 keys to navigate between criteria, then Confirm to edit one of them. Use the Bookmark key to toggle between Text and Numeric input types while entering your text to search. Press Play or press Confirm on the confirm prompt after the last criteria to start the search and display search results list, or press the Star key to cancel the search. You will return to your DAISY Online bookshelf.

- In addition, depending on the Daisy Online service you are using, various functions will be available to you from key 3, notably the options “Return book”, “Copy current book from internal memory to SD card” and “Redownload” but it depends on what the Daisy Online service offers.

- The Where Am I key will announce the book description when available.

- The Go To key will allow you to jump to the book index.

- While downloading a Daisy Online book, an “Added to download queue” message will be heard. Press and hold the Confirm key to hear a download status report.

- In addition to canceling searches, the Star key allows you to go back up in the menu and to leave a menu in the DAISY Online bookshelf.

# Other Stream Wireless Features

When you press the Online button located in the center of the top key row, the Stream will switch between the offline and online bookcases. The airplane mode turns off all wireless communication from the Stream and should be activated when you travel by airplane. Turning off the airplane mode will therefore allow wireless communication. Press and hold the Online button to turn the airplane mode on or off. If you are not using the online bookcase, it is best to leave the airplane mode turned on as it will reduce battery consumption. Press the Online button to toggle from the online bookcase to the offline bookcase.

## Online Check for Updates

When the Stream is connected to a wireless network and has Internet access, it will access the HumanWare website. The Stream will check for any available software updates and will prompt you to download and install them automatically. This automatic check for updates can be disabled from the Software Updates configuration menu, that you can find in the Online settings. You can also choose to check for updates manually from the same menu, and the Stream will immediately check for any available updates. See chapter 10 (Updating Stream Software) to learn how to update your Stream wirelessly.

## Online Services

### NFB Newsline (Eligible United States citizens and residents only)

In the United States, the National Federation of the Blind (NFB) provides a service that allows you to receive accessible newspapers and magazines in DAISY format that is called NFB Newsline. For more information, visit:

http://www.nfbnewslineonline.org

To begin receiving NFB Newsline publications on your Stream, you will need to set up your Favorites list for the content that you want to have delivered to your device. If you have not yet set up a Favorites list, you will need to login to NFB NEWSLINE ONLINE at

http://www.nfbnewslineonline.org

and select the link to MANAGE ALL YOUR FAVORITES. Select MANAGE YOUR FAVORITE IN YOUR POCKET PUBLICATIONS and add your desired newspapers and magazines. There is no limit as to the number of IN YOUR POCKET publications you may have.

To activate the NFB Newsline service:

- Make sure your Stream is connected to a known wireless network.

- Press menu key 7 to access the Configuration menu.

- Use the 4 and 6 keys to access the item Online Services and press Confirm.

- Use the 4 and 6 keys to access the item Book Services and press Confirm.

- Use the 4 and 6 keys to access the NFB Newsline menu and press Confirm.

- To add the NFB Newsline service, select the item “Add account” and press Confirm.

- You will then be prompted to enter your NFB Newsline member ID and PIN each followed by the Confirm key.

- After you have successfully entered your account information, an NFB Newsline online bookshelf will be added to the online bookcase.

Whenever an active Wi-Fi connection is available, the Stream will now automatically synchronize the latest editions of your selected favorite publications and will automatically remove older editions. The NFB Newsline service determines which editions of each publication are available for synchronization. It is possible to check for any updated content on demand at any time by selecting the “Synchronize NFB Newsline Content” item which can be found after the last publication on the NFB Newsline bookshelf when navigating with keys 4 and 6, or by pressing the Go To key twice. From your NFB Newsline bookshelf, you can add a new NFB Newsline publication by pressing the Go To key and selecting the item “Add Publications”. Navigate through the list of publications with keys 4 and 6, and press the Confirm key to subscribe to the new publication.

If you wish to save a copy of a publication, it is possible to use the copy function of key 3 when reading the publication to copy it to the Talking Books bookshelf of the SD card. It is not possible to delete publications from the NFB Newsline online bookshelf as the automatic synchronization will remove old editions and add new ones as they become available. You can choose to make a NFB book persistent by using the “Prevent auto-deletion of an NFB Newsline issue” option on key 3. To unsubscribe from a publication, use the “Unsubscribe from an NFB publication” option on key 3. A confirmation prompt will ask you to confirm before unsubscribing from your publication, press the Confirm key to confirm or any other key to cancel. Note that if you want to delete all associated books related to an unsubscribed publication, use the option “Sync content now”. To delete persistent books, it will be necessary to delete them individually with the key 3.

From the NFB Newsline configuration menu, you have the option of choosing how often the Stream downloads new content with the item “Keep update editions”. You can choose to have new content downloaded “Always” (default), which can be up to several times a day, or “daily”.

Please note that you can manage your NFB NewsLine downloads with the Download manager (please read the “Download manager” section to know more).

### NLS BARD (Eligible United States citizens and residents only)

The National Library Service Braille and Audio Reading Download (NLS BARD) is a free library program of audio materials (for eligible United States citizens only). More information about NLS BARD can be found on http://www.loc.gov/nls/ .

You are able to wirelessly search for books and download them to the Stream. They will appear on the NLS BARD online bookshelf.

To activate the NLS BARD online service:

- Press menu key 7 to access the Configuration menu.

- Use the 4 and 6 keys to access the item Online Services and press Confirm.

- Use the 4 and 6 keys to access the item Book Services and press Confirm.

- Use the 4 and 6 keys to access the NLS BARD menu and press Confirm.

- Select the item “Enter credentials” using keys 4 and 6 followed by the Confirm key.

- Enter your NLS BARD account email address and password. Passwords are usually case sensitive. End your entry with the Confirm key.

Alternatively, the HumanWare Companion software can be used to create a file containing your NLS BARD account details that can be imported from the “Import configuration” option on the NLS BARD menu on the Stream. Details can be found in the HumanWare Companion User Guide.

After you have successfully entered your account information, an NLS BARD bookshelf will be added to the online bookshelves.

To search for and download books:

- When on the NLS BARD online bookshelf, the search can be done either by pressing the Go To key multiple times or by using the option that is found after the last book on the NLS BARD bookshelf when navigating with keys 4 and 6. You can search books as on the NLS BARD website using the “Search collection” function, which allows you to enter the author’s name, book title, key words, etc. You can also browse by category and search for the most recent or popular books, as well as the most recent magazines. You can also use the option “Wish list”, a preselected list of books and magazines that you can manage from the NLS BARD website and from your Stream. To add an item to the wish list, press the Bookmark key on any NLS BARD search result. To delete an item, use key 3 (Delete option) followed by Confirm.

- Use keys 4 and 6 to choose the search criteria followed by the Confirm key. The search text can then be entered using the multi-tap text entry method. After entering the search text, press the Confirm key to begin the online search. If you have done a previous search, then your search text will be retained for convenience in case you wish to refine your search.

- Use keys 4 and 6 to review the search results. Only the books available to your account will show up in the search results, with their titles, authors, ID numbers, etc.

- Use key 5 to read the book synopsis if available.

- To download a book, select it from the list of search results and press the Confirm key.  The book will be downloaded and added to the NLS BARD online bookshelf and you will be returned to the list of search results making it easy to download additional books.

- The Go To key can be used to go directly to a specific result. Press Go To, enter the desired search result number, followed by Confirm.

- To exit the search, press the Cancel key to go back one step or key 1 to return to the NLS BARD online bookshelf.

Downloaded books can be deleted by pressing key 3 while browsing the NLS BARD online bookshelf or while reading a book. While reading a book, key 3 can also be used to move a book from the NLS BARD online bookshelf to the talking books bookshelf on the SD card. This will add the book to your SD card and remove it from internal memory, freeing up some space for future book downloads.

When multiple items are selected for download, they will be placed in a download queue, allowing you to continue using your Stream. A notification will be sounded when a download is completed.

Please note that you can manage your NLS BARD downloads with the Download manager (please read the “Download manager” section to know more).

### Bookshare (eligible users only)

Bookshare is an online library of copyrighted content for people with qualifying print disabilities. More information about Bookshare is available from http://www.bookshare.org

You can wirelessly search for books and download them to the Stream. Books are downloaded in DAISY text or audio format and appear on the Bookshare online bookshelf. Newspapers and magazines are currently not available in the online search.

To activate the Bookshare online service:

- Press menu key 7 to access the Configuration menu.

- Use the 4 and 6 keys to access the item Online Services and press Confirm.

- Use the 4 and 6 keys to access the item Book Services and press Confirm.

- Use the 4 and 6 keys to access the Bookshare configuration menu and press Confirm.

- Select the item “Add account” using keys 4 and 6 followed by the Confirm key.

- Enter your Bookshare account email address and password. Passwords are usually case sensitive. You may toggle between uppercase, lowercase, numeric only by pressing the Bookmark key. End your entry with the Confirm key.

Alternatively, the HumanWare Companion software can be used to create a file containing your Bookshare account details that can be imported from the online configuration menu on the Stream. Details can be found in the HumanWare Companion User Guide.

After you have successfully entered your account information, a Bookshare bookshelf will be added to the online bookcase.

To search for and download books:

- When on the Bookshare online bookshelf, the search can be done either by pressing the Go To key twice located above bookshelf key 1 or by using the option called “Search for books” that is found after the last book on the Bookshare bookshelf when navigating with keys 4 and 6. You can search books by most popular, most recent, browse categories, previous downloads, title, author, full text search and reading lists.

- The Go To key can also be used to go directly to a specific result. Press Go To, enter the desired search result number, followed by Confirm. When on the reading lists, the Go To key can be used to search for a specific one.

- Use keys 4 and 6 to choose the search criteria followed by the Confirm key. The search text can then be entered using the multi-tap text entry method. Use the Bookmark key to toggle between Text and Numeric input types while entering your text to search. After entering the search text, press the Confirm key to begin the online search. If you have done a previous search, then your search text will be retained for convenience in case you wish to refine your search.

- Use keys 4 and 6 to review the search results. The book synopsis is available when reviewing the list of search results by pressing the “Where am I” key 5. Only the books available to your account will show up in the search results.

- To download a book, select it from the list of search results and press the Confirm key.  The book will be downloaded and added to the Bookshare online bookshelf and you will be returned to the list of search results making it easy to download additional books.

- To exit the search, press the Cancel key to go back one step or key 1 to return to the Bookshare online bookshelf.

Note: on the Bookshare’s website, you can create reading lists to filter your preferred books and organise them by genre or custom categories. If you are a student or if your organisation would like you to read specific books, they can also assign you those books that will be available for you afterwards. If you want to delete a book from a reading list, when in the reading lists, select the list in which you wish to access. In that list, you can delete a book by using the Bookmark key when you will be positioned on that book. It is also possible to add a book to a reading list. When positioned on that book, use the Bookmark key. A prompt will be displayed, asking you in which reading list you want to add this book. Circle in the reading lists with the 4 and 6 keys then press the Pound key on the desired one. The book will be added to this reading list. Finally, public reading lists have also been created by the Bookshare team, organising popular books by genre or categories. You can subscribe to them via the Bookshare website. However, it is impossible to add or remove books from a reading list created by the Bookshare team or created by an organisation or a school.

Downloaded books can be deleted by pressing key 3 while browsing the Bookshare online bookshelf with keys 4 and 6 or while reading a book.

While reading a downloaded Bookshare book, key 3 can also be used to move a book from the Bookshare online bookshelf to the talking books bookshelf on the SD card. This will add the book to your SD card and remove it from internal memory, freeing up some space for future book downloads.

Please note that you can manage your Bookshare downloads with the Download manager (please read the “Download manager” section to know more).

### Internet Radio

The Stream supports ooTunes, an Internet Radio service allowing you to search for radio stations and add them to your Favorites playlist.

Please note that this app can be hidden if you don’t intend to use it. To do so, go to the Configuration menu (key 7), then, under General settings, navigate with the 4 and 6 keys until attaining the App toggle menu and press the pound key on this option. Then, navigate with the 4 and 6 keys until attaining the Internet radio app, and press the pound key on this option. The Internet radio app will now be hidden and will not be available while cycling in your online bookshelves. All the data associated to this app will also be lost.

When the Stream is connected to a wireless network, you will be able to access a collection of playlists containing Internet Radio stations. After you have saved a radio station with the Bookmark key, the Favorites playlist, where all your bookmarked stations are saved, will appear on the Internet Radio bookshelf. The HumanWare playlist always appears on the Internet Radio bookshelf and offers a sample list of stations. The selected HumanWare playlist is based on your country or region of purchase and can be changed in the online settings menu. The Internet Radio Recordings playlist contains all your radio recordings. This playlist is only available when there are radio recordings on your device.

The Internet Radio bookshelf also allows you to import new or custom playlists created with the HumanWare Companion software. You can import these playlists in the online settings menu. Circle through the Online settings menu until you reach the option “Other services” and press the Confirm key. After that, use the 4 and 6 keys to go to the item “Internet radio” and finally, use the 4 and 6 keys to go to the option “Import playlist from SD” and press the Confirm key to confirm the operation. You can also import a playlist from SD by selecting the option “Import playlist from SD”, that you will find on the Internet radio configuration menu that you can obtain, while in Internet radio bookshelf, by pressing the 7 key.

You can later delete these playlists with key 3, followed by Confirm.

Note: Make sure to import playlists named differently from one another to avoid overwriting an existing playlist on your Stream.

To export a playlist to your SD card, when in Internet radio bookshelf, press the 7 key to go to the Configuration menu. By navigating with 4 and 6 on this menu, you will hear “Export playlist to SD”. Press Confirm to perform this operation.

From the Internet Radio online bookshelf, you can search in your playlists by going through your playlists with the 4 and 6 keys then select one of them with the Confirm key. Radio stations related to the selected playlist will be displayed and you will be able to go through them with the 4 and 6 keys then press the Play/Stop key to begin playing the selected Internet radio.

To search for Internet Radio stations:

- You can search by Ootunes. To do so, select the option “Search on Ootunes”, that you can find after the last displayed playlist or by pressing the Goto key multiple times. Then, press the Confirm key to begin search.

- Use the 4 and 6 keys to select your preferred search method (Search or Browse), follow by the Confirm key.

- If you select the “Search” option, use the multi-tap text-entry method to enter text search, then press the Confirm key to perform the search.

- If you select the Browse option, you can browse by categories, for example, adult contemporary, classic rock, country, etc. Press the Confirm key to select one of those categories, then use the 4 and 6 keys to navigate in the results list.

To listen to Internet Radio stations:

- From the Internet Radio bookshelf, use the keys 4 and 6 to navigate between playlists or select the option “Search on Ootunes”, then press the Confirm key to select a playlist or the search option. If you have selected the “Search on Ootunes” option, press the 4 and 6 keys to search by categories or press the “Search” item to enter search keywords, then press the Confirm key to be directed to the results list. If you have selected the HumanWare playlist or any other playlist, you are headed directly to the results list. Navigate through the radio stations using keys 4 and 6. The number of the current station will be announced, followed by its name. Skip back or forward 10 items while holding the 4 and 6 keys. If more than 100 results are present, use the Next page button, that you will find at the end of the list, to go to the next results.

- The Go To key can be used to go directly to a specific station. Press the Go to key until you hear “Go to station”. Enter the desired station number, followed by Confirm to navigate to the station.

- Press Play/Stop to start listening to the station.

- Press Play/Stop again to stop listening to the station.

- If you use the 4 or 6 keys while playing a radio station, the previous or next station will start playing immediately.

- You may press the Where Am I key 5 to announce additional information about the station. The first press will indicate the song title and the station name if available and if you press again on the 5 key within 10 seconds, you will obtain additional information on the current station.

- The Bookmark key will allow you to save the current station to your Favorites after pressing Confirm.

- When on a playlist, except the HumanWare playlist, you can remove the current station from the playlist by pressing key 3. Press Confirm to remove it or press the Star key to cancel deletion.

To record internet radio stations:

- While listening to a radio station, press the Record button to start recording the radio station.

- While recording, press Play/Stop to pause your recording. Press Play/Stop again to resume recording.

- Press Record again to stop recording the radio station.

Note: You can add bookmarks and save your position while recording a radio station simply by pressing the Bookmark button while recording.

Note: Internet radio recording parameters are the same as the External Recording parameters.

To access, delete, and transfer internet radio recordings:

- From the Internet Radio bookshelf, press the 4 and 6 keys until you reach the Internet Radio Recordings playlist.

- Press Confirm to enter the Internet Radio Recordings playlist.

- Press the 4 and 6 keys to scroll through your radio recordings. Alternatively, you can select a specific recording by pressing the Go To key followed by the number of the recording, then confirm by pressing the Confirm key.

- Press Play/Stop to start listening to your selected recording.

- During playback, you can use the bookmark function in the same way as you would with any audio track (refer to Chapter 5.3 for detailed instructions on using bookmarks).

- You can delete your selected recording by pressing the 3 key followed by the Confirm key. Press the Confirm key again to confirm deletion.

- To move an internet radio recording to an SD card, select the recording you wish to transfer, then press the 3 key until your hear “Move Recording to SD card”.

- Press the Confirm key to move your selected recording to your SD card, then press the Confirm key again to confirm the transfer.

Notes: Because the Stream supports specific formats, search results may be different than with other devices (ex: iPhone). In addition, certain radio stations may not be accessible while travelling in a region that does not support them, although they are displayed in the search results. Note that you will be able to access these radio stations once you return to the region that supports them.

### References (Wikipedia and Wiktionary)

When the Stream is connected to a wireless network, you will be able to search for references on Wikipedia and Wiktionary. A References bookshelf will automatically be added to your online bookcase.

Please note that this app can be hidden if you don’t intend to use it. To do so, go to the Configuration menu (key 7), then, under General settings, navigate with the 4 and 6 keys until attaining the App toggle menu and press the pound key on this option. Then, navigate with the 4 and 6 keys until attaining the References app, and press the pound key on this option. The References app will now be hidden and will not be available while cycling in your online bookshelves. All the data associated to this app will also be lost.

To search for and add references:

- From the References bookshelf, the search can be done either by pressing the Go To key multiple times or by using the option that is found after the last book on the References bookshelf when navigating with keys 4 and 6. There are two options: “Search on Wikipedia” and “Search on Wiktionary”.

- From a book, you can also search for references on Wikipedia or Wiktionary by pressing the Go To key multiple times on a selected word until you hear “Search on Wikipedia” or “Search on Wiktionary”. For text books, the last word that was read will be entered in the search. You can also enter a word to search using the Multi-tap text entry method.

- If Wikipedia or Wiktionary articles contain the word you searched for, you will hear the number of definitions that were found as well as a short synopsis of the first definition found. You can scroll through the different definitions by pressing the 4 and 6 keys. You will then be asked if you want to read the complete article. Press Play/Stop to access the complete article or Cancel to return to your book.

- While reading Wikipedia or Wiktionary articles, press Cancel to return to your book.

- While reading a complete article, you can save it by pressing the Manage Books key (key 3), followed by Confirm.

- Saved articles will be prefixed by “Wikipedia” and Wiktionary articles by “Wiktionary”, and grouped together by service (Wikipedia or Wiktionary) then by language (EN for English, FR for French, and so on). Use the 4 and 6 keys to navigate in your saved articles. You can also use keys 2 and 8 to navigate by level (service at level 1 and language at level 2).

- To go directly to a specific file, use the “Go to File” option on the Go To key.

- To delete an article from the References bookshelf, use the Delete function of the Manage Books key (key 3), followed by Confirm.

- From the References bookshelf, you can also move a Wikipedia or Wiktionary article on your SD card with the Move function of the Manage Books key (key 3), followed by Confirm.

Note: The TTS language will determine the language of the Wikipedia database that will be used for the search (ex: en.wikipedia.com, fr.wikipedia.com, etc.).

### Podcasts

When the Stream is connected to a wireless network, you will be able to download new podcast feeds. Please note that you can access the Podcasts service if you are connected or not to a wireless network. If connected to a wireless network, the Stream will automatically download the most recent episodes from every subscribed podcast feed. The podcast episodes that have been downloaded will be available even when the Stream is not connected to a wireless network.

Please note that this app can be hidden if you don’t intend to use it. To do so, go to the Configuration menu (key 7), then, under General settings, navigate with the 4 and 6 keys until attaining the App toggle menu and press the pound key on this option. Then, navigate with the 4 and 6 keys until attaining the Podcasts app, and press the pound key on this option. The Podcasts app will now be hidden and will not be available while cycling in your online bookshelves. All the data associated to this app will also be lost.

Searching for downloads and listening to online podcasts:

- A Podcasts bookshelf will automatically be added to your online bookcase.

- From your Podcasts bookshelf, you can add podcast feeds either by pressing the Go To key multiple times or by using the option called “Add podcast feed” that is found after the last podcast feed when navigating with keys 4 and 6. Press the Confirm key. You can now perform a title search, search by category or browse the HumanWare podcast suggestions based on your region and country. If you perform a search by title, you will be prompted to the multi-tap entry keyboard. Use the Bookmark key to toggle between Text and Numeric input types while entering your text to search. Use keys 4 and 6 to browse the search results, followed by Confirm to add a feed to your Podcasts bookshelf.

- Use key 5 to read the feed descriptions and the individual episode descriptions if available.

- You can also browse the HumanWare suggested podcasts to search for podcasts.

- In addition, you can use the “Get more episodes” option at the end of the list of downloaded episodes or on the Go To key rotation to download older episodes on demand.

- The Go To key can also be used to go directly to a specific result. Press Go To, enter the desired search result number, followed by Confirm.

- To listen to a podcast, open a feed and select an episode you want to listen to with keys 4 or 6, followed by Confirm. The Stream will inform you if a podcast feed contains any new episodes.

- To copy an episode to your Saved Podcasts folder on the SD card, press key 3 twice from your list of episodes or when playing an episode.

- To export your subscribed podcast feeds to the SD card, use the item “Export all podcast feeds to SD card” from the Podcasts configuration menu. A file ending with .OPML will be created on your SD card containing all of your subscribed podcast feeds. This file can be used as a backup or for importing the list of podcast feeds in another podcast client.

Once subscribed to a feed, the Stream will automatically start downloading the most recent episodes in the background. The Stream will tag these episodes as “new” as they have never been played before. From the configuration menu, you can use the “Downloaded podcast episodes to keep” item to choose how many podcast episodes should be kept on your Stream (between 1 and 10, the default value being 3 episodes) or whether you want to keep the episodes manually only. You can set the downloaded podcast episodes to keep for a specific podcast feed. To do so, while positioned on a podcast feed, press the key 3 twice to hear “Downloaded episodes to keep”. Press the Pound key, then navigate with the 4 and 6 keys in the list of available values, from 1 to 10, including the “default”  and the “Manual only” values, then press the Pound key on the desired choice.

You can also set the podcast update interval, for all your podcast feeds using the Podcasts menu. Your feeds can be updated every 30 minutes, 1 hour, 2 hours, 8 hours and 24 hours. When the podcast update interval is reached and if the device is active at that time, the Stream will connect to the server to check if new episodes are available and download them if it is the case. If the device is shutdown or in suspend mode, the device will check if new episodes are available when the podcast update interval will be reached and when the device will be active again. Press the pound key to select a value. After that, all your podcast feeds will be updated with the interval set. For example, if the podcast update interval is set to 30 minutes, your Stream will check, if the device is active, every 30 minutes if new episodes are available and download them if this is the case.

Note: Please note that if you have some new episodes and if you set your downloaded podcast episodes to manual only (in the Podcasts menu for all your podcasts feed or using the 3 key for a particular feed), they will be deleted from your Stream as they have not yet been saved, and no new episodes will be automatically downloaded.

When a podcast episode is downloaded, it is saved in the Stream’s internal memory. If the episode was downloaded automatically, it will also be erased automatically when enough new episodes become available, depending on the number of downloaded podcast episodes to keep you have set in the configuration menu or for a specific podcast feed. You can choose to make an automatically downloaded episode persistent using the “Prevent auto-delete of episode” option on key 3. If an episode was downloaded manually, it will have to be erased manually using the key 3 “Delete” function.

When multiple podcasts are selected for download, they will be placed in a download queue in the background, allowing you to continue using your Stream. A notification will be played when a podcast download is completed. To unsubscribe from a podcast feed, use the option “Unsubscribe from podcast feed” on key 3.

To delete all episodes from a podcast feed:

- From the podcast bookshelf, press on the 4 and 6 keys to select the feed whose episodes you wish to delete.

- Press key 3 until you reach the option to delete all podcasts from the podcast feed.

- Press the Confirm key to select the option.

- Press the Confirm key again to delete all episodes.

To move all episodes of a podcast feed to an SD card:

- From the podcast bookshelf, press on the 4 and 6 keys to select the feed whose episodes you wish to move to an SD card.

- Once the podcast feed is selected, press on the key 3 until you receive the option to move all podcasts from the podcast feed to the SD card.

- Press the Confirm key to select the option.

- Press the Confirm key again to move all podcasts feeds.

Note: You can only delete or move all episodes of a podcast feed when you have podcast episodes on your device.

Note: when on the feeds list, it is also possible to move all podcast episodes (from all your feeds) to your SD card. Cycling with the 3 key, you will find this option, press the Pound key to select. The total number of episodes will be announced; press the Pound key again to confirm the operation.

Please note that you can manage your Podcasts downloads with the Download manager (please read the “Download manager” section to know more).

### TuneIn Radio

TuneIn Radio is an application that allows to the user to browse and listen to radio stations and podcasts.

Please note: if you are logged to an account (free or premium), TuneIn Radio will place you at the last position in your podcasts or books where you were when closing the app during the previous session.

Note that Stream must be connected to the Internet in order to access content from this app.

Please note that this app can be hidden if you don’t intend to use it. To do so, go to the Configuration menu (key 7), then, under General settings, navigate with the 4 and 6 keys until attaining the App toggle menu and press the pound key on this option. Then, navigate with the 4 and 6 keys until attaining the TuneIn Radio app, and press the pound key on this option. The TuneIn Radio app will now be hidden and will not be available while cycling in your online bookshelves. All the data associated to this app will also be lost.

#### Accessing TuneIn Radio

- From the online bookshelf, press 1 repeatedly until you reach “TuneIn Radio”.

- Three described options are available: Browse, Search TuneIn Stations and Search TuneIn Podcasts. Press 4 and 6 keys to select one of the options, then press Confirm. Alternatively, these options can be accessed by pressing the Goto key multiple times until reaching the desired option.

#### Browse

The browse feature provides access to a variety of listening categories (note that the categories are subject to change). Press 4 and 6 keys to cycle through and select one of the listening categories, then press Confirm.

#### Search TuneIn Stations

The Search TuneIn Stations feature allows users to find and play specific radio stations.

- Press 4 and 6 keys to select Search TuneIn Stations, then press Confirm.

- Type in a selection, then press Confirm.

Number of results found for this specific search will be indicated.

- Press 4 and 6 keys to scroll through results found until you find your selection.

- Press Play/stop key to access your selection.

- Press Play/stop key to stop playback.

Please note that TuneIn disallows recording of any material available on this app.

#### Search TuneIn Podcasts

The Search TuneIn Podcasts feature allows users to find and play specific podcasts.

- From the main menu of the application, press 4 and 6 keys to select Search TuneIn Podcasts, then press Confirm.

- Type in a podcast title, then press Confirm.

Number of results found for this specific search will be indicated.

- Press 4 and 6 keys to scroll through results found until you find your selection.

- Press the Confirm key to select an episode.

- Press Play/stop key to play the episode or to stop playback.

Please note that unlike the Podcast bookshelf, TuneIn requires an Internet connection to listen to podcasts. Please also note that podcasts can be streamed but are not downloadable and cannot be recorded.

#### Podcast navigation in TuneIn Radio

When listening to a podcast, you can jump by time using the 2 and 8 keys to select the appropriate timeframe, then using the 4 and 6 keys to navigate in your podcast episode with this selected timeframe. Please note that only the values that were configured in the Settings will be displayed. You can also jump to a specific time in your episode by using the Goto key, then typing the desired time. It is also possible to navigate by episodes by selecting this navigation level using the 2 and 8 keys, then by using the 4 and 6 keys to navigate between the episodes of a specific podcast. Moreover, using the 5 key, you can obtain the title of the episode you are listening and the total time of this episode. If you press the 5 key during the following 10 seconds, you can obtain more information about this specific episode. Finally, pressing the Fast forward key will jump you 30 seconds ahead of your current position and pressing the Rewind key will bring you 10 seconds before your current position. Please note that it is not possible to navigate quickly in your podcast episode by pressing and holding the Fast forward or the Rewind key.

Please note: While listening a podcast episode, when pressing the Where Am I key (5) or the Info key (0) will not display the time elapsed and the time remaining.

#### Supported languages

TuneIn Radio employs the Stream's system language. If the Stream's system language is not supported, TuneIn Radio will operate in the default language, which is English.

List of supported languages includes French, Italian, German, Spanish, Japanese, English, Korean, Portuguese, Russian, Swedish, Turkish, Simplified Chinese and Traditional Chinese.

#### TuneIn Radio account types

TuneIn Radio users can register with the online service by using one of three different account types: Anonymous, Free account or Premium account.

- Anonymous account: This bookshelf is immediately available upon boot-up, just as with podcasts or Internet radio. Simply access the bookshelf, browse, and play. Using your serial number, favorites can be saved under your account. Any favorites added in anonymous mode will be saved for the specific device (via serial number). Note that you will not have access to premium content and there will be advertisements.

- Free account: On the TuneIn Radio bookshelf, press 7, then choose Add Account. You will be prompted to go to TuneIn.com/pair on a web browser, log in to your account, and enter the code. The code can be repeated on the Stream using keys 4 and 6. With a free account, you will have the same content as an Anonymous account, but you will be able to save favorites.

- Premium account: Same instructions as with the Free account, except that you will have access to premium content and there will be no ad content (advertisements will be absent).

Note: Because the Stream supports specific formats, search results may be different than with other devices (example: iPhone). In addition, certain radio stations may not be accessible while travelling in a region that does not support them, although they are displayed in the search results. Note that you will be able to access these radio stations once you return to the region that supports them.

## NLS Online Authorization

To use an NLS Bard account on your Victor Reader Stream, you first need to register your player on the NLS website:

- Access the NLS website: https://nlsbard.loc.gov/nlsbardprod/login/NLS

- On the website, enter your email address and password to log into your NLS account.

- Once connected, go to the section Account Settings, and access the item Add Purchased Player.

- Select HumanWare Victor Reader Stream from the list.

- Enter your player’s serial number and select the item Submit Player Request. Note that you can find the serial number of your Stream device by pressing and holding key 5.

- NLS will process your request. Note that this step could take up to 48 hours.

- You will receive an email with a .kxo file to be transferred on your device. Follow the instructions in the email on how to transfer the file, either via an SD card or directly in the internal memory of the device.

Note that after you submitted your player on the NLS website, if your player is connected to the Wi-Fi, the authorisation should process automatically when the Stream will detect a new update.

# Updating Stream Software

HumanWare may from time to time offer new versions of the Stream software. There are several ways to update your Stream’s software; wirelessly, by downloading an update file on your SD card or in the internal memory, and with the help of the HumanWare Companion software (see section 1.9 "HumanWare Companion" for more information on the HumanWare Companion software).

To update your Stream wirelessly, you must first have an active connection to the Internet using the built-in Wi-Fi (see section 6.3 "Wireless" for more information on the wireless configuration menu). You can perform the update on battery if it is above 50%, but it is highly recommended to plug your device during a software update. If the battery is too low, a message will indicate that you will have to plug the device to perform the update. If the Stream announces that it is in airplane mode, activate the Wi-Fi by pressing and holding the Online button to turn off the airplane mode. The Stream will automatically check for any available update. If an update is available, press Confirm to start the download or cancel with any other key. The download time may vary depending on the speed of your Internet connection. Once the download is completed, the Stream will ask your confirmation to proceed with the update. Press Confirm to start the update or cancel with any other key. Once the update is completed, the Stream will announce the new version number and power off. Please note that if you complete the update process on battery, you will have to restart manually the device to complete the installation, after the completion of the file download. Note: the software update file is visible in the download manager. Please read the “Download manager” section to know more. When you receive a prompt to update your system and you accept this update, the software update file will automatically be downloaded, prioritising itself among any other file, including  the current download. To download the software update file later, it will be necessary to cancel the update.

You can also update your Stream by downloading a SWU software update file from the HumanWare website. Copy the SWU file to the root of an SD card or of the internal memory. Connect the Stream to a power outlet with the AC power adapter or do it on battery if the device has more than 50% of charge. Power on the player and insert the card where the file is present, if this is the case.

The update will start and report the new version number that is being installed. The installation may take up to 5 minutes during which the Stream will periodically announce the update status in percentage. Once the update is completed, the player will power off. The SWU file will be automatically deleted from the SD card or the internal memory when you power on the Stream after performing an upgrade. To upgrade multiple Streams using the same SD card, be sure to remove the card from the upgraded Stream before turning the Stream back on.

Note: It is not possible to update from a USB flash drive.

Note that a software update will also update NLS keys and the different HumanWare list for podcasts and internet radio stations.

# Technical Specifications

Victor Reader Stream Specifications:

- Size: 114 x 62 x 18mm (4.5 x 2.4 x 0.7 inches)

- Weight: 110g (3.9 ounces)

- 3.5mm stereo Headphone/headset jack. Headset could include a mono microphone.

- 3.5mm stereo external microphone/line-in jack. Input Impedance: 2.5K

- Built-in mono omni-directional microphone

- Built-in 700 mW speaker

- Battery: Lithium-Ion, 3.7V nominal, 2500 mAh

- Battery recharge time: Up to 3 hours with the included AC adapter/cable. May be longer with other chargers, cables, or PC recharge.

- Battery Playtime: Up to 15 hours continuous play of DAISY or NISO book using headphones without Wi-Fi (may vary depending on content and player settings)

- Power Supply: Switching type AC/DC. Input 100V – 240V, 50Hz – 60Hz. Output: Type A USB socket, 5VDC, 1.5A

- Operating temperature range: +5 to + 45 degrees Celsius

- Battery charging temperature range: +10 to + 45 degrees Celsius

- Storage and transportation temperature: -20 to + 45 degrees Celsius

- Operating humidity: 5% to 95% (non-condensing)

- Storage and transportation humidity: 5% to 95% (non-condensing)

- USB-C OTG compliant interface

- SD (Secure Digital) card slot supports SD, SDHC and SDXC cards.

- Book formats supported: DAISY 2 / 2.02, NISO Z39.86 2002/2005, unprotected EPUB 2, LGK

- DRM: 2002 PDTB1 (so-called I.P.P.) and 2006 PDTB2

- Audio formats supported: AAC (.mp4, .m4a, .m4b, .m4v), AMR-WB+ (.3gp), Flac, MPEG2, MP3, OGG Vorbis (.ogg), Opus, Speex (.spx), Wav P.C.M.

- Text files supported: bra, brf (including bopf and ebrf), docx, fb2, html, lkf, pdf, rtf, txt, xml.

- Text-to-Speech: Acapela

- Recording: Mono: MP3 16-bit at a sampling rate of 44,100Hz with a bit rate of 32, 64 or 96 kbps, PCM 16-bit at a sampling rate of 44.100Hz and FLAC. Stereo: MP3 16-bit at a sampling rate of 44,100Hz with a bit rate of 128, 192 or 320 kbps, PCM 16-bit at a sampling rate of 44.100Hz and FLAC.

- Wi-Fi: IEEE 802.11a/b/g/n/ac operating in the 2.4GHz and 5GHz bands (channels 36 to 48 and 149 to 165 are available in 5GHz).

# Safety Precautions

## Contraindications

There is no special contraindication for the use of the device.

## Warnings

- Never use household cleaners to clean the Victor Reader Stream.

- Never use cleaning products that contain Ethyl Alcohol, Ethyl Acid, Ammonia, Acetone, or Methyl Chloride.

- Any serious incident that has occurred in relation to the device should be reported to the manufacturer and the competent authority of the Member State in which the user and/or patient is established.

- Do not use headphones while walking on the street.

## Care and maintenance

To keep your Stream clean, we recommend that you wipe the surface of the unit periodically using a soft damp cloth. Squeeze cloth to remove excess moisture. Only use warm water. Do not use any cleaning agents. The product is not intended to be disinfected.

## Storage and transportation

The product should not be folded or dismantled in storage or transportation.

The product can be carried as any electronic device in a car or plane. It does not have any specific restrictions.

## Additional information

After long exposition to the sun, the surface temperature might increase.

The product has performed immersion and electromagnetic immunity tests and should not cause any interference, or be influenced by any other product.

The product material has a V-0 level rating to ignition.

The product is designed to have a lifetime exceeding 5 years.  The Battery is designed to have a lifetime of 3 years.

## Measured sound Power Level

Headphone sound power output is limited to levels of EN50332.

The speaker maximum power level is 92 dBA at 1 meter.

## Service information

No maintenance, calibration or preventive inspection is needed on the player.

If the user finds the battery autonomy has decreased, a replacement request may be made to HumanWare (for a fee).

Contact HumanWare or your distributor for any repair or malfunction questions.

Service shall only be done by HumanWare or its authorized distributor.

## Susceptibility to Interference

There may be temporary degradation of the sound when the Stream is subjected to a strong radio-frequency field, electro-static discharge or transient electrical noise.

FCC Warning

This device complies with part 15 of the FCC Rules. Operation is subject to the following two conditions: (1) This device may not cause harmful interference, and (2) this device must accept any interference received, including interference that may cause undesired operation.

Changes or modifications not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

Note: This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

—Reorient or relocate the receiving antenna.

—Increase the separation between the equipment and receiver.

—Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

—Consult the dealer or an experienced radio/TV technician for help.

Industry Canada statements

This device complies with Industry Canada licence-exempt RSS standard(s). Operation is subject to the following two conditions: (1) this device may not cause interference, and (2) this device must accept any interference, including interference that may cause undesired operation of the device.

Le présent appareil est conforme aux CNR d'Industrie Canada applicables aux appareils radio exempts de licence. L'exploitation est autorisée aux deux conditions suivantes: (1) l'appareil ne doit pas produire de brouillage, et (2) l'utilisateur de l'appareil doit accepter tout brouillage radioélectrique subi, même si le brouillage est susceptible d'en compromettre le fonctionnement.

CAN ICES-3 (B)/NMB-3(B)

## Audio Warning

To prevent possible hearing damage, do not listen at high volume levels for long periods. Exercise caution when holding your device near your ear while the loudspeaker is in use.

## Battery Safety Precautions

Battery becomes warm during recharging. Do not recharge battery near a heat source. Battery will not recharge if internal temperature is too high. Only use supplied adapter and cable to charge the Victor Reader Stream. If the battery is discharged for a long period of time, there will be a delay of up to a few minutes before the device shows any activity, including charging. Do not submerge the product – there is a risk of overheating, fire or even explosion if the battery is put in a fire, heated, subjected to impact, fully submerge for a long period of time or if the battery terminals are shorted. Do not disassemble or modify the unit. Do not strike, throw or voluntary hit the device. Do not strike the unit with sharp objects or blunt object.

In the event that the battery leaks from the device and gets on your skin, wash with fresh clean and seek medical attention to prevent injuries. If battery leaks and liquids gets in contact with eyes, DO NOT RUB, rinse with clean running water and seek immediate medical attention.

## Disposal Instructions

At the end of this device operational lifetime, its internal components must be disposed in compliance with local authorities.

This device contains no hazardous materials. For disposal, return to HumanWare or follow local governing ordinances or hospital procedure.

# Technologies HumanWare Contact Information

1800, Jean-Berchmans-Michaud street

Drummondville, Quebec

Canada J2C 7G7

Telephone: 1 (819) 471-4818

Toll-free: 1 (888) 723-7273 (Canada) ; 1 (800) 722-3393 (USA)

Fax: 1 (819) 471-4828

E-mail: support@humanware.com

Website: www.humanware.com

# End User License Agreement

By using this product (Victor Reader Stream) you agree to the following minimum terms.

      - License Grant. HumanWare grants to End User a non-exclusive, non-transferable right and license to use the Software on this product.

      - Ownership of Software. End User acknowledges that HumanWare retain all right, title and interest in and to the original, and any copies, of software which is incorporated into this product. End User agrees not to: modify, port, translate, decompile, disassemble, reverse engineer, or make public in any way the software of this Product.

This product includes software developed by the OpenSSL Project for use in the OpenSSL Toolkit (http://www.openssl.org/).

This product uses libraries from the ffmpeg project under the lgplv2.1.

# Appendix 1 - Manufacturer Warranty

This device is a high quality product, built and packaged with care. All units and components are guaranteed against any operational defects as follows:

US and Canada: One (1) year

Continental Europe and UK: Two (2) years

Australia and New Zealand: One (1) year

Other countries: One (1) year

Warranty covers all parts (except battery) and labor. If any defect should occur, please contact your local distributor or the manufacturer technical assistance line.

Note: Warranty terms may periodically change, please consult our website for the latest information.

Conditions and Limitations:

No replacement or repair covered by the warranty will be carried out unless the unit is accompanied by a copy of the original bill of purchase. Please retain your original. If the unit has to be returned, please use the original packaging. This warranty applies to all cases where the damage is not a result of improper use, mistreatment, negligence or acts of God.

(Final)