User

Guide

⠥⠎⠑⠗ ⠛⠥⠊⠙⠑

Victor Reader Stratus 2

Victor Reader

Stratus 2

User guide by HumanWare

Version 1.6 2026-04-15

©Copyright 2026. All rights reserved, Technologies HumanWare Inc.

This manual is protected by copyright belonging to Technologies HumanWare Inc., with all rights reserved. According to copyright law, the manual may not be copied in whole or in part, without written consent from Technologies HumanWare Inc.

Table of Contents

About Victor Reader Stratus 2	4

Getting Started Session with Victor Reader Stratus 2	5

To insert and play a new book	6

Key Describer mode	6

1.	Overview of Victor Reader Stratus 2	7

1.1	Unpacking the Player	7

1.2	Stratus 2 functionalities	7

1.3	Stratus 2 cover	7

1.4	Physical Description of Victor Reader Stratus 2	7

1.4.1	The Face of the Player	7

1.4.2	The Left Side of the Player	8

1.4.3	The Right Side of the Player	8

1.4.4	The Front Side of the Player	8

1.5	Charging the Battery	8

1.6	Powering On/Off	9

1.7	Book Detection	9

1.8	Bookshelf Structure	9

1.9	Other Reserved File Names	10

1.10	HumanWare Companion	10

2.	Basic Functions	11

2.1	Audio Controls	11

2.2	Play/Stop	11

2.3	Rewind and Fast Forward	11

2.4	Key Describer Mode	11

3.	Navigation Functions	12

3.1	Description of Keys	12

3.1.1	Undo navigation	12

3.2	Bookshelf	12

3.3	Time Jump Navigation Mode	12

3.4	Ten page or ten chapters Jump	12

3.5	Text-to-Speech Navigation levels	13

3.6	Spell Mode for Text Bookshelf	13

3.7	Stratus 2 and text input fields	13

3.8	User Guide — Bookshelf key (Press and hold)	13

3.9	Bookshelf Navigation — Bookshelf Key	13

3.10	Deleting Books – Press the Book management key	13

3.11	Information	13

3.11.1	Available Information Items	14

4.	ADVANCED FUNCTIONS	15

4.1	Sleep Mode and Time announcement	15

4.2	Bookmarks	15

4.2.1	Inserting bookmarks	15

4.2.2	Going to a bookmark	15

4.2.3	Deleting bookmarks	15

5.	Bookshelf Structure and Characteristics	16

5.1	Talking Books	16

5.1.1	Talking Books Structure	16

5.1.2	Talking Books Characteristics	16

5.2	Other Books	17

5.2.1	Other Books Structure	17

5.2.2	Other Books Characteristics	17

5.3	Music Bookshelf	17

5.3.1	Music Structure	17

5.3.2	Music Characteristics	18

5.3.3	Playlists	18

5.3.4	Folder and File Name Announcement	18

5.4	Saved podcasts Bookshelf	18

5.4.1	Saved Podcast Structure	18

5.4.2	Saved Podcasts Characteristics	18

5.5	Text Bookshelf	19

5.5.1	Text File Structure	19

5.5.2	Text File Characteristics	19

5.5.3	Navigating HTML/XML/DOCX Headings	19

6.	Configuration Menu	20

6.1	Menus and Menu Items List	20

6.2	General Settings	21

6.2.1	Language	21

6.2.2	System	22

6.3	Navigation and Playback	22

6.3.1	Time Jump	22

6.3.2	Save Last Used Navigation Level for Each Book	22

6.3.3	Bookmark alert	23

6.3.4	Audio adjustment mode	23

6.3.5	Loop	23

6.3.6	Music	23

6.3.7	Book time details	23

6.4	Wireless	23

6.4.1	Airplane Mode	23

6.4.2	Wi-Fi	24

6.4.3	Bluetooth	25

6.5	About	26

6.6	Online settings	26

6.6.1	Software Update	26

6.6.2	Notification mode	27

6.6.3	Book Services	27

7.	Daisy Online	29

7.1	Daisy Online configuration menu	29

7.2	Using Daisy Online Service	29

7.3	Download Methods	30

7.4	Navigating in your DAISY online bookshelves	30

8.	Updating Stratus 2 Software	31

9.	Troubleshooting	32

9.1	Book Navigation	32

9.2	Battery and Charging	32

9.3	General	32

10.	Customer support	34

11.	Technical specifications	35

12.	Safety precautions	36

12.1	Contraindications	36

12.2	Warnings	36

12.3	Care and maintenance	36

12.4	Storage and transportation	36

12.5	Additional information	36

12.6	Service information	36

12.7	Susceptibility to Interference	36

12.8	Audio Warning	37

12.9	Battery Safety Precautions	38

12.10	Disposal Instructions	38

13.	Manufacturer Warranty	39

14.	End User License Agreement	40

# About Victor Reader Stratus 2

HumanWare is proud to introduce Victor Reader Stratus 2, the updated and improved version of the Stratus 4M player. As the Stratus 4M, the Stratus 2 is the simple and friendly digital talking book player you were looking for, actualized to fulfill today’s needs. The Stratus 2 is no longer a CD player, leaving more place to dematerialized content through a cartridge, external media content and online libraries. Moreover, Stratus 2 menus were standardized with Stream 3’s menus, that will ensure a more convenient navigation.

Victor Reader Stratus 2 was designed for leisure reading like best-sellers, novels, and magazines. It allows you to easily navigate within the structure and quickly go to desired information.

With its simple to use functions, you can consult the table of contents, go to a section, or return to a bookmark.

We wish you many pleasant hours discovering all the facets of our digital talking book player!

# Getting Started Session with Victor Reader Stratus 2

Welcome to Victor Reader Stratus 2, your new digital talking book player.

The package contains the following items:

- A Victor Reader Stratus 2, with a keypad cover (the cover can be removed to expose device’s navigation keys);

- A power adapter with a USB-A cable, and

- A plastic bag with documentation.

The first time you use Victor Reader Stratus 2, you should connect the player to a power outlet.

To connect to a power outlet, plug one end of the provided USB-C in the USB-C port. It is located near the right corner on the right side of the player. Plug the USB-A part of the cable in the AC power that you will plug in an electrical outlet. When first receiving the player or after an extended storage period, it is best to leave the player plugged into the power outlet overnight to ensure that the battery is fully charged.

The Power button will glow steady when the device is used and it will blink when off but recharging, the flashes indicating that the battery is recharging. When it stops flashing, the battery is fully charged. When the player is turned On, the power button glows green, whether it is plugged in or not. Under regular circumstances, the battery takes up to 3 hours to recharge and will provide up to 20 hours of playtime for most DAISY books.

Please note that you can use Victor Reader Stratus 2 while charging the battery.

You can pause and resume reading by simply pressing the Play/Stop key, which is located at the bottom center of the player between the Rewind and Fast-Forward keys.

We will now begin the Getting Started session. Place the player in front of you with the carrying handle on the left. Please note that this description is valid if the cover has been removed. If the cover is on the device, the available options are limited.

Above the handle in the top left corner is a built-in speaker.

On the top center of the player are three pairs of up and down keys. From left to right, these are the Tone, Volume and Speed keys.

Beneath the Tone, Volume and Speed controls, roughly in the center of the player, you will find the Navigation keys, pointing UP, DOWN, LEFT and RIGHT. These allow you to move through the structure of a book. In the middle of these navigation keys is the Information key. The Information key announces your current reading position, information about the book, and is used to open the menu while doing a long press.

To the left of the four Navigation keys, about midway down the player, are three more keys aligned vertically. The first key is the Bookshelf key. You can use the Bookshelf key in combination with the 4 arrow keys to access the books you may have organized in different bookshelves. Immediately underneath the Bookshelf key is the Bookmark key. The Bookmark key is used to insert bookmarks to save important pages or paragraphs for future reference. The third and lowest key is the Book Management key and is used to manage books. With a long press, you can also obtain information about the battery and current downloads.

To the right of the four Navigation keys is the Power key. It is recessed to prevent accidental pressing. The Power key is used to power the Stratus 2 ON and OFF. Note that when the Stratus 2 is off and you connect it to the power outlet, it will take a few moments before the Power key becomes operational.

Beneath the four Navigation keys is a group of 4 keys. Three of them are lined up along the lower edge of the player. These are, from left to right, the Rewind, Play/Stop and Fast Forward keys. Finally, above the Play/Stop key is the Sleep key. It is a round key with a moon symbol and has an LED indicator to its right. Press it multiple times to set different time intervals after which the Stratus 2 will automatically power off. Press and hold the key to get the date and time.

On the right side of the player, you will find from top to bottom the following components: a USB-C port (used for charging), a USB-A port, and the headphone jack, which can also be used for an external speaker.

On the front side of the player is the cartridge slot to insert a cartridge.

## To insert and play a new book

Turn On Victor Reader Stratus 2 by pressing and holding the Power key. You will hear a beep and a Welcome message.

Insert your cartridge or any external storage support (USB) if you have one. After a few seconds, if the cover is on the device, Victor Reader Stratus 2 will announce the title and start playing the book. If not, the focus will be on the first book. Press the Play/stop key to start reading.

You can adjust the Tone, Volume, and Speed by pressing the keys up or down. The Tone, Volume, and Speed control keys are located on the top center of the face.

To move back or forward at high speed in the book, press and hold the Rewind or Fast-Forward key until you reach the desired spot, then release. The normal play speed will be resumed.

To pause a reading session, simply press the Play/Stop key. To resume reading where you left off, press the Play/Stop key again. Note that if your player is not connected to a power outlet, and if it stays in pause mode for more than 30 minutes, it will shut Off automatically to save battery life.

To end a reading session, simply turn the player Off by pressing and holding the Power key until the player announces that it is closing. Victor Reader Stratus 2 will remember where you left off in the book for your next reading session.

To start a new reading session in the same book, turn On your Victor Reader Stratus 2 by pressing and holding the Power key. After a few seconds, if the cover is on the device, you will hear the title of the book, a beep and reading will start where you left off in your last reading session. If not, when booting your Stratus 2, you will hear the title of the book. Press the Play/Stop key to start reading.

## Key Describer mode

When there is no book present, the Stratus 2 is automatically set to Key Describer mode. In this mode, anytime a key is pressed, the player announces a short description of its function. This can be useful when learning how to use the player.

This completes the Getting Started session. The User Guide follow.

# Overview of Victor Reader Stratus 2

## Unpacking the Player

The package contains the following items:

- A Victor Reader Stratus 2, with a keypad cover (the cover can be removed to expose device’s navigation keys);

- A power adapter with a USB-A cable, and

- A plastic bag with documentation.

This manual contains a short Getting Started session followed by the User Guide.

Before using Stratus 2 for the first time, you should connect the player to a power outlet to charge the battery. You can use your Victor Reader Stratus 2 while charging the battery.

## Stratus 2 functionalities

The Stratus 2 player can play content found on cartridges and USB flash memory. Stratus 2 also has access to books on the Internet through a Wi-Fi connection.

## Stratus 2 cover

Your player has a keypad cover installed. The cover prevents the use of book navigation keys for those users who may not wish to use them. If you wish to take full advantage of the book navigation features of your Stratus 2, then you may remove the cover.

The Keypad cover is attached at the four corners of the player. The cover is held in place by a tab on each corner.

The Keypad cover prevents the use of the following keys: Bookshelf, Bookmark, the four navigation arrow keys and the Information key.

If you wish to remove the Keypad cover:

- Hold the player with the carrying handle using your left hand.

- Using your right hand, locate the top right corner of the cover. The tab is in the middle of the corner.

- Place the side of your right index finger underneath the tab.

- Using your thumb as leverage on the side of the player, apply pressure to slightly lift the tab from the side of the player.

- Using same method, slightly lift the tab on the lower right corner of the cover.

- With both corners raised, you should be able to lift the cover off. If needed, lift the bottom left corner and remove the Keypad cover.

If you wish to reinstall the Keypad cover, you will have to align the cover properly over the Stratus, making sure the carrying handle of the Stratus and that of the cover are overlapping.

Then, press down on the cover until all four corners snap into place.

Note: after the removal or insertion of the cover, a prompt will ask you to reboot your Stratus 2. Press the Play/Stop key to confirm.

## Physical Description of Victor Reader Stratus 2

### The Face of the Player

The built-in speaker is located on the upper left corner of the face.

On the top center, there are three pairs of up and down keys. From left to right, these are the Tone, Volume and Speed controls.

Beneath the Tone, Volume and Speed controls, you will find the four arrow shaped navigation keys UP, DOWN, LEFT and RIGHT allowing you to easily move through the structure of a book. In the middle of these navigation keys, is the Information key. The Information key announces the current reading position and general information about the book. When doing a long press, it allows you to go directly to the menu.

To the left of the four Navigation keys, about midway down the player, are three more keys aligned vertically. The first key is the Bookshelf key. Immediately underneath the Bookshelf key is the Bookmark key. The Bookmark key is used for bookmarks, that are important positions in your book that you want to mark for future reference. Press the Bookmark key multiple times to choose to display the bookmark list, remove all bookmarks, or press and hold the Bookmark key to insert a bookmark.

The third and lowest key is the Book management key. When in a bookshelf, this key can be used to return (or delete) a book or to copy them to a USB key. In an input field, this key is used to cancel the operation.

To the right of the four Navigation keys is the Power key. It is slightly recessed to prevent accidental pressing. The Power key is used to power the Stratus 2 ON and OFF.

Beneath the four Navigation keys is a group of 4 keys. Three of them are lined up along the lower edge of the player. These are, from left to right, the Rewind, Play/Stop and Fast Forward keys. Finally, above the Play/Stop key is the Sleep key. It is crescent shaped and has an orange LED indicator to its right. The Sleep key is used to get time announcements and activate the sleep timer. You can press and hold this Sleep key to announce the time or press multiple times to set different Sleep time periods after which the player will automatically shut off.

### The Left Side of the Player

On the left side of the player, you will find the built-in handle and speaker.

### The Right Side of the Player

On the right side of the player, near the center, you will find from top to bottom the following components: USB-C port (used for charging), USB-A port, and headphone jack.

### The Front Side of the Player

You will find the slot to insert a cartridge, if you have one.

## Charging the Battery

To recharge the battery, simply connect the Stratus 2 to AC power. If the player is on when you connect, you will hear a short chime confirming that power is connected. You will hear the chime again when disconnecting from the AC power outlet. However, the player will recharge regardless of whether it is on or off. The battery will automatically recharge whenever the player is plugged to AC power. The player can be used while recharging.

Under normal conditions, the fully charged battery can last up to 20 hours for continuous play of DAISY books. However, the battery playtime may be reduced in the following cases:

- If the battery is charged for less than 3 hours.

- Extensive use of book navigation commands.

- High Volume Level or High-Speed reading.

- Connection to the internet.

- Use of external Bluetooth speakers.

NOTE: When the player is not connected to a power outlet and staying in pause mode for more than 30 minutes, it will shut off automatically to save the battery.

## Powering On/Off

To turn the player On, press and hold the Power key. You will hear a beep and a welcome message.

To turn the player Off, press and hold the Power key again. You will hear a “shutting down” message, followed by two beeps indicating the player is powering Off. Note that when the Stratus 2 is off and you connect it to the power outlet, it will take a few moments before the Power key becomes operational.

## Book Detection

Please note that the following description applies if the key cover is on the device.

When the Stratus 2 is powered on, it will look for the last book read. If this book is not found, it will open the first book it finds in the available media according to the following order: cartridge, USB-A, and Internal memory. If your last book read is found, it will begin playing automatically.

If the key cover is not on the device at the boot of the Stratus 2, it will place your focus on the last opened book. If your last opened book is not present, you will be directed to the last bookshelf used if there is some content; if not, you will be directed to the next bookshelf containing any content.

## Bookshelf Structure

The Stratus 2 recognizes various book types which are saved in separate folder structures called Bookshelves. You can select the Bookshelf you want to read with multiple presses of the Bookshelf key. Move backward and forward along each Bookshelf using the Right and Left keys. The Bookshelf list is circular. On the cartridge or USB source, each Bookshelf is stored within a reserved folder name beginning with “$VR”. Within each of these reserved folders (Bookshelves), there may be user defined subfolders containing separate books or categories of files. You may put other folders and files on your media but it is only the content of the $VR reserved folders that define the Stratus 2 bookshelves.

The Bookshelf structure and usage are defined as follows:

Bookshelf: Talking Books ($VRDTB folder).

Book types: DAISY 2.x, NISO Z39.86 (DAISY 3), EPUB 2

Usage: The files comprising each of your DAISY or NISO books should be saved in separate subfolders under this folder. Use the Right / Left keys to navigate through the book list in the bookshelf if more than one book is present. Press Play to select the book. In this bookshelf, the current reading position and bookmarks are saved separately for each book.

Bookshelf: Other Books ($VROtherBooks folder).

Book types: Non-DAISY or non-NISO recorded books composed of any of the Stratus 2 supported audio file types.

Usage: The files comprising each book should be saved in separate subfolders under this folder. Folders may be nested. Optionally, single-file books may be directly placed in the root folder. Use the Right / Left keys to navigate throughout the books. Press Play to select the book. In this bookshelf, the current reading position and bookmarks are saved separately for each book. Playback stops at the end of the last file of each book.

Bookshelf: Music ($VRMusic folder).

Book types: Recorded files composed of any of the Stratus 2 supported audio file types.

Usage: A music book is either a playlist or the entire music structure (referred to as All Music book). The current reading position and bookmarks are saved separately for each book. Navigate between music books using the Right / Left keys on the bookshelf. The All Music book consists of nested folders containing audio files. For example, the structure could be genre, artist, album, and tracks. Navigate the All Music book using the Up / Down keys to choose folder or file navigation level and then move within that level using Right / Left keys. Playback continues from the last file of one folder to the first file of the next folder.

Bookshelf: Saved podcasts ($VRPodcasts)

Book types: Any of the Stratus 2 supported audio file types.

Usage: Each podcast file is defined as a separate book meaning you must move from file to file using the Right / Left keys while in the Bookshelf. Reading position and bookmarks are saved separately for each book (file). Playback stops at the end of each book (file).

Bookshelf: Text ($VRText folder).

Book types: Text files in any of these formats: docx, fb2, html, lkf, pdf, rtf, txt, xml.

Usage: Each text file is defined as a separate book meaning you must move from file to file using the Right / Left keys while in the Bookshelf. Reading position and bookmarks are saved separately for each book (file). Playback stops at the end of each book (file).

Bookshelf: Notes ($VRNotes folder, USB key only)

Book types: Recorded audio notes

Usage: This is a single list of files recorded by another Victor Reader product (the Stream) where each file was given an incremental number or a custom filename. Each note is defined as a single book, meaning there is one current reading position and set of bookmarks for each note. If a subfolder is present (for example, the Generic folder), press the Play/Stop key to open this subfolder, then navigate with the Right and Left keys throughout the notes files. Press the Play/Stop key to begin reading a note. Press the Play/Stop key to stop reading. Playback stops at the end of each book (file).

## Other Reserved File Names

The Stratus 2 may create other file names beginning with “$VR” on your media. Removing or altering these files could result in unpredictable behavior.

## HumanWare Companion

The HumanWare Companion is a PC software running on the Microsoft Windows operating system used in conjunction with the Victor Reader Stratus 2 and other Victor Reader products to manage your books, music, podcasts and text files. You can use the HumanWare Companion to copy your books, music, podcasts and text files to the device, or remove these files from the device through the use of a computer. To download and install the HumanWare Companion software, please visit: https://www.humanware.com/en-international/support/humanware_companion/

# Basic Functions

## Audio Controls

Each of the Tone, Volume, and Speed controls has an Up and Down key. There is a beep to mark the upper and lower range of each control. If the book is not playing (or the key cover is attached) Stratus 2 will announce the setting position. For tone and speed, there is also a beep to mark the normal or 0 position. This indicates either a flat tone setting or normal speed setting.

## Play/Stop

To start playing a book, press the Play/Stop key.

To stop playing a book, press the Play/Stop key again.

## Rewind and Fast Forward

These keys allow you to go backward or forward quickly. You will hear the recording at a faster speed.

Press and hold the Rewind or Fast Forward key until you reach the desired spot, the movement of the Rewind or Fast Forward keys will accelerate the longer you hold the key down. For each few seconds you hold the keys, you will move forward or backward 1 minute, 2 minutes, 5 minutes, 10 minutes, 15 minutes and so on. The largest jump is 5 minutes. Stratus 2 will announce the time jumps and play a normal speed clip between each time jump. You can also press the Fast Forward and Rewind keys once to jump 5 seconds forward or backward or to jump to the next or previous line in text-based documents.

## Key Describer Mode

With the Key Describer feature, you can learn the function associated with the Stratus 2 keys. This is especially helpful when learning how to use the player.

If no book is present: The Key Describer mode will be activated automatically.

If a book is present: You can activate the Key Describer mode by pressing the Power key. Press the Power key again to exit Key Describer mode.

While in Key Describer mode, you may press any key to hear its function.

# Navigation Functions

## Description of Keys

Beneath the Tone, Volume and Speed controls, you will find the four Navigation keys: Up, Down, Left and Right. These keys allow you to easily choose the level of book navigation to move from one book element to another, quickly going to desired information.

Examples: moving by level 1 (chapter), level 2 (section), page, and so on. These navigation levels are defined by the book’s producer so they may vary from book to book.

First, choose a book navigation level. Press the Up or Down key successively until you hear the desired level. The last level spoken is the one selected.

Then, use the Right key to move forward or the Left key to move backward from one element to another until you reach the desired book element. The last element spoken is the one selected. Reading will start in the selected book element.

### Undo navigation

The Undo function allows you to reverse an unwanted move if you accidentally press the Left or Right navigation keys. For example, if you accidentally press the Right key and jump to the next book element, you can press the Left key to undo the move and return to where you were. Similarly, you can press the Right key to reverse an accidental move made with the Left key.

Note: To undo an unwanted action, you must press the opposite button within 10 seconds of the action.

## Bookshelf

If there are multiple titles available on your media, then you may press the Bookshelf to access the book list. The Left and Right keys allow you to move from title to title to choose the book you want. Confirm your book choice with the Play/Stop Key. Stratus 2 will start to play the selected book.

## Time Jump Navigation Mode

Time Jump navigation can be activated using the Up or Down keys.

Press the Up or Down key to select the Navigation level, until Stratus 2 announces "Time Jump" followed by the Jump Interval. The Time Jump interval can be set at 30 seconds, 1, 5, 10 or 30 minutes in the Navigation and Playback section of the configuration Menu (Press and hold Information key).

## Ten page or ten chapters Jump

The Right and Left keys allow you to move in 10-page jumps to quickly reach a specific page.

Select Page Navigation level by pressing the Up or Down key until you hear "Page".

Then, press and hold the Right or Left key. You will hear Victor Reader Stratus 2 skip backward or forward in 10-page jumps.

Release the key when you are close to the desired page, then forward a page at a time by pressing the Right or Left keys successively until you reach the desired page. If your book does not offer page navigation, then the "Page" element will not be announced when you press the Up/Down keys.

Note: It is also possible to jump backward or forward ten elements while level 1 to 6 are selected, as described above.

## Text-to-Speech Navigation levels

When Stratus 2 is in Text-to-speech mode, there will be additional text navigation levels added to the Up / Down key rotation. These include the text levels: screen, paragraph, line, sentence, word, spell and character. Screen and line levels are defined arbitrarily as 25 lines per screen and a maximum of 80 characters per line. As with other navigation levels, move between elements of the selected level using the Right / Left keys. Note that the Screen level will only be available when no pages are defined in the book.

## Spell Mode for Text Bookshelf

Spell mode can be selected using the Up / Down keys. The Spell mode can be found between the Word and character navigation levels. Use the Right / Left keys to move word by word. Stratus 2 will announce the word and spell it. Note that capital letters are indicated and that words will be spelled at normal speed, regardless of TTS speed setting.

## Stratus 2 and text input fields

When in text input fields, you can navigate through the characters with the Up and Down keys. Characters are circular: lower case letters are displayed at first, then upper case letters, punctuations, symbols, etc. To go directly to a specific category, use the Bookmark key. To type a character, press the Right key. To erase a character, press the Rewind key. To make the Stratus 2 indicate all the text written in the text input field, press the Fast forward key. Finally, to end typing, press the Play/Stop key.

## User Guide — Bookshelf key (Press and hold)

You can access the built-in user guide at any time by pressing and holding the Bookshelf key. Press and hold Bookshelf again to exit.

## Bookshelf Navigation — Bookshelf Key

Press the Bookshelf key multiple times to rotate through the circular list of Bookshelves. Empty bookshelves will not be announced. Bookshelves contain different types of books and files as defined in the section 1.7 “Bookshelf Structure”. You can move backward and forward along a Bookshelf using the Right and Left keys. When you reach the desired book, press Play to open it.

## Deleting Books – Press the Book management key

When navigating the bookshelf, you can delete a book on your media or on the internal memory (except the All music book) by pressing Book management key.

Note: to delete a book, you have to be positioned on that book in the bookshelf or while reading this book.

## Information

The Information key announces your position in your current book or, while in a bookshelf, gives information about the book where your focus is positioned. Pressing and holding the Information key will direct you to the device’s menu. Press the Information key to announce available information items.

### Available Information Items

Book information items vary according to book type. Typically, information will include the following:

- Book description (if available)

- Book subject (if available)

- Book type (if available)

- DAISY/NISO talking books: Current position, current page (if available), current heading, time remaining in the current section

- Music Bookshelf: Current position, current folder, current file in this folder, total time, time elapsed, time remaining

- Other books bookshelf: Current position, current file, filename, total time for this file, time elapsed for this file, time remaining for this file

- Saved podcasts bookshelf: Current position, total time, time elapsed, time remaining

- Text files: Current position, current page (if available), current heading

- Notes bookshelf: Current position, current folder name (if any), current note name/number, total time for this note, time elapsed for this note, time remaining for this note, creation date

Note: to know more about your device status (battery power and if it is charging or not, status of the current download if any and Stratus 2 version), press and hold the Book management key.

# ADVANCED FUNCTIONS

## Sleep Mode and Time announcement

The Sleep button allows you to obtain time announcements and set the Sleep Timer.

Press and hold to hear the time and date.

The Sleep feature allows the Stratus 2 to be set to automatically power off after different lengths of time: 15 minutes, 30 minutes, 45 minutes, 60 minutes.

Press Sleep multiple times to select your desired Sleep time. To deactivate the Sleep function, press Sleep repeatedly until you reach the OFF option.

When the sleep timer is active, the LED located to the right of the Sleep button is turned ON. One minute before the timer powers off the player, the light will start blinking and you will hear a message informing you that 1 minute of Sleep remains. When the timer expires, the player announces “sleep off”, then it powers off without beeping.

## Bookmarks

The Bookmark key marks positions in the book, so you can return to them later.

### Inserting bookmarks

Press and hold the Bookmark key to insert a bookmark. A number will be assigned to each bookmark. A message will confirm the bookmark insertion and inform you of the assigned number.

### Going to a bookmark

Press the Bookmark key until you hear the message “Bookmark List”. Then, press the Right or Left keys to select the desired bookmark, and the playing of the bookmarked position will start automatically. Press the Play/Stop key to stop reading. To leave the bookmark list, use the Up or Down key to select another level, then press the Play/Stop key to start reading from the current position.

NOTE: The bookmark list always contains one bookmark for the beginning and end of book. Bookmarks are sorted by their position in the book, not by the bookmark number.

### Deleting bookmarks

Press the Bookmark key twice to delete all bookmarks from the current book. The player will ask you to confirm the deletion. To confirm, press Play/Stop. A prompt will indicate that the bookmarks has been deleted. To cancel, press any other key.

NOTE: Bookmarks and current position are always saved separately for each book.

# Bookshelf Structure and Characteristics

For files on all bookshelves, the maximum file name length, including the file path, is 512 characters (includes media/book path). When scanning the USB key at insertion, any folder or file with a full path name greater than 512 characters will be ignored.

The following sections provide general rules and information for listening to the bookshelves.

## Talking Books

### Talking Books Structure

Talking Books are DAISY, NISO or EPUB 2 type books and are saved in the $VRDTB bookshelf folder. DAISY and NISO require one folder per book. EPUB 2 books are single files.

Folder: \$VRDTB.

Rules: Each EPUB 2 file or DAISY folder is defined as a book with its own reading position and bookmarks. Books can be deleted by pressing the Book management key while browsing the bookshelf or when positioned directly in the book to delete.

File: \$VRDTB\Book1.EPUB 2

Rules: This is an EPUB 2 book at the root of $VRDTB.

Folder: \$VRDTB\Book1

Rules: This is a folder containing a set of DAISY or NISO book files. Thus, it defines a DAISY or NISO book at the root of $VRDTB.

Folder: \$VRDTB\Drama.

Rules: This is an optional folder to categorize the type of book. It will contain further subfolder categories, EPUB 2 books, or DAISY/NISO book folders.

Folder: \$VRDTB\Drama\BookA.EPUB 2

Rules: This is an EPUB 2 book file in the Drama category.

Folder: \$VRDTB\Drama\BookB.

Rules: The folder containing the DAISY or NISO book files. The entire folder is defined as the book.

Folder: \$VRDTB\Mystery.

Rules: This is an optional folder for categorizing type of book.

Folder: \$VRDTB\Mystery\BookC.

Rules: Same as Book B.

### Talking Books Characteristics

Bookshelf Browse: Navigate between each book file with the Right and Left key. Press the Book management key until hearing “Delete current book” to delete a book.

Reading: Navigate the book using the Right and Left keys. Choose navigation level with the Up and Down keys. Headings, page, phrase, and Time Jump navigation levels are available.

Information: Reports information for the current book.

## Other Books

### Other Books Structure

Folder: \$VROtherBooks.

Rules: Each file in the root or each folder is defined as a book with its own reading position and bookmarks. Each single file in the root is considered as a separate book. If a book is in a folder, all the files in this specific folder are the constituent of the same book. The single book files in the root and the single folders can be deleted by pressing the Book management key while browsing the bookshelf.

Folder: \$VROtherBooks\Drama.

Rules: This is an optional folder to categorize the type of book.

Folder: \$VROtherBooks\Drama\BookA.

Rules: This is the folder containing the book files. The entire folder is defined as the book with one reading position and bookmark set. While browsing the bookshelf, you may press the Book management key to delete the book which means the entire book folder will be deleted.

Folder: \$VROtherBooks\Drama\BookB.

Rules: Same as Book A.

Folder: \$VROtherBooks\Mystery.

Rules: This is an optional folder for categorizing type of book.

Folder: \$VROtherBooks\Mystery\BookC.

Rules: Same as Book A.

### Other Books Characteristics

Bookshelf Browse: Navigate throughout the other books list with the Right and Left keys. Press the Book management key until hearing “Delete current book” to delete a book.

Reading: Playback loops between the files within a specific book. Navigate between files in a book using the Right and Left keys. File and Time Jump navigation levels are available.

Information: Reports information for the current file in the book.

## Music Bookshelf

The music bookshelf is in the reserved folder $VRMusic. It contains a folder structure of audio files and this entire structure is defined as the All Music book. Unless you create playlists, this will be the only Music book. If you add playlist (M3U) files to the root of $VRMusic, then each playlist will be an additional music book.

### Music Structure

You can navigate up to 8 levels of folders in music. Use the Up and Down keys to select the folder or file navigation level. Then, you can move backward or forward at the chosen level with the Right and Left keys. If you have more than 8 folder levels, those additional levels will be added to the level 8 list of folders.

If you select the ‘File’ navigation level, you will hear the name of the current file. If you select any folder Level navigation, you will hear the name of the current folder at that level. At any point in your structure, you can always select the deepest navigation level. If the current folder is at a higher level than your deepest level then the folder name is not announced. For example, let’s say you have a music structure that contains $VRMusic\Classic\Mozart and $VRMusic\Classic\Beethoven\Ninth_symphony. If you are currently located in Classic\Mozart while playback is inactive, changing the navigation from File to Level 1 will announce ‘Classic’. Changing from Level 1 to Level 2 will announce ‘Mozart’. Changing from Level 2 to Level 3 will announce nothing since there is no folder name for a level 3 folder in Classic\Mozart. If located in Classic\Beethoven while playback is inactive, changing from Level 2 to Level 3 will announce ‘Ninth_symphony’.

If you place any files in the root of $VRMusic, they will be listed only at the file navigation level.

### Music Characteristics

Bookshelf Browse: Use the Right and Left keys to move between each Music book. Note that only 1 music book (All Music) will be on the music bookshelf unless you create additional playlist books. You cannot use the Book management key to Delete a Book.

Reading: If loop is enabled, playback loops within the files of a music book. Playback will continue from the last file of one folder to the first file of the next folder. You can use the Up and Down keys to select folder or file navigation.

Information: Reports information for the current music book such as total folders and files and total time for the book.

Speed: Stratus 2 will reset the variable speed control to normal speed and return to the previous speed setting when you exit the Music Bookshelf. Note: It is only possible to listen to stereo music at normal speed. If you speed up or slow down the music, it will play in mono.

### Playlists

The Music Bookshelf may contain playlists. A playlist is a text file containing a list of supported audio files to play. It must have the "m3u” file extension. A playlist cannot link to other playlists.

### Folder and File Name Announcement

When browsing the Music bookshelf, you may or may not want to hear file and folder name announcements. The Stratus 2 will provide this information if the player is in Stop mode but will omit the announcements if the player is in Play mode. Thus, if you wish to use the Right or Left key, navigate back and forth between music files and do not want to hear the folder/file announcements, simply press the Play key first to put the player in play mode.

## Saved podcasts Bookshelf

The Saved podcasts bookshelf contains a structure of audio files where each audio file represents an episode of a podcast feed. Each file (episode) is defined as a separate podcast book.

### Saved Podcast Structure

Saved podcasts can be organized by folders and subfolders, but Stratus 2 will only consider the files in these folders/subfolders as those files would be on the root. Each file will be treated separately, without considering the folder or subfolder where it is located.

### Saved Podcasts Characteristics

Bookshelf Browse: Use the Right and Left keys to move between each Podcast book file (episode). Press the Book management key until hearing “Delete episode” to delete a Book file.

Reading: Playback stops at the end of each file. Time Jump navigation level is available on Up and Down keys. If chapters are present in a podcast file, it will be possible to navigate by chapters level.

Information: Reports information for the current Podcast book (file).

If you don't want to hear all the tags, press Play to interrupt and resume playback.

## Text Bookshelf

The Stratus 2 can also store text files grouped in the $VRText reserved folder. If there are folders or subfolders, Stratus 2 will read each text file as if they would be on the root of the $VRText folder. They are played with the Stratus 2 text-to-speech. Each text file within the $VRText folder is considered a book. Supported formats include: txt, html, xml, rtf, ses3 and DOCX. Note that when present, image description tags will be announced in html and xml documents.

### Text File Structure

The text files can be either at the root of $VRText or organized into subfolder categories, but in that case, Stratus 2 will treat them as if they were on the root.

### Text File Characteristics

Bookshelf Browse: Use the Right and Left keys to move between each text file. Press the Book management key until hearing “Delete file” to delete a text file.

Reading: Playback stops at the end of each file. The navigation levels on keys Up or Down include: page (if page markers are in the file), screen (if no page markers), paragraph, line, sentence, word, spell, and character.

Information: Reports information for the current text file.

### Navigating HTML/XML/DOCX Headings

For html/xml files that have heading tags (<H1> to <H6>), these will correspond to equivalent navigation levels 1 to 6 on the Up and Down menu. Be aware that some html/xml files do not use these heading tags consistently. For example, the file may have an H1 and H3 heading with no H2 heading. In this case, the Stratus 2 will present only level 1 and level 3 on the Up and Down menu. Also, an HTML/XML file could have a single H1 heading for a title with no other H1 tags in the file. In this case, Stratus 2 would jump to the end of the file if you pressed the Right key after choosing level 1 navigation.

# Configuration Menu

The Stratus 2 can be customized using the Configuration menu.

To open the Menu, press and hold the Information key. To move to the next or previous menu item and select it, press the Right or Left key. To move to the previous menu level, press the Up key. To select a value and return to your content, interact with the element, then close the menu.  To close the menu, press and hold Information key or, when at the first menu level, press the Up key.

## Menus and Menu Items List

The available menus, menu items and values are listed in their respective order below.

- Menu level 1: General settings.

  - Menus level 2, General settings: Language, System.

    - Menus level 3, language: Download language and voices, Select content voice (from the voices installed on the device, toggle).

      - Menu level 4, Download language and voices: change menu voice (current menu voice, can be changed from the available voices in the system language), change additional voice (current additional voice, can be changed from any voice in any available device’s language), swap voices (between the two voices installed on the device), change system language (from the languages available for the device)

    - Menu level 3, System: keyboard, date and time, sleep.

      - Menu level 4, keyboard: key beeps

        - Menu level 5, key beeps: ON, OFF, play only

      - Menu level 4, Date and time: change time (enter hour, enter minute, AM-PM or 24 hours), change date (year, month, day), advanced time settings

        - Menu level 5, advanced time settings: time format (12 hours, 24 hours, toggle), date format (year-month-day, day-month, day-month-year, month-day, month-day-year), daylight saving time (ON or OFF, toggle), time and date announcement (announce both time and date, announce time only, toggle)

      - Menu level 4, Sleep: sleep messages (ON or OFF).

- Menu level 1, Navigation and playback

  - Menu level 2, Navigation and playback: Time Jump, Save last used navigation level (ON or OFF, toggle), Bookmark alert (ON or OFF), Audio adjustment mode (Tone or Pitch, toggle), Loop (ON or OFF, toggle), Music, book time details.

    - Menu level 3, Time Jump: 30 seconds, 1 minute, 5 minutes, 10 minutes, 30 minutes (ON or OFF for each value)

    - Menu level 3, Music: Random: OFF, Random: ON, Loop: Folder, Loop: Single

    - Menu level 3, Book time details: time elapsed, time remaining, both.

- Menu level 1, Wireless

  - Menu level 2, Wireless: Airplane mode (ON or OFF, toggle), Wi-Fi, Bluetooth.

    - Menu level 3, Wi-Fi: Wi-Fi (ON or OFF, toggle), Status, New connection, Launch connection, Delete connection, Import Wi-Fi configuration, Validate connection.

    - Menu level 3, Bluetooth: Bluetooth (ON or OFF, toggle), Pair a new device, Pair keyboard, Delete paired device, Connect device, Disconnect device, Reconnect devices.

Note: when Airplane mode is turned on, you cannot access to all Wi-Fi and Bluetooth options. When Wi-Fi or Bluetooth are turned off, all related options are disabled.

- Menu level 1, about: version number, model (Stratus V2), serial number, copyright, licenses

- Menu level 1, Online settings

  - Menu level 2, Online settings: Software update, notification mode, book services.

    - Menu level 3, Software update: automatic check (ON or OFF, toggle), Check for update

    - Menu level 3, Notification mode: beep, message and title, no notification, beep only, beep and message

    - Menu level 3, Book services: Bookshare, DAISY Online, Eole

## General Settings

### Language

#### Download language and voices

##### Change menu voice

Use this item to change the voice that will read the menus. Use the Right and Left keys to cycle through the available variants of your system language. Press Play/Stop to accept the variant of your choice. Then, using the Right and Left keys, select your desired voice then press Play/Stop to select it. The download and installation of the selected voice will begin automatically and the new voice will apply upon restarting the device.

##### Change Additional Voice

Use this item to add a second text-to-speech voice (note that the additional voice reads content, not menus). Use the Right and Left keys to cycle through the available languages supported by the device and their variants if any, then press the Play/Stop key on the desired language/variant. Finally, using the Right and Left keys, cycle through the available voices, then press the Play/Stop key on the desired voice. The download and installation of the new voice will begin automatically and the new voice will apply upon restarting the device.

##### Swap voices

The Swap voices feature allows you to quickly swap the Content voice with the Menu voice. Use the Right and Left keys to select “Swap voices” then press Play/Stop to confirm voice swap (note that the unit will automatically shut down after confirmation). Please also note that this option only works if the two voices are in the same language and in your system language.

##### Change system language

The Change system language feature allows you to change the system language. Use the Right and Left keys to cycle through the available languages supported by the device. Once selected, this will trigger a software update. See section 6.6.1.3 "Download and install a software update" for details.

#### Select content voice

Use the Select content voice to select the voice that is used to read content. Press the Play/Stop key to toggle between the two voices. The change will take effect immediately.

### System

#### Keyboard

##### Key Beeps

You can turn on or off the beeps heard when pressing keys. Turning off this option will also suppress the second beep heard when starting playback. You can also set the “key beeps” option to “play only”. In this case, all the beeps are deactivated except the beep that is heard when resuming the playback after a pause in a book. The tone, volume and speed keys will always beep at their minimum, normal, and maximum positions regardless of this menu setting.

#### Date and Time

##### Change Time

Select this item to change the time on the device. You can set the hours, minutes, and switch from AM to PM (if in 12h hour format). To change the value entered, use the Up or Down keys, then press the Play/Stop key to confirm or the Book management key to cancel.

##### Change Date

Select this item to change the date on the device. You can set the year, month, and day. To change the value entered, use the Up or Down keys. Press the Play/Stop key to confirm or the Book management key to cancel.

##### Advanced Time Settings

Select this item to change the format of the time and date.

You can change the Time Format: (12h/24h); the Date Format (Month/Day/Year, Year/Month/Day, Day/Month/Year); the Daylight-Saving Time/Standard Time; the Time and date announcements (Announce Time Only, Announce Both Time and Date).

#### Sleep

##### Sleep Messages

The Sleep timer's 1 minute warning and Sleep off messages can be turned on or off.

## Navigation and Playback

### Time Jump

You can choose to turn on or off any or all of the following time intervals: 30 seconds, 1 minute, 5 minutes, 10 minutes, or 30 minutes. Only the time jump intervals you turn on will appear in the up and down (keys navigation menu when playing books).

### Save Last Used Navigation Level for Each Book

Select this item if you wish to save your navigation level for a book you are reading. With this feature, when reopening the book, the current navigation level remains the same as the one used before closing it.

### Bookmark alert

The “Bookmark alert” option is Off by default, so when you will reach a location in your book where a bookmark is saved, you will not be warned. Press the Play/Stop key to turn it on.

### Audio adjustment mode

When you listen to audio content except music, this option allows to toggle between Tone and Pitch. By default, the option “Tone” is selected. If you toggle the “Audio adjustment” to Pitch and then use the Tone key, you will be able to adjust the pitch instead of the tone.

### Loop

Playback will play repeatedly when Loop mode is selected. It will automatically restart again at the beginning when reaching the end of a book.

### Music

#### Random and Loop

Select this menu to configure your music settings on the device. You have the choice to play music randomly or not, to loop a folder, or to loop a single file.

### Book time details

You can configure the amount of information given when you want to be informed of the book details. You can be informed about only the time elapsed in your book, or only the time remaining or obtain both information as it was before.

## Wireless

Here are a few points to note before describing the wireless configuration menu:

- The Airplane Mode is used to turn off all wireless communication for situations where you are not allowed to use a wireless device. If the Airplane Mode is turned on, then the wireless communication is turned Off. You can also turn on the Airplane Mode in situations where you know you don’t need wireless communication to save battery power. In the menu, when turning on the Airplane mode, you will not have access to Wi-Fi and Bluetooth settings. Similarly, when turning OFF the Wi-Fi or Bluetooth, you will not have access to their related options.

- The SSID is the name of a network router. It is announced when you perform a Scan for Available connections. It is possible that a router has been configured to hide its SSID. In that case, it will not appear in the list of connections when performing a scan. To connect to such a network, you need to use the Create a New Connection option and manually enter the SSID.

- The Password is the authentication key used with your network router. It is case sensitive so when entered with the Stratus, you may toggle between lowercase, uppercase, and numeric only with the Bookmark key.

- The Nickname is a friendly name to be used by the Stratus to identify a network. This optional name will not be spelled as is the case with the SSID so you should enter an easily pronounceable name for the built-in text-to-speech. The maximum length of the nickname is 50 characters long.

### Airplane Mode

The first item of the Wireless menu is “Airplane mode”. Use this item to turn on or off the airplane mode. By default, the airplane mode is on. When the airplane mode is on, the wireless and Bluetooth features are turned off. When airplane mode is turned off, the wireless features are turned on automatically.

### Wi-Fi

#### Wi-Fi

The first item of the Wi-Fi submenu allows to toggle Wi-Fi on or off.

#### Status

Use this item to hear information on the network status. You can use the Right and Left keys to navigate between the various available network information like the SSID, the signal strength and more.

#### New Connection

Use this menu if you wish to establish a new Wi-Fi connection. You will be given the choice to scan for a connection, to establish a WPS connection, or to manually create a connection.

##### Scan for SSID

Use this item to scan for Wi-Fi routers within range of your Stratus. After a few seconds, a list of available routers should be displayed. Use the Right and Left keys to browse the available router SSIDs that were found. Press Play/Stop to confirm the router you wish to access. You will then be prompted to enter the password for the chosen router. Enter the password using the keyboard. If needed, perform a short press on the Power key to access the key describer mode. Use the Up and Down keys to navigate through characters. To go directly to a specific category, for example, symbols, use the Bookmark key. End your entry with the Play/Stop key. If successful, the router SSID and password will be added to your list of configured connections (routers) and the new connection will be launched as your active router. You will then be asked to enter an optional nickname for the new connection.

##### WPS Connection

Select this item to connect to a protected Wi-Fi network. If a WPS network is detected by the Stratus, press on the WPS button on your router to establish the connection.

##### Manually Connect

Use this menu item instead of scanning if you know the specific router SSID you wish to configure or if your router does not broadcast its SSID. You will be prompted to enter the SSID. Use the keyboard to enter the SSID and press Play/Stop. You will then be asked to enter the router password. Enter it and end with the Play/Stop key. Use the Bookmark key to toggle between uppercase, lowercase, and numeric only. If needed, perform a short press on the Power key to access to the key describer mode. If successful, the router SSID and password will be added to your list of configured connections (routers) and the new connection will be launched as your active router. You will then be asked to enter an optional nickname for the new connection. Please note that both SSID and password are case sensitive.

#### Launch a Connection

By default, the Stratus will automatically connect to the best available Wi-Fi router in range. Optionally, you may use the Right and Left keys to select from a list of Wi-Fi routers that you have previously configured. Press Play/Stop to confirm your selection. The selected router will then be used for subsequent wireless data transfer. You only need to use this item when you have configured multiple routers and want to launch an alternate router as your active connection for data transfer.

#### Delete a Connection

Use the Right and Left keys to select from a list of Wi-Fi routers that you have previously configured. Press Play/Stop to confirm your selection. The selected router will then be deleted from your list of configured Wi-Fi routers.

#### Import Wi-Fi Configuration

Use this item to process a network configuration file generated with the HumanWare Companion software. Press Play/Stop to process a network configuration from a file on a USB key. To generate a network configuration file, select the Wi-Fi Configuration item in the Tools menu of the HumanWare Companion. Enter your SSID and password. Optionally, you can enter a nickname for the network. The configuration file will be saved on the USB key that you will then insert in the Stratus.

#### Validate Connection

Use this item to validate your current connection to the Internet. The Stratus will connect to a webpage and if successful, the Stratus will prompt a short message that informs you of a successful connection. You can replay the message by pressing the Play key.

Note: If you require the MAC address of your Stratus in order to configure the MAC filtering of your router, you can find it in the Status submenu of the Wi-Fi submenu, under the Wireless menu.

### Bluetooth

Use this menu to configure your Bluetooth settings. You have the option to connect to, disconnect from or forget Bluetooth devices.

#### Bluetooth

Use this item to toggle Bluetooth on or off. Turn it off to preserve your battery if you don’t intend to use Bluetooth.

#### Pair a new device

First, make sure the Bluetooth device you want to pair with is turned on. Upon selecting this sub-menu, the Stratus will start scanning for available Bluetooth devices nearby. Scroll to the list of available devices using the Right and Left keys then press Play/Stop to pair with it. You will receive confirmation once you are connected.

Once you are paired, your Stratus will remember your Bluetooth device and pair with it whenever the device is activated and in close proximity.

#### Pair keyboard

You can pair an external Bluetooth keyboard to your Stratus. To do so, press the Play/stop key on this option. Stratus 2 will scan the available Bluetooth devices. Keyboards found will be displayed at first, then you can scroll through your complete list of Bluetooth devices available, using the Right and Left keys. If your desired external Bluetooth device is found, you can press the Play/stop key on it to connect to this device.

#### Delete paired Device

In this sub-menu, the Stratus will list the name of the devices your player has detected. Scroll to the name of the device you wish to forget using the Right and Left keys, then press Play/Stop. You will receive confirmation once the device is forgotten.

#### Connect device

Upon selecting this sub-menu, the Stratus will display a list of the Bluetooth devices you are paired with, if they are activated and located nearby. Scroll through your devices using the Right and Left keys then press Play/Stop to connect to it. You will receive confirmation once you are connected.

Once you connect to a device, Bluetooth will automatically connect to it each time you turn on the device.

#### Disconnect device

In this sub-menu, the Stratus will list the name of the devices you are currently connected to. Scroll to the name of the device you wish to disconnect from using the Right and Left keys then press Play/Stop to disconnect from it. You will receive confirmation once you are disconnected.

#### Reconnect devices

If some of your Bluetooth devices are not connecting correctly to the Stratus, you can use this option. Press the Play/stop key on this option and Bluetooth will be deactivated then reactivated to reconnect your Bluetooth devices correctly.

## About

In this submenu, you will find all the information related to your Stratus device.

- Version number;

- Model: Stratus V2;

- Serial number (of your device);

- Copyright;

- Licenses.

Use the Right and Left keys to navigate between the available information.

## Online settings

### Software Update

The Software Update menu contains two items: “Automatic check” and “Check for update”. By default, the Stratus will automatically check for any available update if connected to a Wi-Fi network. If you select the item “Check for update”, the Stratus will immediately check for any available updates. This item can be used multiple times even if the “Automatic check” item has already been used once. If the “Automatic check” is set to Off, it is necessary to use the “Check for update” item to check for updates as the Stratus will not do so automatically.

#### Automatic check

When opening the Software Update menu, this item will be the first. It’s a toggle, which is turned on by default, meaning that the device will check for updates automatically after you boot your device and connect or reconnect your device to a Wi-Fi network.

#### Check for update

When navigating in the Software Update menu with the Right key, the second item will be “Check for update”. Press the Play/Stop key to check if an update is available. Note that if the option “Automatic check” is put at off, it will be necessary to check for available updates using the “Check for update” function. You can perform a check for update at anytime when you are connected to a Wi-Fi network.

#### Download and install a software update

If an update is available, a prompt will be read indicating that an update is available, the build version and announce the new version that will be installed?] and will invite you to press the Play/Stop key to download the update, or any other key to download the update later. Please note that you can use the device while downloading the update.

When the download will be completed, a prompt will be read inviting you to press the Play/Stop key to install the update. Press the Play/Stop key and the device will restart automatically to install the update. Please note that if your device is on battery, it will shutdown and you will have to restart it manually to install the update. Percentages of progression will be announced during the installation process. When the installation will be completed, the device will restart if it is plugged, or will shutdown if not.

You can also download software updates from the HumanWare website and copy the file directly to the root of a USB key. After that, insert the USB key to the device and a prompt asking if you want to install the update will be read. Press the Play/Stop key to install the update or any other key to install the update later. Please note that after the installation process, the file will remain on your USB key until you reboot your device for the first time after the update process. If you want to update more than one device using this method, simply eject the USB key from the device after the shutdown of the device and before rebooting it.

NOTE: It is highly recommended to plug your device during a software update, to be sure to perform it properly. You can also perform a software update on battery, but battery must be at least 50% of charge when beginning the software update installation process. If the battery is too low to update, a message will be read indicating this and you will have to plug the device to perform the software update installation.

### Notification mode

In this submenu, you can select the notification mode used to transmit information about the online services. By default, the option “Beep, message and title” is selected, meaning that if a notification about an online service appears, for example the completion of a book downloading from an online service, a beep will be heard, a message and the book title will be said. You can change this setting to “Beep and message”, “Beep only” or “No notifications”.

### Book Services

Use this menu to access all of the book services available on the device.

#### Bookshare (applicable to some countries)

Use the Bookshare menu to manage the online service. The service can be turned on by going to the Online services menu, Book services, then selecting the Bookshare submenu and selecting the option “Add account”. Press the Play/Stop key to validate this option. You will be prompted to enter your registered email address and password using the Stratus keyboard. Passwords are usually case sensitive. End your entry with the Play/Stop key. It is also possible to use the “Import configuration” item to import your Bookshare account information with a file created using the HumanWare Companion software. Once you have successfully entered your account information, a Bookshare bookshelf will be added to the online bookcase. To remove the Bookshare online service, select the option “Remove account”. Use the item “Reauthorize” to modify your account information and reauthorize the account. Finally, it is possible to select the book format by toggling between DAISY and BRF.

#### Daisy Online (applicable to some countries)

Use this menu to manage the Daisy Online service. In the Online services menu, Book services and the DAISY online service submenu, select the item “Add account”, then choose a service to add. You will be prompted to enter your username and password. You can also use the item “Import configuration” to import a custom Daisy Online configuration to your Stratus. This custom configuration can be created with the HumanWare Companion software (see section 1.9 about the HumanWare Companion to know more). The software will generate a file that you will be able to import to your Stratus using the “Import configuration” option.

For each DAISY online service configured, you can use the item “Modify login information” to modify your account information. You can also select the item “Download Method” to choose the way in which the Stratus will download books from this Daisy Online library. You can choose between Automatic download (by default) and Manual download. With the Automatic download method, your chosen books or those suggested by the library will automatically be downloaded on your Stratus. With the Manual download method, you will be able to use the option “Download more books” in the appropriate Daisy Online bookshelf and manually select which books you want to download and return other books directly without downloading them. In each DAISY online library added, you can use the item “Remove account” to remove this Daisy Online account and delete all associated books from the Stratus.

#### Eole

Eole is a French library that contains accessible books for disabled people. You can access that resource by using the following URL: https://eole.avh.asso.fr/

More information about EOLE library can be found in the French user guide of this device. Alternatively, this user guide can be downloaded from our website.

# Daisy Online

## Daisy Online configuration menu

To manage the Daisy Online service, go to the menu and select the option Online services, then the option Book services and the item DAISY online. Select the item “Add account” to choose a service to add. You will be prompted to a list of libraries that you can add, select one of them. You will have to enter your username and password. You can also use the item “Import configuration” to import a custom Daisy Online configuration to your Stratus. By navigating with the Right and Left keys, you will find a list of libraries that are already added on your device. If you select one of these libraries, you will have the opportunity to change the download method, to modify login information and to remove account. The item “Download method” offers two ways to download your books from this library: manual download and automatic download (by default). With the Automatic download method, your chosen books or those suggested by the library will automatically be downloaded on your Stratus. With the Manual download method, you will be able to use the option “Download more books” in the Daisy Online bookshelf and manually select which books you want to download and return other books directly without downloading them. The item “Modify login information” allows to change the username and password used to connect to this library. Finally, use the item “Remove account” to remove this specific library and delete all associated books from the Stratus.

## Using Daisy Online Service

When the Stratus is connected to a wireless network, you will be able to access Daisy Online, an online service used to access and download books from compatible Daisy Online libraries.

To activate the Daisy Online service:

- Press and hold the Info key to access the Configuration menu.

- Use the Right and Left keys to access the item Online Services and press Play/Stop.

- Use the Right and Left keys to access the item Book Services and press Play/Stop.

- Use the Right and Left keys to access the Daisy Online menu and press Play/Stop.

- Select the item “Add account” using the Right and Left keys followed by Play/Stop.

- Using the Right and Left keys, select a service to add followed by Play/Stop.

- Enter your Daisy Online username and password.

- Alternatively, the HumanWare Companion software can be used to create a file containing your Daisy Online account details, which can be imported from the “Import configuration” item on the DAISY Online service menu on the Stratus.

- Once you have configured a Daisy Online account and added it from the online configuration menu, a Daisy Online bookshelf with the DAISY Online service name will be added to your online bookcase. Subsequently and for each configured account, a new Daisy Online bookshelf will be added to your online bookcase. If there is more than one configured account for the same DAISY Online server, the account username will be appended to the service name when announcing the bookshelf to help you differentiate your accounts.

- In your Daisy Online bookshelves, you will be able to find the books downloaded from your account. When a book’s expiration date is known, it will be announced after the book title. When a book is expired and needs to be returned, it will be announced before its title. Pressing the Info key will give you a book description when available, whether a book is downloaded or not. From a book, you can also press the Info key to know its expiration date.

## Download Methods

If you selected Manual download method in the Daisy Online menu, you will have an option on the bookshelf named “Download more books”. Use this option to browse all the books issued to your account that are available for download. You can choose to download these books with the Play/Stop key, or return them directly without downloading them using the “return” option, on the Book management key. You can also return a book while offline; the return will be completed once you are connected to a wireless network.

If you selected Automatic download method in the Daisy Online menu, the Stratus will periodically download new books issued to your account on your Daisy Online bookshelf. However, if you want the Stratus to immediately download any available books, you can use the option on the bookshelf named “Check for New Content”. Once the player’s memory is full, new ones will be downloaded as books are returned.

## Navigating in your DAISY online bookshelves

- Each Daisy Online service offers a menu that may differ from one service to another. For example, some services allow the ability to copy books using the Book management key, while others do not. Different download methods can be set as default, while different return policies may apply for expired books.

- Two search types are possible: Single input and Multiple inputs. Generally, DAISY Online Service supports either one of these search types. Most services use the Single input search type, while only a few use the Multiple inputs search type.

- With the Single input search type, you will be prompted to enter your text using the keyboard. Use the Bookmark key to toggle between Text and Numeric input types while entering your text to search. You can edit a previous search by entering new search terms which will be added after your previous search criteria. Press Play/Stop to start the search and display search results list, or press the Book management key to cancel the search. You will return to your DAISY Online bookshelf.

- With the Multiple inputs search type, you can enter multiple criteria to search, such as the title, author, etc. Each available criteria will have a distinct input field, and you can enter multiple criteria to search for at the same time. Press the Right and Left keys to navigate between criteria, then Play/Stop to edit one of them. Use the Bookmark key to toggle between Text and Numeric input types while entering your text to search. Press Play/Stop on the confirm prompt after the last criteria to start the search and display search results list, or press the Book management key to cancel the search. You will return to your DAISY Online bookshelf.

- In addition, depending on the Daisy Online service you are using, various functions will be available to you from the Book management key, notably the options “Return book”, “Copy current book to USB” and “Redownload” but it depends on what the Daisy Online service offers.

- The Info key will announce the book description when available.

- While downloading a Daisy Online book, an “Added to download queue” message will be heard. Press and hold the Book management key to hear a download status report.

# Updating Stratus 2 Software

HumanWare may from time to time offer new versions of the Stratus 2 software. There are two ways to update your Stratus 2 software; wirelessly, and by downloading an update file on a USB key.

To update your Stratus 2 wirelessly, you must first have an active connection to the Internet using the built-in Wi-Fi (see section 6.4 for more information on the wireless configuration menu). Connect the Stratus 2 to a power outlet with the AC power adapter. If the Stratus 2 announces that it is in airplane mode, activate the Wi-Fi by turning off the airplane mode (first item in the Wireless menu). The Stratus 2 will automatically check for any available update. If a newer version of the software is available, Stratus 2 will inform you that there is an update available. Press on the Play/Stop key to start the update process or any other key to cancel. The download will be done seamlessly and you can use your Stratus 2 during this time. To hear the download progression, go to the Software update submenu that you will find in the Online services menu. The download time may vary depending on the speed of your Internet connection. Once the download is completed, the Stratus 2 will ask your confirmation to proceed with the update. Press Play to start the update or cancel with any other key. Please note that if your device is on battery, it will shutdown and you will have to restart it manually to install the update. Also, battery must be at least 50% to proceed. If plugged, the device will restart automatically. During the installation, Stratus 2 will periodically announce the update status in percentage. Once the update is completed, the Stratus will restart if plugged, or power off if on battery.

You can also update your Stratus 2 by downloading a SWU software update file from the HumanWare website. Copy the SWU file to a USB key. Connect the Stratus 2 to a power outlet with the AC power adapter. Power on the player and insert the USB key containing the update file. Stratus 2 will inform you that there is an update available, with the version number. Press on the Play/Stop key to start the update process or any other key to cancel. If the device is on battery, it will shutdown and you will have to restart it manually to install the update. If plugged, the Stratus 2 will restart then will install the update. It will periodically announce the update status in percentage. Once the update is completed, the player will restart if plugged, or power off if on battery. Please note that if the device is on battery, it is possible to perform the update if the battery is charged at 50% or more.

The SWU file will be automatically deleted from the USB key when you power on the Stratus 2 after performing an upgrade. To upgrade multiple players using the same USB key, be sure to remove the key from the upgraded Stratus 2 before turning the Stratus 2 back on.

# Troubleshooting

## Book Navigation

Q: Why does Victor Reader Stratus 2 not always announce the same DAISY levels when I press the Up and Down keys?

A: The Up and Down keys cycle through the DAISY navigation levels provided by the producer of the book. Not all DAISY books have the same navigation levels. For example, your book’s producer may have only inserted level 1 (often used for chapters) marks into the book. In this case, the Up and Down keys will announce only "level 1" and "phrase". A phrase level is always included. Victor Reader Stratus 2 can only navigate at the levels provided by the book producer. Also, the phrase jump is not the same for each book. Some producers may include a phrase mark for a single sentence while others may markup phrase as an arbitrary time jump. Still others have converted older cassette books and inserted a phrase as an entire side of one cassette. For books with little or no DAISY navigation marks, you will need to navigate with the Fast forward and Rewind keys. The Rewind key is to the left of the Play/Stop key and the Fast Forward key is to the right of the Play/Stop Key.

Q: How do I know what DAISY navigation levels are available in my book?

A: The Victor Reader Stratus 2 Up and Down keys will cycle through the levels of navigation available in the book. Many book producers include a section at the beginning of the book that describes what the DAISY levels correspond to in that book. Check with your book producer if you are unsure of the book’s navigation features.

Q: How do I quickly jump to the start or end of a book?

A: Victor Reader Stratus 2 has automatic bookmarks for the beginning or end of the book. Press the Bookmark key until you hear Stratus 2 announce, "Bookmark List". Then press the left key until you hear, "beginning of book". The beginning of book is always to the left of the first user bookmark. Similarly, to reach the end of the book, press the right key in the bookmark list until you hear "end of book". The end of book is always to the right of the last user bookmark. If you have not set any bookmarks, then you need only press the left key once in the bookmark list to reach the beginning of the book or press the right key once to reach the end.

## Battery and Charging

Q: Can I damage my battery if I leave the power cord connected for a long time?

A: No. However, if you are not going to use your Victor Reader Stratus 2 for a long time such as being away on vacation, it is best to disconnect it from the power mains to avoid potential damage from power surges.

Q: Can I replace my battery?

A: No. Please contact HumanWare or authorized dealers for battery replacement.

Q: What precautions should be taken if I am not planning to use the player for an extensive period of time?

A: The player and batteries should be kept within a temperature of -20 to 35 Celsius and humidity levels between 5 and 90% without condensation.

## General

Q: What should I do if the On/Off key is not working?

A: If no beep is heard when you press the On/Off key, first ensure the player is plugged into the AC power source. If the On/Off key is still not working, press and hold it down for 10 seconds. This will reset the player. Then, press it again and Victor Reader Stratus 2 should beep and start normally.

Q: Can the internal Victor Reader Stratus 2 software be updated?

A: Yes. If you are connected to a wireless network and the Automatic check for update option is turned on, you will be prompted to update your device to the latest version whenever HumanWare releases a new version of the Victor Reader Stratus 2 software. Alternatively, you can visit the HumanWare website on a computer and copy the latest software update onto a USB key, and then insert it into your Stratus 2. Refer to Chapter 8 for more details.

Q: Should I delete bookmarks to save memory space?

A: No. Victor Reader Stratus 2 can store over 1500 bookmarks spread over many books. Even if you exceed this large number, Stratus 2 will replace the bookmarks that were used the least often. So, you only need to delete bookmarks if you do not want them.

# Customer support

For customer support, please contact the HumanWare office nearest you or view our web site at: www.humanware.com

North America: 1(800) 722-3393 or send an email to us.support@humanware.com

Europe: (0044) 1933 415800 or send an email to eu.support@humanware.com

Australia / Asia: (02) 9686 2600 or send an email to au.sales@humanware.com

# Technical specifications

- Dimensions:	21.15. X 20.02 X 4.06. cm (8.33 X 7.88 X 1.6 in.)

- Weight:	0.89 kg (1.95lbs) with battery

- DAISY Formats Supported:	DAISY 2.0 and 2.0.2

DAISY NISO Z39.86 2002 and Z39.86 2005

- Battery type:	Rechargeable Li-ion battery. 6600 mAh

- Charging time:	2.5 hours

- Operating time:	20 hours for continuous play of DAISY MP3 book in airplane mode

- Power Supply:	The supplied power adapter meets power requirements only for

the country in which it was sold. Check with your dealer for

alternate power adapters used for other countries.

- DRM: 2002 PDTB1 (so-called I.P.P.) and 2006 PDTB2

- Audio formats supported: AAC (.mp4, .m4a, .m4v), AMR-WB+ (.3gp), Flac, MPEG2, MP3, OGG Vorbis (.ogg), Opus , Speex (.spx), Wav P.C.M.

- Text files supported:            docx, fb2, html, lkf, pdf, rtf, txt, xml.

- Text-To-Speech: Acapela for Linux Embedded

- Power Supply: Switching type AC/DC. Input 100V – 240V, 50Hz – 60Hz. Output: Type A USB socket, 5VDC, 1.5A

- earphone jacks	3.5 mm

Environmental

- Operating Temperature 0 to 45º C

- Transport & Storage Temperature -20 to 60º C

- Operating Humidity 5% to 95% (RH)

- Transport & Storage Humidity 5% to 95% (RH)

Wireless

- Wi-Fi : IEEE 802.11 a/b/g/n/ac operating in the 2.4GHz and 5GHz bands

# Safety precautions

## Contraindications

There is no special contraindication for the use of the device.

## Warnings

- Never use household cleaners to clean the Victor Reader Stream.

- Never use cleaning products that contain Ethyl Alcohol, Ethyl Acid, Ammonia, Acetone, or Methyl Chloride.

- Any serious incident that has occurred in relation to the device should be reported to the manufacturer and the competent authority of the Member State in which the user and/or patient is established.

- Do not use headphones while walking on the street.

## Care and maintenance

To keep your Stratus 2 clean, we recommend that you wipe the surface of the unit periodically using a soft damp cloth. Squeeze cloth to remove excess moisture. Only use warm water. Do not use any cleaning agents. The product is not intended to be disinfected.

## Storage and transportation

The product should not be folded or dismantled in storage or transportation.

The product can be carried as any electronic device in a car or plane. It does not have any specific restrictions.

## Additional information

After long exposition to the sun, the surface temperature might increase.

The product has performed immersion and electromagnetic immunity tests and should not cause any interference, or be influenced by any other product.

The product material has a V-0 level rating to ignition.

The product is designed to have a lifetime exceeding 5 years.  The Battery is designed to have a lifetime of 3 years.

## Service information

No maintenance, calibration or preventive inspection is needed on the player.

If the user finds the battery autonomy has decreased, a replacement request may be made to HumanWare (for a fee).

Contact HumanWare or your distributor for any repair or malfunction questions.

Service shall only be done by HumanWare or its authorized distributor.

## Susceptibility to Interference

There may be temporary degradation of the sound when the Stratus is subjected to a strong radio-frequency field, electro-static discharge or transient electrical noise.

FCC Warning

This device complies with part 15 of the FCC Rules. Operation is subject to the following two conditions: (1) This device may not cause harmful interference, and (2) this device must accept any interference received, including interference that may cause undesired operation.

Changes or modifications not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

Note: This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

—Reorient or relocate the receiving antenna.

—Increase the separation between the equipment and receiver.

—Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

—Consult the dealer or an experienced radio/TV technician for help.

Industry Canada statements

This device complies with Industry Canada licence-exempt RSS standard(s). Operation is subject to the following two conditions: (1) this device may not cause interference, and (2) this device must accept any interference, including interference that may cause undesired operation of the device.

Le présent appareil est conforme aux CNR d'Industrie Canada applicables aux appareils radio exempts de licence. L'exploitation est autorisée aux deux conditions suivantes: (1) l'appareil ne doit pas produire de brouillage, et (2) l'utilisateur de l'appareil doit accepter tout brouillage radioélectrique subi, même si le brouillage est susceptible d'en compromettre le fonctionnement.

CAN ICES-3 (B)/NMB-3(B)

## Audio Warning

To prevent possible hearing damage, do not listen at high volume levels for long periods. Exercise caution when holding your device near your ear while the loudspeaker is in use.

## Battery Safety Precautions

Battery becomes warm during recharging. Do not recharge battery near a heat source. Battery will not recharge if internal temperature is too high. Only use supplied adapter and cable to charge the Victor Reader Stream. If the battery is discharged for a long period of time, there will be a delay of up to a few minutes before the device shows any activity, including charging. Do not submerge the product – there is a risk of overheating, fire or even explosion if the battery is put in a fire, heated, subjected to impact, fully submerge for a long period of time or if the battery terminals are shorted. Do not disassemble or modify the unit. Do not strike, throw or voluntary hit the device. Do not strike the unit with sharp objects or blunt object.

In the event that the battery leaks from the device and gets on your skin, wash with fresh clean and seek medical attention to prevent injuries. If battery leaks and liquids gets in contact with eyes, DO NOT RUB, rinse with clean running water and seek immediate medical attention.

## Disposal Instructions

At the end of this device operational lifetime, its internal components must be disposed in compliance with local authorities.

This device contains no hazardous materials. For disposal, return to HumanWare or follow local governing ordinances or hospital procedure.

# Manufacturer Warranty

This device is a high-quality product, built and packaged with care. All units and components are guaranteed against any operational defects as follows:

US and Canada: One (1) year

Continental Europe and UK: Two (2) years

Australia and New Zealand: One (1) year

Other countries: One (1) year

The warranty covers all parts (except battery) and labor. If any defect should occur, please contact your local distributor or the manufacturer technical assistance line.

Note: Warranty terms may periodically change, please consult our website for the latest information.

Conditions and Limitations:

No replacement or repair covered by the warranty will be carried out unless the unit is accompanied by a copy of the original bill of purchase. Please retain your original. If the unit has to be returned, please use the original packaging. This warranty applies to all cases where the damage is not a result of improper use, mistreatment, negligence or acts of God.

# End User License Agreement

By using this product (Victor Reader Stratus 2) you agree to the following minimum terms.

      - License Grant. HumanWare grants to End User a non-exclusive, non-transferable right and license to use the Software on this product.

      - Ownership of Software. End User acknowledges that HumanWare retains all rights, title and interest in and to the original, and any copies, of software which is incorporated into this product. End User agrees not to: modify, port, translate, decompile, disassemble, reverse engineer, or make public in any way the software of this Product.

This product includes software developed by the OpenSSL Project for use in the OpenSSL Toolkit (http://www.openssl.org/)