BRAILLENOTE EVOLVE

# KeySoft Update 1

Release notes

This update introduces the Braille Terminal app, adds search and replace tools for BRF content, updates browse-mode shortcuts, and delivers Outlook and general reliability improvements.

# Release overview

## Highlights

- Use BrailleNote evolve as a Braille display with VoiceOver, MAC, Apple TV, NVDA or JAWS through the new Braille Terminal app.

- Find text in BRF files opened in KeyBRF or Victor Reader, and use new replace commands in KeyBRF.

- Use revised browse-mode shortcuts with fewer command conflicts.

- Benefit from Outlook, Braille IME, NVDA, external-display and localisation fixes.

# Install the update

The update is delivered through KeySoft Control Center. Connect BrailleNote evolve to the internet before starting.

- From the KeySoft Main Menu, open All apps.

- Open KeySoft Control Center. The Updates tab checks automatically for available updates.

- Press TAB until you reach Update all, then press ENTER.

- If prompted, confirm that running applications may be closed so the update can complete.

- After installation, open All apps to find Braille Terminal. To pin it to the Main Menu, select it and press ENTER + M.

# New: Braille Terminal

Braille Terminal lets BrailleNote evolve work as a Braille display for a supported iPhone or computer. Once connected, use the standard Braille commands provided by the host screen reader.

## Supported screen readers

Braille Terminal supports VoiceOver on iPhone and Mac, Apple TV with TVOS, and NVDA or JAWS on a Windows computer. Android OS is not supported in this update. Please note that USB connections is not possible with the Braille terminal.

## Pair and connect

- Open Braille Terminal from the Main Menu or the All apps folder.

- Choose Add a device and pair your iPhone, Mac, Apple TV or Windows computer in the usual way.

- Select the paired device and press ENTER. Alternatively, open the Context Menu and choose Connect and activate. To reconnect a previously paired nearby device, choose Reconnect devices.

## While connected

- Press the Home button once to return to the Braille Terminal menu.

- Press and hold the Home button to switch between the most recently connected device and the current BrailleNote evolve application.

- Use the Context Menu to disconnect from or delete a paired device.

# BRF and browse-mode updates

## Find in BRF files

Find, Find next and Find previous are now available in KeyBRF and when a BRF book is open in Victor Reader. The commands are also available from the Context Menu.

## Find and replace in KeyBRF

KeyBRF now includes Replace, Replace and find next, and Replace and find previous. The Context Menu has been reorganised to include the new actions.

## Browse mode

To reduce conflicts with browse-mode navigation, most non-browse commands no longer require SPACE. Continue to include SPACE for the web-browser and other commands shown in the command reference below.

Use SPACE + X to toggle browse mode in Microsoft applications and web browsers.

## Command reference

Shortcut notation: A plus sign means that the named keys are pressed together. For example, SPACE + F means press SPACE with the Braille letter F.

| Area | Action | Braille shortcut |
| --- | --- | --- |
| BRF search | Find | SPACE + F |
| BRF search | Find next | SPACE + N |
| BRF search | Find previous | SPACE + P |
| KeyBRF | Replace | BACKSPACE + F |
| KeyBRF | Replace and find next | BACKSPACE + N |
| KeyBRF | Replace and find previous | BACKSPACE + P |
| Web browser | Back | SPACE + BACKSPACE + B |
| Web browser | Forward | SPACE + BACKSPACE + F |
| Web browser | New window | SPACE + BACKSPACE + N |
| Other | Toggle input help | SPACE + BACKSPACE + K |
| Other | Toggle Braille grade | SPACE + BACKSPACE + G |
| Other | Quick settings | SPACE + BACKSPACE + Q |
| Microsoft apps and web browsers | Toggle browse mode | SPACE + X |

# Improvements and fixes

## Outlook

- Attachments are now announced and included in HumanWare view.

- Resolved a Braille conversion issue when using the Braille IME keyboard layout.

## General

- Removed the landscape and portrait messages that appeared when connecting or disconnecting an external display.

- Resolved an issue that could display a blank window when launching Word or NVDA.

- When modifier keys are disabled, the setting now remains disabled after restarting the device.

- Resolved an issue affecting Method 2 and Method 3 Braille tables.

- When using Braille IME, NVDA now reads typed words followed by punctuation.

- Resolved localisation issues.